MultiBatchDS

Professional Batch Distillation System

User Manual

multiple configurations

multiple modes of operation

multiple fractions

multiple levels of simulation models

Batch Process Research Corporation

8001 Lingay Drive

Allison Park, PA  15101

USA

bprc @ bprc.com

1-800-684-BPRC

1-412-635-0521 Fax

Software developed by Dr. U. Diwekar,

author of the book

Batch Distillation: Simulation, Optimal Design and Control

Taylor & Francis Inc.  1995

BPRC Inc.

8001 Lingay Drive

Allison Park, PA  15101  USA

bprc @ bprc.com

1-800-684-BPRC

1-412-635-0521

Documentation written by Mike B. Berkenpas, Urmila Diwekar, and John J. Fry

Contents

Introduction	1

What you can do with MultiBatchDS ?	1

Technical Highlights	2

Conceptual Design	2

Column optimization and optimal control	2

Numerical Methods	3

Modeling	3

Visual Highlights	3

What you get with MultiBatchDS	4

Hardware/Software Requirements	4

MultiBatchDS Add-ons	5

MultiBatchDS/DataLink	5

MultiBatchDS/Probabilistic	5

MultiBatchDS/PackedCol	5

MultiBatchDS/RT	5

Getting Started	7

The Main Window	7

File Menu	7

New BDT File	8

Open BDT File	8

Save BDT File	8

Save BDT File As	9

New Text File	9

Open Text File	9

Close Text File	9

Save Text File	9

Save Text File As	9

Print Preview	10

Print	10

Print Setup	10

Exit	10

Edit Menu	10

Undo	10

Cut	11

Copy	11

Paste	11

Clear All	11

Delete	11

Search Menu	11

Find	12

Replace	12

Next	12

Parameters Menu	12

Policy Options…	12

Thermodynamic Options…	12

Task Specification…	12

Units…	13

Stochastic Analysis	13

Calculate Menu	13

Run Calculation	13

Show Errors	14

Show Results	14

Plot Menu	14

View Simulation Plot	14

CDF Plot	15

Copy Plot to Clipboard	15

Print Plot	15

Window Menu	15

Cascade	15

Tile	15

Arrange Icons	15

Close All	15

Help Menu	16

Contents	16

Tutorial	16

Using Help	16

About	16

Graphical Input	17

Overview	17

Policy Options	17

Thermodynamic Options and Data Bank	19

Units	23

Stochastic Analysis	23

Task Specifications	23

Design	24

Simulation	25

Optimization	25

Thermo-Estimation	25

Model Options	26

Short Cut Model	26

Semi-Rigorous Model	26

Reduced Order Model	26

Rigorous Model	26

Column Specifications	26

Design Related	27

Operation Related	28

Calculation Related	32

Optimization Related	32

Estimation Related	35

MultiBatchDS Tutorial	37

The Tutorials	37

Example 1	37

Solution	38

Part a)  Step by Step Procedure	38

Part b)	46

Part c)	46

Part d)	46

Example 2	46

Solution	47

Part a) Step by Step Procedure	47

Part b)	49

Operation Involving Side Stream	49

Step by Step Procedure	50

I. Definition of Side Products	50

II. Definition of Side Feeds	50

Stripping Operation	51

Step by Step Procedure	51

Multifraction Operation	51

Step by Step Procedure	51

Units Conversions	53

Step by Step Procedure	53

Heat Balance Calculation	54

I.  Heat Balance Specification	55

II.  Numerical Results in Energy Balance Calculations	57

III.  Graphical Results in Energy Balance Calculations	57

MultiBatchDS Sample Problems	59

Example Files	59

References	63

References	63

Appendix A	65

Component list from the CRANIUM databank	65

Appendix B	77

Keyword Input	77

Description of Keywords:	78

Policy Commands	78

Thermodynamic Option Commands	79

Task Commands	80

Column Specification Commands	81

Index	85

# Introduction

## What you can do with MultiBatchDS ?

The recent increase in the production of high value-added, low-volume specialty chemicals and biochemicals has generated a renewed interest in batch processing technologies.  Batch distillation is an important unit operation in the batch processing industry and is widely used.  The flexibility of batch distillation, combined with the inherent unsteady nature of the process, poses challenging design and operation problems.

| {bmc configure.wmf} | The transient nature of batch distillation allows for configuring the column in a number of different ways, some of which are shown in Figure 1 (Diwekar, 1995). Click here to view the figure. |
| --- | --- |

Figure 1: Different column configuration including emerging designs

The column in Figure 1a is a normal batch distillation column with a reboiler at the bottom and a condenser at the top, which essentially performs the rectifying operation.

One column can be used to separate several products using the multi-fraction operation of batch distillation presented in Figure 1b.

In the multi-fraction operation several cuts or fractions are made in a single column. Some cuts can be desired and others can be intermediate products. These intermediate fractions can be recycled. Figure 1c shows this periodic operation in which each charge consists of fresh feedstock mixed with recycled off-specification material from the previous charge.

Figure 1d represents the stripping column for separating light ends, where the storage vessel is kept at the top of the column in which the original mixture is fed.

In 1994 Devidyan et al. presented a batch distillation column that has both stripping and rectifying sections embedded in it (Figure 1e).  Although this column has not been investigated completely, preliminary analysis has demonstrated that it provides added flexibility for batch distillation operation.

These are a few examples of the different kinds of batch distillation columns combined with different modes of operation, the number of possible column configurations tends to be vary high.   MultiBatchDS is a unique professional batch distillation systems which offers multiple levels of models for multiple column configurations, multiple operating modes, multiple fractions, and multiple products (Diwekar and Madhavan, 1991a; Diwekar 1996).

Complete 32-bit integrated visual environment on your PC or workstation running under Windows 95 or NT for simulation, optimal design, and control of batch distillation systems. You have the power of on-line monitoring and control of program execution. Change configuration, operating mode, or any parameter and see MultiBatchDS recalculate the output—It’s that easy.

MultiBatchDS will increase your productivity from day one.

## Technical Highlights

### Conceptual Design

- Simple-yet-effective models for preliminary design, synthesis, optimization, and rapid analysis of column behavior.

- Design feasibility analysis.

### Column optimization and optimal control

- Solution of maximum distillate, minimum time, or maximum profit problems.

### Numerical Methods

- Robust algorithms for non-stiff and stiff systems.

- Model switching for highly stiff systems

- Collocation models for partial differential equation systems including packed columns

- Rigorous error handling.

- 32-bit implementation of the complete calculation engine and user interface for better performance and large scale problems.

### Modeling

- Capabilities for analyzing and optimizing complex and unconventional column configurations, including emerging designs such as columns with a middle vessel and stripper.

- Rigorous models for detailed column design based on solution of fundamental equations governing transient heat and mass transfer column operation.

- Simulation of startup conditions.

- Dynamic simulation models with varying degrees of complexity and rigor.

- Various modes of operation like constant reflux, constant reboil ratio, variable reflux, variable reboil ratio, optimal reflux, and optimal reboil ratio. Possibility of analyzing side feeds and side products.

- Databank and thermodynamic models for predicting properties and vapor-liquid equilibria for ideal and  and nonideal mixtures.

### Visual Highlights

- A user-friendly graphical input interface based on Windows 95 and Windows NT. Also supports keyword input structure where commands can be written in simple English.

- Flexible and easy-to-use menus and sub-menus.

- Powerful, interactive plotting with multiple options for abscissa and ordinate for each fraction.

- Graphical as well as textual output results.

- Extensive error checking and reporting.

- On-line help screens.

- On-line monitoring and control of the program execution.

| {bmc screen.bmp} | Click here to view a sample MultiBatchDS screen. |
| --- | --- |

## What you get with MultiBatchDS

- Distribution disks, files and directory locations

- Sample problems and results

- Terms and conventions

- Using online help

- Software registration and technical support

- Installation

- System requirements

- Installation options

- Figure Install Screen

- Databank  installation, databanks supported

- Hardware security key

- Quick Tour

- Starting MultiBatchDS

## Hardware/Software Requirements

- 100 MHz 486 Processor or Pentium

- 16 Megabytes main memory

- 10 Megabytes hard disk space

- Windows 95 / Windows NT (Windows 3.11 can be made available on request)

## MultiBatchDS Add-ons

### MultiBatchDS/DataLink

MultiBatchDS can be easily linked to commercially available or inhouse databanks and thermodynamic models of your choice.

### MultiBatchDS/Probabilistic

A fully integrated add-on for uncertainty analysis based on probabilistic techniques and Monte Carlo simulations. Contains a library of distribution functions and various sampling techniques including a recently developed efficient sampling technique which can be 100 times faster than the Monte Carlo simulations.

### MultiBatchDS/PackedCol

The collocation-based reduced-order model can be easily adapted for packed columns or for non-equilibrium distillation columns with this add-on. Mass transfer relations for specific packings can be readily input using this add-on for packed columns simulation.

### MultiBatchDS/RT

A customized data-acquisition and control add-on that provides a real-time interface to inhouse control systems. You can modify thermodynamic parameters using the simulation and plant data with built-in optimization-based parameter estimation techniques.

# Getting Started

## The Main Window

| {bmc main.bmp} | Figure 2 shows the main window. Click here to view the Main Window. The rest of this section describes the menus shown in this window. |
| --- | --- |

Figure 2: The Main Window

## File Menu

Allows the user to select and create data files for execution. There are two kinds of input files.  Input via the graphical interface is stored in files with the extension .BDT.  Keyword input files have the extension .TXT.  Graphical input is described in "Graphical Input" on page 17.  Keyword input language is presented in "Keyword Input" on page 73.  The user can provide inputs in either manner. The graphical interface creates a keyword input file internally.  The user can edit this file, provide a new .TXT file, or let the default file be used for calculations. Note that if you modify the keyword file the changes will not be reflected in the graphical input file.  Figure 3 displays the File menu.

Figure 3: The File Menu

| {bmc file.bmp} | Click here to view the File menu. |
| --- | --- |

The File menu provides commands for creating new files, opening existing files, saving files, printing files, and exiting the application.

### New BDT File

Create a new, untitled graphical MultiBatchDS input file.

This command resets the current data set to a new empty data set.

### Open BDT File

Open an existing MultiBatchDS graphical input file.

This command allows you to open a BDT file that was previously saved to disk.

### Save BDT File

Save the current MultiBatchDS graphical input file,  if its contents have changed.

This command saves the current set of parameters to disk. If the document is unnamed, the Save File As dialog box is displayed so you can name the file, and choose where it is to be saved.

### Save BDT File As

Save the current MultiBatchDS   BDT file under a new name.

This command allows you to save the current set of parameters under a new name, or in a new location on disk.  The Save File As dialog box is displayed.  Enter the new file name, including the drive and directory.  All windows containing this file are updated with the new name.  If you choose an existing file name, you are asked if you want to overwrite the existing file.

### New Text File

Create a new, text input file.

This command opens a new, untitled document, and makes it the active window. The application prompts you to name untitled documents when they are closed.

### Open Text File

Open an existing text input file.

This command displays the Open a File dialog box so you can select a file to load into a new document window.  You can also create a new document by naming a file that doesn't currently exist.

### Close Text File

Close the current document.

This command closes the currently active window.

### Save Text File

Save the current document if its contents have changed.

This command saves the document in the active window to disk. If the document is unnamed, the Save File As dialog box is displayed so you can name the file, and choose where it is to be saved.

### Save Text File As

Save the current document under a new name.

This command allows you to save a document under a new name, or in a new location on disk.  The command displays the Save File As dialog box.  Enter the new file name, including the drive and directory.  All windows containing this file are updated with the new name.  If you choose an existing file name, you are asked if you want to overwrite the existing file.

### Print Preview

View a sample printout of the current document.

This command opens a window that displays how the active document will appear when printed.  The preview window shows one or two pages of the active document as they would be laid out on printer pages.  Controls on the window allow you to page through the pages of the document.

### Print

Print the current document.

This command prints the contents of the active window.  Use Print Preview to see how the document will be laid out on printer pages.  Use Print Setup to select a printer and to set printer options.

### Print Setup

Set printer characteristics.

This command displays the Printer Setup dialog box which allows you to select and configure the printer to be used to print documents in the application.

### Exit

Exit MultiBatchDS.

This command exits MultiBatchDS.  If you've modified documents without saving, you'll be prompted to save before exiting.

## Edit Menu

Used to modify or create the keyword input file, via the editor (filename.TXT). See subsection 3 for keywords.

This menu provides commands to undo edits, access the clipboard, and to delete text.

### Undo

Undo the previous operation.

This command restores the file in the current window to the way it was before your most recent edit operation.  Undo inserts any characters you deleted, deletes any characters you inserted, replaces any characters you overwrote, and moves your cursor back to a prior position.

### Cut

Delete selected text and move it to the clipboard.  You may also press Ctrl-X on the keyboard.

This command removes the selected text from your document and places the text in the clipboard.  Choose Paste to paste the cut text into another document.  The text remains selected in the clipboard, and can be pasted multiple times.

### Copy

Copy selected text to the clipboard.  You may also press Ctrl-C on the keyboard.

This command leaves the selected text intact and places an exact copy of it in the clipboard.  To paste the copied text into another document, choose PasteCM_EDITPASTE.  The text remains selected in the clipboard, and can be pasted multiple times.

### Paste

Move text from the clipboard to the current document.  You may also press Ctrl-V on the keyboard.

This command inserts the text currently selected in the clipboard into the current window at the cursor position.

### Clear All

Delete all text in document.  You may also press Ctrl-Delete on the keyboard.

This command deletes the entire contents of the current document.  Use Undo to restore the contents.

### Delete

Delete selected text.  You may also press the Delete key on the keyboard.

This command deletes the currently selected text from the current document.  The text is not placed in the clipboard.  Use the Undo command to restore the text.

## Search Menu

Used for searching when editing any text file, the Search menu provides commands to find and replace text.

### Find

Find a pattern of text.

This command searches the current document for a text pattern.  The Find dialog box controls the search process.  Options in the dialog determine whether only whole words are to be matched, whether the case of characters is significant, and whether the search should be conducted forwards or backwards through the document.  As each match is found, it is highlighted in the document.

### Replace

Replace one pattern of text with another.

This command searches the current document for a text pattern, and replaces occurrences of the of the pattern with new text.  The Replace dialog controls the search/replace process.  Options in the dialog determine whether only whole words are to be matched, and whether the case of characters is significant.  The dialog is also used to specify the pattern to search for, and the text to replace occurrences with.

### Next

Find and/or replace the next occurrence of text pattern.  You may also press the F3 key on the keyboard.

This command repeats the last Find or Replace operation.

## Parameters Menu

Used to create input to MultiBatchDS. The inputs to MultiBatchDS are described in Chapter 3.

This menu provides commands to enter parameters for the simulation.

### Policy Options…

Choose an overall operating policy.  See "Policy Options" on page 17 for details.

### Thermodynamic Options…

Choose components and thermodynamic property models.  See "Thermodynamic Options and Data Bank" on page 19 for details.

### Task Specification…

Choose number of tasks.  See "Task Specifications" on page 23 for details.

### Units…

Choose units.  See "Units" on page 23 for details.

### Stochastic AnalysisIDD_STOCHASTIC

Stochastic Analysis.  See "Task Specifications" on page 23 for details.

## Calculate Menu

| {bmc calculate.bmp} | Ater the keyword input file is created or selected, the run option is used to execute MultiBatchDS. The Calculate menu is shown in Figure 4. Click here to view the Calculate menu. The user can browse through the results and error files using this menu. Also shown in the Figure 4 is the progress of the execution and partial results. The user can abort the execution anytime using the cancel button shown in the run time window. |
| --- | --- |

Figure 4:  The Calculate menu.

The Calculate menu provides commands to perform calculations and view results.

### Run Calculation

Run the simulation.

This command runs the simulation on the current input data set.  If there is a text file open, that text file will be run as the input data set.  The text file must be saved before running the calculations.  You will be prompted to save the file if it needs to be saved.

If there is not a text file open, the current data set from the input screens will be used to run the simulation.

### Show Errors

Show errors from the last simulation run.

This command shows the list of errors from the last run of the calculations.  This command is automatically done after each run of the calculations.  If there are no errors, no window will appear.

### Show Results

Show results from the last simulation run.

This command displays the textual results from the last run of the calculations.  It is important that you Run Calculations before doing Show Results, so that the most current data set is used for the results that are shown.

## Plot Menu

The Plot menu provides commands to manage plots.

### View Simulation Plot

| {bmc plot.bmp} | Display a simulation plot. Figure 5 shows sample plot options. Click here to view sample plot options. |
| --- | --- |

Figure 5:  Plot options.

This command brings up a dialog box which allows various variables to be plotted.  Select a an X variable and a Y variable.  The available data depends on the options in the input data set.  The fraction number and the graph type may also be selected.  When OK is pressed the plot will be displayed.  You may view multiple plots at one time by selecting the View Plot Command for each plot that you wish to see.

### CDF Plot

Show a CDF plot for stochastic analysis results.

### Copy Plot to Clipboard

Copy the current plot window to the clipboard.

This command copies the current plot into the Windows clipboard.  Then the plot may be pasted into any other Windows application which supports pasting from the clipboard.

### Print Plot

Print the current plot window.

This command prints the currently active plot to the printer.

## Window Menu

This menu provides commands to control the position and layout of application's windows.

### Cascade

Resize and position all windows in an overlapping pattern.

This command arranges all document windows from the top-left position of the application's main window so that the title bar of each is visible.

### Tile

Resize and position all windows in an non overlapping pattern.

This command arranges all document windows side-by-side in a non-overlapping pattern.

### Arrange Icons

Align all iconized windows along a grid.

This command arranges all iconized windows into rows along the bottom of the application's main window.

### Close All

Close all windows.

This command closes all document windows open in the application.

## Help Menu

Provide access to the online help file—information is included about every screen along with tutorials and example problems.

The Help menu provides access to the help system and the about dialog.

### Contents

Help topic contents.

This command brings up the help window with the contents of the MultibatchDS online help displayed.

### Tutorial

MultiBatchDS tutorial

This command brings up the help window with the tutorial portion of MultiBatchDS online help displayed.

### Using Help

This command brings up the help window with the contents of Microsoft Windows Using Help displayed.

### About

This command displays the About MultiBatchDS dialog box, which contains release and copyright information.

# Graphical Input

## Overview

| {bmc structure.wmf} | The structure of the package is outlined in Figure 6. The user can choose the operating policy, and the tasks (simulation, design, or optimization with different levels of models). Various thermodynamic options and property databanks are also available with this package. The educational version of the package allows for maximum of 10 components and 30 plates. Click here for a visual representation of the structure of the package. A brief description of each of the above options is furnished below: |
| --- | --- |

Figure 6: Structure of the Package.

## Policy Options

Conventional batch distillation column (a rectifier)  in practice, operates in one of the following modes (Figure 7):

•	Constant reflux and variable product composition.

•	Variable reflux and constant product composition of a key component.

•	Optimal reflux or Optimal control policy which is a trade-off between 1 and 2. In Optimal control the column is operated with a reflux rate policy which optimizes a chosen performance index. In MultiBatchDS the use of shortcut method (with time implicit equations) and Pontryagin’s maximum principle with nonlinear programming (NLP) method facilitates the development of an optimal control policy (regular profile). It is possible to obtain optimal reflux policy for multi-fractions for all the different models (shortcut, semirigorous, reduced order, and rigorous) using different profile options (step, linear,  or exponential) by using NLP optimization.  The performance index can be either maximum distillate, minimum time, or maximum profit based on variety of cost models.    For details, please refer to Chapter 5 and 6 (Diwekar, 1995).

| {bmc policy.bmp} | Click here to view the Policy Options dialog box. |
| --- | --- |

Figure 7: Policy options.

Similar to the rectifier, the inverted batch distillation column (a stripper) has three operating policies , namely constant reboil (ratio), variable reboil  (ratio), and optimal reboil (ratio).

The middle vessel column which is a combination of a rectifier and a stripper can have constant reflux and constant reboil, variable reflux and variable reboil, and optimal reflux and optimal reboil  as the three main policies.  Apart from these three, it can have combinations of reflux and reboil policies which can be specified using the sub-policy command in the design related parameters menu, once the middle vessel is selected.

The user can choose the operating policy— constant reflux and/or constant reboil, variable reflux and/or variable reboil , or optimal reflux and/or optimal reboil.

## Thermodynamic Options and Data Bank

MultiBatchDS could be easily linked to commercially-available or in-house data banks.  Please contact BPRC for additional  information about datalinks.

Apart from providing a constant relative volatility option, a wide variety of thermodynamic models are included in the package for calculating enthalpies, liquid densities, and vapor and liquid fugacities. The fugacity methods are based on the equation of state as well as the group contribution methods for both liquid and vapor-phase nonidealities (Table 1a and b).

This version of the package includes the CRANIUM Databank of 640 components from.  See "Component list from the CRANIUM databank" on page 63 for a list of components and the component code.

The models which are currently running with the CRANIUM data bank are

Vapor Phase :

  - Ideal

Liquid Phase :

  - Ideal

  - Wilson (with user entered interaction parameters, please see the         data- entry screens below)

  - UNIFAC - 1 Parameter

  - UNIFAC - 2 Parameter

  - UNIFAC - 3 Parameter

A. Vapor Phase

1. Ideal

2. Virial equation

3. Redlich-Kwong

4. Soave-Redlich-Kwong

5. Peng-Robinson

6. Lee-Kesler-Plocker

7. Lee-Kesler-Plocker

Table 1a - List of Fugacity options

B. Liquid Phase

1. Ideal solution

2. Margules

3. Van-Laar

4. Wilson

5. Renon (NRTL)

6. UNIQUAC 1,2, or 3 parameters

7. UNIFAC-1, 2, or 3 parameters

8. Redlich-Kwong

9. Soave-Redlich-Kwong

10. Peng-Robinson

11.Lee-Kesler-Plocker

Table 1b: List of Fugacity Options

Figure 8: Thermodynamic Options

MultiBatchDS allows user to overwrite the data from the data-bank or enter data for a new component (user1, user2..), or enter data for the Wilson binary interaction parameters through the data-entry screens.

data-entry screens

Data Entry Component Selection

The user can override existing data or enter new data for each selected using the  Data Entry for ComponentIDD_DATAENTRYCOMPONENT button.

Data Entry  for Component

This screen allows the user to override the existing data or enter new values for the vapor pressure constants (Ai, Bi, and Ci), molar liquid volume data (Vi), and interaction parametersij ) for each component.

Vapor pressure equation (Antoine constants, Ai, Bi, and Ci) :

log 10(Pis, mm Hg) = Ai - Bi/ (t [degrees C] + Ci)

Molar liquid volume: Vi in cubic meters/kg moles

:

Wilson equation for activity coeffcient i:

ln (I) = -ln (xi ij) + 1 -  (xk ki)/xj kj)

where ij are given in terms of the temperature dependent interaction parameters ij as

ij =  Vj / Vi [exp ( - ij / T [degrees K] )]

reaction-constants  screens

Reaction Constants

MultiBatchDS allows reactions in the pot (for the conventional batch column).  You can enter reactions constants for each component.

See Also

Specifying Reaction ConstantsIDD_REACTCONSTCOMPONENT

Specifying Reaction Constants

The reaction rate is specified using the following equations for each component i.

dCidt=Kfie(−EfiRT)CjGfj−Krie(−EriRT)CjGrj

where

- C is the concentration of component given by the subscript in k-moles/m3

- E is the activation energy in kcal/k moles

- K is the pre-exponential factor for the rate constant of reaction in m3/(hr*kmole)

subscripts and superscripts

- f refers to forward reaction

- i refers to component i

- r refers to reverse reaction

Greek symbols

- refers to the order of the reaction with respect to the specific component given by the subscript

## Units

| {bmc units.bmp} | Figure 9 shows Click here to view unit options for various input and output quantities. The figure also shows the default units (The educational version of the package is restricted to the default units). Currently Energy is expressed in SI units. |
| --- | --- |

Figure 9: Unit Options

## Stochastic Analysis

A fully integrated add-on for uncertainty analysis based on probabilistic techniques and Monte Carlo simulations. Contains a library of distribution functions and various sampling techniques including a recently developed efficient sampling technique which can be 100 times faster than the Monte Carlo method.  Please contact BPRC Inc. for details.

## Task Specifications

The Task Command allows you to select the number of tasks in your data set.  Then each separate task may be configured by pressing the IDD_TASKDIALOG Task button.  The default input parameters for tasks 2 to 10 are the same as that of Task 1 unless changed using each task option.  The task option can be used for sensitivity analysis by changing few parameters from the previous task.

| {bmc task.bmp} | After the number of tasks (maximum 10) is selected, user can enter the data for each task using the following menus (Figure 10a and b). Click here to view the menus. |
| --- | --- |

Figure 10a: Tasks specifications

Figure 10b: Task specifications

### Design

This module is used for the iterative design of batch columns using the shortcut model. It yields preliminary estimates of the design parameters and more importantly the feasible operating window. The user has to supply minimum input information (components, thermodynamic model and design specifications). The design can be verified using rigorous models.

### Simulation

| {bmc guide.wmf} | This module is used for batch distillation column simulation. In MultiBatchDS different simulation models are incorporated, which makes the package computationally more efficient. Guidelines for the use of models are provided in Figure 11. Click here to view guidelines for the use of models. |
| --- | --- |

Figure 11: Guidelines for the use of Models

### Optimization

The shortcut method (for rectifier) is used in this module to arrive at an optimal design for batch columns operating under constant or variable reflux conditions (Diwekar et al., 1989; Diwekar, 1995).  The problem analysis, feasibility conditions,  and solution methods are incorporated in the package.

### Thermo-Estimation

MultiBatchDS can estimate thermodynamic parameters for a user defined component (component ID from 3001-3100) from the X, Y data stored in file ‘BATTHE.TMP’.  The data is stored in a free formatted two columns input.  A simple distillation (Number of plates =0) data is preferred.  The parameters which can be estimated include: relative volatility or vapor pressure data, mass transfer coefficients, and binary interaction parameters.  Thermodynamic estimation uses the same nonlinear programming (NLP) optimization method and hence, optimization method related parameters need to be specified along with other input specifications.  This is an add-on for the package.  Please contact BPRC Inc. for the details.

## Model Options

The hierarchy of models available with MultiBatchDS (Figure 11) is given below in decreasing order of computational efficiency and increasing order of complexity.

### Short Cut Model

(Diwekar and Madhavan, 1991b) - Based on the shortcut method for continuous columns and involves solution of algebraic equations at each time step.  Applicable to nearly-ideal systems and columns with constant molal overflow and negligible holdup effects.  This model is used in the design option to find the feasibility region of operation.

### Semi-Rigorous Model

Uses the plate-to-plate computation and involves solution of a few differential equations and algebraic equations for plate-to-plate calculations. Rigorous model for nonideal systems with negligible holdup effects. Especially useful for highly stiff systems where stiff integrators have difficulty in finding the step size.

### Reduced Order Model

Employs the collocation method to reduce the order of the system differential equations of the rigorous method.  Reduced order approximation for ideal/nonideal systems, columns with significant holdup effects.  Heat effects are considered. This model can be used for non-equlibrium or rate-based approach and/or for packed columns.

### Rigorous Model

Involves solution of stiff differential equations for the material and energy balances.  Rigorous model for ideal/nonideal systems, columns with significant holdup effects.  Heat effects are considered.

## Column Specifications

The column specifications are divided into five categories: Design Related, Operation Related, Calculation Related,  Optimization Related, and Estimation Related.

### Design Related

| {bmc design.bmp} | The Design Related dialog box (Figure 12) includes selection of type of column (rectifier, stripper, or middle vessel), middle vessel sub-policies, and other design parameters described below. Click here to view the dialog box. |
| --- | --- |

Figure 12: Design Related Parameters

#### Middle Vessel Sub-Policies

Included in the design parameters is the selection of sub-policy for the middle vessel.  Apart from the main policies namely; constant reflux/constant reboil ratio, variable reflux/variable reboil ratio, and optimal reflux /optimal reboil ratio, the middle vessel can operate with constant reflux /variable reboil ratio, variable reflux/constant reboil ratio, constant reflux /optimal reboil ratio, variable reflux/ optimal reboil ratio, optimal reflux/constant reboil ratio, and optimal reflux/variable reboil ratio ( total of nine possible policies for middle vessel).  These  can be specified using the sub-policy options.  The selected sub-policy will replace one of the two specifications (reflux or reboil ratio) from the main policy.  For example, if the main policy is constant reflux/ constant reboil ratio and sub-policy selected is variable reflux resulting in variable reflux/constant reboil ratio policy for the middle vessel.

The various design parameters which can be specified using this screen are reflux ratio R (for the rectifier and the middle vessel) , top plates N (for the rectifier and the middle vessel), bottom plates NB (for the stripper and the middle vessel), and Reboil ratio Rb (for the stripper and the middle vessel).  For the design task (currently only available for the rectifier) one can specify initial N/Nmin (constant reflux) or initial R/RMIN (variable reflux).

### Operation Related

| {bmc operation.bmp} | The first part of the operation related screen (see Figure 13) is related to the startup operation of batch distillation. Click here to view the screen. In general for the reduced order and the rigorous model, the startup condition is the total reflux condition. The startup operation in batch distillation consists of the following three steps. |
| --- | --- |

- Charging the feed from the top so that initially the pot composition as well as  composition on each plate is equal to the feed composition.

- Preheating of the still charge to its bubble point.

- Running the column at the total reflux condition.

The total reflux operation can be carried out till the steady state is achieved.  The time required to attain this steady state is the Equilibriation Time.  Simulating the startup operation can be computationally expensive.  Therefore, MultiBatchDS allows for obtaining the compositions and flows at the end of the startup operation using a steady state model.  This can be achieved by not specifying the equlibriation time.  Several times in the plant, the total reflux operation is interrupted before the equlibriation time is reached, with MultiBatchDS one can simulate this operation by specifying the equilibriation or the startup time equal to the time when the operation is likely to be interrupted.  If the time specified is greater than the time required to reach the steady state conditions, MultiBatchDS will stop at the steady state condition providing the user the estimate of the actual Equilibriation time.

| {bmc pressure.bmp} | This screen also allows you to input total feed, feed composition, plate and condenser holdup (needed for the reduced order, and rigorous models), vapor rate or distillate rate for the top column (for the rectifier and the middle vessel), vapor bottom rate (for the stripper and the middle vessel), and temperature of the feed and pressure of the column (if thermodynamic option is other than constant relative volatility). The pressure of the column can be specified as a constant or as a profile (Figure 14), again if the thermodynamic option is not constant relative volatility. Click here to view the Pressure Profile screen. |
| --- | --- |

Figure 13: Operation Related parameters

| {bmc fraction.bmp} | In batch distillation, different cuts or fractions can be obtained from the same column. Number of fractions (maximum allowed 10) can be specified in the operation related parameters. For each fraction the input can be specified using the Edit Fraction screen (Figure 15). Click here to view the Edit Fraction screen. |
| --- | --- |

Continuous side feeds and/or products to the column is possible using MultiBatchDS. For each side feed and side products the input can be supplied using the Edit Side Feed and Edit Side Product (Figure 16 and 17) screens.

Figure 14: Entering Pressure Profile

Figure 15: For Each Fraction

For each fraction the user can select stopping criterion (1=batch time, 2=final bottom composition, 3=average distillate composition, 4=total distillate) and the data corresponding to the stopping criterion (batch time, final composition of the key component in the bottoms, average composition of the key component in the distillate, and final distillate).  For variable reflux or reboil ratio operation the average distillate composition stopping criterion can not be used.  However, the average composition of the key component in the distillate for variable reflux operating mode, and average composition of the key component in bottoms for variable reboil ratio operating mode, need to be specified.

Apart from the stopping criterion related data, one can specify a reflux ratio for each fraction (this specification will override the overall reflux ratio specified in the design related variables. It is also possible to just specify decanter at the top instead of the reflux ratio for each fraction.

Recycling a waste cut/fraction (or fraction of this waste cut) is a common practice in batch distillation.  How much of the current fraction is to be recycled back can be specified using the percent waste fraction recycled command.  The fraction after which this waste cut is recycled to the reboiler needs to be specified to use the recycle waste cut operation.

This screen is also used to specify additional feed to be added after the current fraction, and composition of this additional feed.

#### Edit Side Feed

Figure 16: Side Feed Specifications

| {bmc side.bmp} | Inputs for each side feed include: location of the side feed (side feed plate), side feed flow, feed condition (vapor or liquid), and composition of the side feed. Click here to view the Side Feed Specifications screen. |
| --- | --- |

#### Edit Side Product

Figure 17: Side Product Specifications

| {bmc product.bmp} | Inputs for each side product include: location of the side product (side product plate), side product flow, and side product vapor fraction. Click here to view the Side Product Specifications screen. |
| --- | --- |

### Calculation Related

| {bmc calc.bmp} | This screen (Figure 18) includes input variables required for calculation of shortcut (initial light key and initial heavy key component) and for calculation of reduced order model (number of collocations point, default = 1). Click here to view the Calculation Related parameters screen. |
| --- | --- |

Figure 18: Calculation Related Parameters

### Optimization Related

| {bmc frame.wmf} | Figure 19 shows the pictorial representation of the NLP optimization framework. Click here to view a pictorial representation of the NLP optimization framework. |
| --- | --- |

Figure 19: NLP Optimization Framework

Figure 20: Optimization Related Parameters

The goal of an optimization problem is to determine the decision variables (e.g. number of plates N, Reflux R,  stopping criterion, or coefficients of the reflux profile Cf1 and Cf2) that optimize some aspect of the model represented by the objective function.  The objective function can  be profit, distillate, or time formed by manipulating the cost coefficients (Figure 20) and product sales values for each fraction (Pt, Ps, Pb) (Figure 21).

| {bmc opt.bmp} | For maximum distillate and minimum time problem the cost coefficients should be set to zero with batch time as a stopping criterion for maximum distillate problem and total distillate as the stopping criterion for minimum time problem. Click here to view the Optimization Related parameters screen. |
| --- | --- |

The purity constraints can be specified in terms of average overall product composition for each fraction (Figure 21).   As seen in the Figure 19, the optimizer invokes the model with a set of decision variables.    The model simulates the phenomena and calculates the objective function and constraints.  The NLP optimization algorithm needs partial derivatives in most cases (except for the shortcut model optimal regular profile and for the constant reflux, variable reflux optimization) obtained by perturbing each decision variable by a small fraction (given by the perturbation size in Figure 20, default=0.05).    The information is utilized by the optimizer to calculate a new set of  decision variables using either feasible or infeasible path approach (Biegler and Cuthrell, 1985).    This iterative sequence is continued until the optimization criterion that Kuhn-Tucker error  should be zero within fractional tolerance (provided by the Kuhn-Tucker error parameter in Figure 20) is satisfied or the iteration limit (default 20, maximum 200) is reached.   In general the optimization related parameters shown in Figure 20 include  algorithm related, cost model related, overall design related, and parameters related to each fraction.  The algorithm related parameters include perturbation size, optimization method,  Kuhn-Tucker error tolerance, and maximum number of iterations.  cost model related parameters are defined below.

Cost Model Id=1 (Diwekar et al., 1989):

where c1 is the amortized incremental cost in $/sq.m/plate/y, c2 is the amortized incremental cost of tubular equipment in $/sq.m/yr, and c3 is the cost of steam or coolant to vaporize and condense respectively, per Kgmole of the distillate. Ga is allowable vapor velocity, Gb is the vapor handling capacity of the equipment, and ts is the setup time.   The c1,c2, and c3 are cost coefficients, and Ga and Gb are velocity constant.

Cost Model Id = 2 (Logsdon et al., 1991):

for cost index= 2:

where K1, K2, and K3 represent the cost coefficients and HRs represents the hours per year.

| {bmc frac.bmp} | The overall design parameters include the upper bound, the lower bound and the initial values of the overall design related parameters for the constant reflux (number of plates and reflux ratio), variable reflux (initial reflux ratio and bottom composition), and optimal reflux ( total batch time cumulative of all the fractions) conditions. The fraction related parameters can be accessed by the Edit Fraction command leading the fraction related input screen shown in Figure 21. Click here to view the fraction related input screen. |
| --- | --- |

Figure 21: Optimization Related Parameters for Each Fraction

The optimization related parameters for each fraction includes  the product purity specification, product costs, and profile coefficients.  The product purity  can be specified by average overall product composition for the particular  key component (key product fraction).  The product composition can be greater than (average overall product specification= +1) or less than (average overall product specification= -1) the specified purity.  The products for each fraction for various configurations are top and still product for the rectifier, still and bottom product for the stripper, and top, still, and bottom product for the middle vessel.    The profile coefficients are the decision variables related to the optimal reflux/optimal reboil mode and represent the coefficients in the following equations.  The upper bound, lower bound, and initial values of these decision variables are necessary for the optimizer.

### Estimation Related

The parameters which can be estimated include: relative volatility or vapor pressure data, mass transfer coefficients, and binary interaction parameters of each component.  Thermodynamic estimation uses the same nonlinear programming (NLP) optimization method where the properties to be estimated form the decision variables and hence upper, lower bounds, and initial value needs to be specified for the selected decision variables.  This is an add-on for the package.  Please contact BPRC Inc. for the details.

# MultiBatchDS Tutorial

## The Tutorials

MultiBatchDS is a general-purpose package for the design, simulation and optimization of multicomponent batch distillation columns operating under different modes. The first part of this chapter is intended to show some of its main features and applications by using two simple examples.  Then, a step by step procedure is given for some of the most specialized features of MultiBatchDS.  It will be assumed that the user has started a session with MultiBatchDS and is ready for the definition of the parameters required for the solution of the problem.

## Example 1

A mixture containing 60 moles of pentane and 40 moles of hexane is to be distilled in a batch distillation column. The column has 5 theoretical plates with a holdup of  2 moles per plate a a condenser holdup of  2 moles. The vapor boilup rate of the reboiler is 100 moles/hr.  Assume an initial total reflux condition. The column is operating under a constant reflux mode with reflux ratio equal to 1.0.  It can be assumed that the ideal model gives a good estimation for the fugacity of both the liquid phase and the vapor phase. Assume a feed temperature equal to 20 C and a constant pressure operation with the pressure equal to 1 atm.

- Simulate a 1 hr operation of the column using MultiBatchDS without specifying the equilibration time.

- Repeat a) but now set the value of the equilibration time to 1 hr.

- Change slowly the plate holdup for the colum specified in a) and find out the approximate switching point from the rigorous model to  the semirigorous model.

- Simulate a 1 hr operation for the column specified in a) using the semirigorous model (zero holdup model)

### Solution

The solution to each part of this problem (as any other simulation or design in MultiBatchDS) can be divided in  two main tasks: in  three main tasks:

- To define the parameters of the problem

- To perform the calculations required for the solution.

- To analyze the results obtained from the simulation

Hence, here the attention will be focused in three of the buttons of the main menu of MultiBatchDS:

- The Parameters button

- The Calculate button.

- The Plot button.

### Part a)  Step by Step Procedure

In order to perform a simulation, sufficient information should be provided in each of three submenu options of the parameters menu:

- Policy Options

- Thermodynamic Options

- Task Specification

preferably in that specific order.  The numerical values and options used in this example will be in agreement with the statement of the problem given above.

#### I.  Policy Specification

- Hit the Parameters button.

- Select Policy Options. The Policy Options window was shown in Figure 7.  There are 3 possible policy options: Constant Reflux/Reboil, Variable Reflux/Reboil and Optimal Reflux/Reboil.

- Select Constant Reflux/Reboil

- Hit the OK button.

#### II   Thermodynamic Options

Figure 8 shows the Thermodynamic Options window.

- Hit the Parameters button.

- Select Thermodynamic Options. There are two main things that should be defined here: Thermo Settings and Fugacity Options. Thermo Settings is used to define the number of components and the relative volatility (for constant  models) or the Components Codes (for components appearing in the data bank).  Fugacity Options are ignored for the case of constant  models. There is also a small box in which the user can specify that energy balance calculations must be performed.  Since in this example energy balance calculations are not going to be required, that box will be ignored.

- Under Thermo Settings select Component Codes.

- Define Number of Components equal to 2.

- Define the component 1 to be the component pentane.

- Define the component 2 to be the component hexane.

- Under Fugacity Options and Liquid Phase select ideal. If you do not define this option, the default is ideal.

- Under Fugacity Options and Vapor Phase select ideal.

- Hit the OK button.

#### III   Task Specifications

- Hit the Parameters button.

- Select Task Specifications. The Task Specification window show in Figure 10a appears.

- Define Number of Tasks to be 1. If you do not define this number, the default value of 1 is assumed. It can be be number in between 1 and 10.

- Hit the Task 1 button. The Task 1 Specification window show in Figure 10b will pop up.

- Under Tasks, select Simulation. Other options are Design and Optimization.

- Under Model Options, select Rigorous. Shortcut, Semirigorous and Reduced Order are also available.

- Under Column Specification, hit the Design Related button.  The Design Related window of Figure 12 appears.

- Select Rectifier. As it can be seen, Stripper and Middle Vessel are also available.

- Define Reflux Ratio to be 1. Note that since Rectifier was already set, the only options now available are Reflux Ratio and Top Plates.

- Define Top Plates to be 5.

- Hit the OK button. You will return to the Task 1 window.

- Under Column Specification, hit the Operation Related button. The Operation Related window shown in Figure 13 appears. Note: In this section you will realize that the default units for pressure are atmospheres, Celsius degrees for temperature, hours for time and moles for flow.

- Define Initial Reflux to be Total.

- For this part DO NOT define Equilibration Time.

- Under Pressure, select Constant and define the value to be 1.

- Define Feed Temperature to be 20

- Define Feed to be 100.

- Define Feed Compositions for each component. Component 1 is 0.6 and component 2 is 0.4.

- Assign a value of  2 to both the Condenser Holdup and the Plate Holdup.

- Select Vapor Rate and define a value of 100.

- Define Number of Fractions to be 1. If you do not specify this number, the value of 1 is assumed.

- Hit the Edit Fraction button.  The Fraction 1 window of Figure 15 appears.  Note that the user must select one of the following Stopping Criterion:

- Batch Time

- Final Bottom Composition

- Average Distillate Composition

- Total Distillate

- and the data corresponding to the Stopping Criterion selected above are:

- Batch time

- Key Component Bottom

- Final Bottom Composition of the key component in the bottoms

- Key Component Distillate

- Average Distillate Composition of the key component in the distillate

- Final Distillate

- For instance, if the total Distillate is defined as Stopping Criterion, then the Final Distillate has to be given.

- The average distillate composition stopping criterion can not be used for variable reflux or reboil ratio operations.  On the other hand, the average distillate composition of the key component in the distillate for variable reflux operating mode, and average bottom composition of the key component in bottoms for variable reboil ratio operating mode, need to be specified.

- According to the problem statement, under Stopping Criterion, select Batch Time.

- Assing the value of 1 to the Batch Time.

- Hit OK button. You will return to the previous window. Note that Side Feeds and Side Products are also available, but they are not used in this example.

- Hit OK button. You will return to the Task 1 window.

- Hit OK button. You will return to the Task Specification window.

- Hit OK button. The definition of the problem is complete. Now you are ready to ask for the execution of the calculations. However, you may want to save all this information first. You can do that by choosing the “Save BDT File As” option under the button File in the main menu.

#### IV  Performing the Calculations

Once the parameters required for the simulation have been give, the user can proceed to perform the numerical calculations by the Calculate Menu shown in Figure 4.

- Hit the Calculate button.

- Select Run Calculations. A small window will appear showing the remaining portion of the simulation. See Figure 22.

- Once the calculations have been performed, you will want to see the results.  Hit the Calculate button.

- Select Show Results. A window displaying the data and the results will appear.  See Figure 23.  Click in that window and examine the results. There is another option under the Calculate button. This option is Show Errors. MultiBatchDS will find out the cases on which you are not providing enough information for the solution of a problem. Error messages are saved in temporal files. You can access to this information by clicking the Show Errors button.

Figure 22: Displaying the progress of a simulation

Figure 23: Numerical Results

#### V   Plotting Results

The result of the simulation can be analyzed graphically. MultiBatchDS allows you to plot several dynamic variables against each other. This capability is provided under the Plot Menu.

- Hit the Plot button.

- Select View Simulation Plot.  The View Plot window shown in Figure 5 appears.

- Select your X and Y variable.  Several combinations can be selected.

- Hit OK button.

- The graph you asked for will appear in a separate window. Note that both the results window and any graph window can be closed at any time. A typical graph is shown in Figure 24. Note that, for this example, the number of tasks and the number of fraction is 1 and, therefore, these two default values did not have to be modified in the View Plot window. For the cases in which either the number of tasks or the number of fractions is different from 1, the user can modify either value in order to plot the data of interest.

Figure 24: Graphical Results

#### V   Editing the Chart

The graph generated by MultiBatchDS can be modified accordingly to the preferences of the user by using the tool bar located at the top of the window displaying the plot (See Figure 24). There are 16 options in the tool bar. Click in each of these options with the right button of the mouse: a short message will appear describing the meaning or application of it. If you click with the left button of the mouse, then you actually are selecting that option. As a summary (the user can experiment by himself/herself in detail), the options are:

- Import chart

- Export chart

- Copy chart to the clipboard as a bitmap

- Copy the data to the clipboard as text

- Print the Chart

- Change color

- Switch between 3D and 2D. In Figure 25, the numerical results shown in Figure 24 are shown in 3D.

- Rotate the Chart

- Zoom

- Show or hide the series legend

- Vertical grid

- Horizontal grid

- Edit titles

- Change Text font

- Tools. This option has a submenu. Basically, the options in the submenu are toggles to able/disable the following:

- Palette Bar

- Pattern Bar

- Data Editor

- When the three of them are enabled, the chart looks like the one shown in Figure 26.

- Change chart. This option gives the user a number of options to modify almost every single aspect of the chart. The chart properties window shown in Figure 27 appears by selecting this option.

Figure 25: Graphical Results in 3D

Figure 26: Tools

Figure 27: Chart Properties

### Part b)

The procedure is the same as in part a) , the only difference is in III-15. For this case the Equilibration Time should be set to 1. Of course you can repeat all of the steps  for solving this part, but a much easier approach is to modify the BDT generated in part a) and to save this new file with a new name. Again, to do that use “Save BDT File As” under the File option in the main menu.

### Part c)

Same procedure as in a), but the values for the Condenser Holdup and Plate Holdup (step III-19) will be less than 2. A Value of 0.01 works slowly, but it is near the solution.  If you reduce the holdup further, the rigorous model will not be able to integrate and you are forced to use the Semirigorous model.

### Part d)

Same procedure as in part a), but in III-6 select Semirigorous. Semirigorous implies a zero holdup model. You can take the file generated in part a) and modify it. If you do that, you will find out that the values for the holdup defined in part a) are deactivated, since they do not make sense any more.

## Example 2

A mixture containing 40 percent A, 40 percent B and 20 percent C is to be distilled in a batch distillation column. Assume A = 1.5, B=1.25 and C=1.0.

- Use MultiBatchDS to find the feasibility region for the variable reflux policy, if the column is operating in such a way that XD(A) remains constant at 0.9 throughout the operation. The operation is stopped when 40 percent of the original feed is distilled. Assume N=10 for the initial calculation.  Use the semirigorous model and then the shorcut model.  Selection of  initial light key and heavy key components is very important in the shortcut model.  Assume initial light key equal to 2 and initial heavy key equal 2.

- Use MultiBatchDS to find the feasibility region for constant reflux condition if the average distillate composition of A is equal to 0.9. Use the shortcut model.

### Solution

The general procedure is the same as that in Example 1. The difference lies, of course, in different selection for Tasks, Model Options, specific values for the parameters, etc.

### Part a) Step by Step Procedure

#### I.  Policy Specification

- Hit the Parameters button.

- Select Policy Options. There are 3 possible policy options: Constant Reflux/Reboil, Variable Reflux/Reboil and Optimal Reflux/Reboil.

- Select Variable Reflux/Reboil

- Hit the OK button.

#### II   Thermodynamic Options

- Hit the Parameters button.

- Select Thermodynamic Options.

- Under Thermo Settings select Relative Volatility.

- Define Number of Components equal to 3.

- Define a for the component 1 to be 1.5.

- Define a for the component 2 to be 1.25

- Define a for the component 3 to be 1.0

- Hit the OK button.

#### III   Task Specifications

- Hit the Parameters button.

- Select Task Specifications

- Define Number of Tasks to be 1.

- Hit the Task 1 button.

- Under Tasks, select Simulation.

- Under Model Options, select Semirigorous  in the first part and  for the second part use Shortcut.

- Under Column Specification, hit the Design Related button.

- Select Rectifier.

- Define Top Plates to be 10.

- Hit the OK button. You will return to the Task 1 window.

- Under Column Specification, hit the Operation Related button.

- Define Feed to be 100.

- Define Feed Compositions for each component. Component 1 is 0.4, component 2 is 0.4 and component 3 is 0.2.

- Select Vapor Rate and define a value of  50.

- Define Number of Fractions to be 1.

- Hit the Edit Fraction button.

- Under Stopping Criterion, select Final Distillate.

- Assign the value of  40 to the Final Distillate.

- Assign the value of  0.9 to the Average Distillate Composition.

- Hit OK button. You will return to the previous window (Operation Related). If you are using Semirigorous model then go to step 26.

- Hit the Calculation Related button.

- Define Initial Light Key  to be 2.

- Define Initial Heavy Key to be 2.

- Hit OK button. You will return to the previous window (Operation Related).

- Hit OK button. You will return to the Task 1 window.

- Hit OK button. You will return to the Task Specification window.

- Hit OK button. The definition of the problem is complete. Now you are ready to ask for the execution of the calculations. However, you may want to save all this information first. You can do that by choosing the “Save BDT File As” option under the button File in the main menu.

#### IV  Performing the Calculations

- Hit the Calculate button.

- Select Run Calculations. The error window will appear telling you to check your design specifications for both the Semirigorous model and the Shortcut model. See, for instance, the error message shown in Figure 28.

- Now take the Shortcut model. What you can do to solve the problem is to go to step III-5 and select Design instead of Simulation. Select Run Calculations again. A window will appear showing the remaining portion of the simulation. The Design option is iterative. Once the calculations have been performed, you will want to see the results. The Design option will give the feasible values for the Reflux Ratio and Number of plates.

- Hit the Calculate button.

- Select Show Results. A window displaying the data and the results will appear. Click in that window and examine the results.

Figure 28: Displaying errors in the input data

### Part b)

Repeat Part a) with the following modifications:

- I-3 is now  Constant Reflux/Reboil.

- III-18 Stopping Criterion is now Average Product Composition.

- Most of the numerical values for these parameters remain the same as in part a).  However, since we have learnt from the above exercise , the Number of  Plates needs to be higher, so let us try number of plates equal to 14.

- For Constant Reflux case, we also need to specify Reflux Ratio.  Use Reflux Ratio equal to 1.  Specify these parameters using Design Related menu (steps III-22 to III-24).

- Hit the Calculate button.  The column is again not feasible.

- For solving this part, it will be required a Design step similar to that in IV-3.To check what is wrong with this design, use the design option.  Here we need to re-specify the Stopping Criteria as  Final Distillate (steps III-18 and III-19) as Average Product Composition is now your specification.

## Operation Involving Side Stream

A simulation involving continuous Side Feeds and/or continuous Side Products to the column is possible using MultiBatchDS. While defining the specifications of the simulation task, the Number of Side Feeds and the Number of Side Products are defined in the Operation Related window (Figure 13).  These specification are both optional and independent from each other. In the simulation of a batch distillation column, both specifications, anyone of them, or none of them may be given, according to the requirements of the user.  When these features are used, for each side feed and side product, the input can be supplied using the Edit Side Product window (Figure 17) and Edit Side Feed window (Figure 16).

### Step by Step Procedure

### I. Definition of Side Products

For the specification of a Side Product the following steps should be taken:

- Define the Number of Side Products in the Operation Related. For each of the Side Products:

- Select the number of the side product to be specified (if more than 1).

- Click in the button Edit Side Product.

- The Side Product window shown in Figure 17 will appear indicating the number of the side product being defined. Then, 3 parameters will have to be defined:

- The Number of Plate from which the side product will be extracted.

- The amount of the Side Product

- The Vapor Fraction of the Side Product.

- Hit the OK button to return to the Operation Related window.

### II. Definition of Side Feeds

The specification of side feeds is very similar to the definition of side products. For the specification of a Side Feed :

- Define the Number of Side Feeds in the Operation Related window as explained above. For each of the Side Feeds:

- Select the number of the side feed to be specified (if more than 1).

- Click in the button Edit Side Feed.

- The Side Feed window will appear indicating the number of the side product being defined.  See Figure 16.  Then, the following parameters have to be given:

- The Number of Plate to which the side is introduced.

- The amount of the Side Feed.

- If energy balance calculation are going to be performed, define the Pressure and Temperature of the side feed.  The user will not be able to define these two values if the Heat Balance option is not enabled.

- The Condition of the feed. That is, if the feed is liquid or vapor.

- Finally, the Side Feed Composition for each of the components.

- Hit the OK button to return to the Operation Related window.

## Stripping Operation

While defining the task to be performed in the distillation column, MultiBatchDS gives the user the option of performing the simulation in one of three different types of columns: A Rectifier, a Stripper or a Middle Vessel. The selection of one of this options is simple .

### Step by Step Procedure

The step by step procedure was already given in the examples 1 and 2. Very quickly:

- Select Task Specification

- Define the number of Tasks

- Hit the Task 1 button (or the task number that is going to be defined).

- Under Model Specification, hit the Design Related button. The Design Related window shown in Figure 12 will appear.

- The option Stripper can be selected at this point.  The user can verify that once the stripper option has been selected, MultiBatchDS enables the user to define only the data related to a Stripping operation:

- Reboil Ratio

- Bottom Plates

- Enter the corresponding values for Reboil Ratio and Bottom Plates.

## Multifraction Operation

A batch distillation column can be operated in such a way that several fractions or cuts ca be obtained. MultiBatchDS allows the user to specify a maximum of 10 different fractions.

### Step by Step Procedure

The number of fractions can be specified in the Operation Related window (Figure 13):

- Select the Number of the Fractions.

- Hit the button Edit Fraction. The Edition Fraction (Figure 15) screen will appear showing the number of the fraction being currently edited.

After the number of fractions has been defined, the input for each Fraction must be specified by using that window. For each fraction, the definition of the parameters will be basically the same as the definition in a single fraction simulation. The user must define:

- One of the following Stopping Criterion

- Batch Time

- Final Bottom Composition

- Average Distillate Composition

- Total Distillate

- The data corresponding to the Stopping Criterion selected above

- Batch time

- Key Component Bottom

- Final Bottom Composition of the key component in the bottoms

- Key Component Distillate

- Average Distillate Composition of the key component in the distillate

- Final Distillate

- For instance, if the Total Distillate is defined as Stopping Criterion, then the Final Distillate has to be defined.

- The average distillate composition stopping criterion can not be used for variable reflux or reboil ratio operations.  On the other hand, the average distillate composition of the key component in the distillate for variable reflux operating mode, and average bottom composition of the key component in bottoms for variable reboil ratio operating mode, need to be specified.

- Besides the stopping criterion related data, the user can specify a Reflux Ratio for each fraction. This specification is optional and, if given, it will override the overall reflux ratio specified in the Design Related variables.

- As an optional specification, MultiBatchDS lets the user define an Additional Feed introduced to the column after the current fraction. If this option is used, the value and the Composition of this additional feed have to be specified in the boxes provided.

- Recycling a Waste cut/fraction (or fraction of this waste cut) is a common practice in batch distillation.  How much of the current fraction is to be recycled back to the column can be specified using the Recycle Waste Percent box provided in this window.  If this value is defined, then the Number of the Fraction after which this waste cut is recycled to the reboiler needs to be specified as well.

Only items 4 and 5 are specific to the Multifraction operation capability provided by MultiBatchDS.  Also, both of them are optional and may not be needed to perform a Multifraction operation simulation.  The rest of the specifications in the FractioN window are the same already explained before in this tutorial document.

After executing the calculations of a multifractional operated batch distillation column, the numerical results will be displayed separately for each fraction.. The information about the kind of simulation performed and the initial values for the compositions will separate the numerical results among the different fractions.

In the same way, the graphical results can be analyzed for each of the fraction specified by the user.  The View Plot Simulation window (Figure 5) allows the specification of a Number of Fraction. Hence, if, for instance, the user specifies a value of 2 for this parameter, then the simulation plots will be those corresponding to the numerical results of Fraction number 2.

## Units Conversions

MultiBatchDS provides the user with the capability of using different unit systems for the main variables of a Batch Distillation calculation: Temperature, Pressure, area and Flow.

### Step by Step Procedure

To select among the different choices for each of the variables:

- Go to the Parameters menu and click in the option Units. The Units window will appear showing the alternatives for each of the variables (See Figure 9).

- Click in the options of your convenience.

- Click in the OK button.

In particular, for the case of Flow Units, the units to be used depend on the basis used for the calculation:

- Moles

- k moles

- lb moles

- Volume

- m3

- gallons

- liter

- ft3

- Weight

- kg

- lb

The defaults defined by MultiBatchDS are:

- Degrees Celsius for Temperature.

- Bars for Pressure.

- m2 for Area.

- moles for Flow.

Internally, MultiBatchDS always performs its calculations in the default units, but the definition of the data and the graphical and numerical report of the results will be given in the units defined by the user.

The internal conversion of units for variables like Temperature, Pressure and Area is straightforward.  On the other hand, the conversion of the units in the case of the Flow becomes complicated for a number of reasons.  First of all, the conversion of units for the Flow is available only for real (not constant relative volatility) components in a mixture; that is because such a conversion requires the use of the molecular weight of each component. The second and main reason of the complexity appears while working with Volume Flows and compositions. In such a case, the calculation of the density of the components and the mixture for the vapor phase is required. A basic approximation to that problem is using the ideal gas law.

## Heat Balance Calculation

One of the most remarkable features of MultiBatchDS is its capability to perform heat balance calculations during the simulation of a batch distillation column.

A very usual simplification while performing the simulation of a distillation column is the assumption of constant molar flows. That means that the molar amounts of the vapor going upstream and the liquid going downstream do not change from tray to tray along the column. For mixtures containing components with similar heat of vaporization, this is a very reasonable assumption.

MultiBatchDS allows the user removing the assumption of constant molar flow. As a consequence, energy balance calculations have to be performed in order to determine the values of the liquid and vapor flows in each tray as well as the temperature profiles along the distillation column. This is computationally expensive, but it is definitely required when the chemical species present in the mixture are different in nature. An obvious restriction for the execution of heat balance calculation in MultiBatchDS is the fact that the components present in the mixture must be real (not constant relative volatility) components, so that the physical properties needed in the calculation are available.

In this section, we will explain the steps that the user has to follow in order to use energy balance; we will focus, however, only in those steps which are particular to this kind of simulation.

### I.  Heat Balance Specification

- Under the Parameters menu, select Thermodynamic Options. The Thermodynamic Options window (Figure 8) is used to specify that the simulation will be executed with energy balance calculations on it. Just click in the small box belonging to the option Heat Balance. The definitions of the components of the mixture and the thermodynamic model is as explained before.

- The following steps in the specification of energy balance are optional and have as their main goal the definition of the Heat Duty of the reboiler (if this value is known, it can be defined instead of the Vapor Rate. See Steps 8 and 9 in this procedure). Going further in the definition of the problem, follow the steps already explained for the specification of the task:

- Under the Parameter menu, select Task

- Select the Number of Tasks of the simulation

- Hit the OK button

- Hit the Task 1 button

- Click in the Design Related button and define the parameters  of the problem corresponding to this window

- Hit the OK button.

- Click the Operation Related button (Figure 13). Here there is important to give the following explanation. In a heat balance calculation, the rate of the vapor going up from the reboiler can be selected in two possible ways. The first one is explicitly select the option Vapor Rate and define its value, as it has been done in all of the examples presented before in this tutorial.  The second option is the implicit definition of the Vapor Rate by giving the value of the Heat Duty in the reboiler of the column.  The user may notice that there is a button Edit Heat Duty that will only be available if the Heat Duty option is selected. In general it is better to define explicitly the Vapor Rate if such value is available, but the definition of the Heat Duty will be explained here to give an example of its application.

- Click the Heat Duty option.

- Click the Edit Heat Duty button. The window presented in the Figure 29 will appear.

- Select the Heat Duty option and give the value of the heat duty of the reboiler in the corresponding box. An important point here is that, while performing energy balance calculations, MultiBatchDS assumes that the feed is at its bubble point temperature. Therefore, since the amount of the feed is know, by defining the heat duty in the reboiler, the vapor rate is implicitly defined. Note: Both values cannot be defined simultaneously.

- Hit the OK button to return to the Operation Related window.

- The rest of the specifications in the Operation Related window are done as usual.

Figure 29: Edit Heat Duty Window

### II.  Numerical Results in Energy Balance Calculations

Besides the usual profiles of compositions against time, after performing energy balance calculations the profiles of Temperature and Flows of liquid and vapor along the column are available and reported.  It is obvious that these profiles are not required when the simplification of constant molar flow is assumed.

### III.  Graphical Results in Energy Balance Calculations

Since the numerical profiles of Temperature and Flows with respect to the plate number and time are available for energy balance calculations, MultiBatchDS allows the user to graphically analyze those results. To obtain the plots of Temperature and Flows of each of the trays against time, use the following

#### Step By Step Procedure

- Under the Plot Menu, select Flow Rates. The Flow Rate (Figure 30) window will appear asking for the number of tray to be analyzed.

- Select the number of plate of your convenience. the graphical profiles are displayed as in Figure 31.

Figure 30: Selection of the Plate to be graphically analyzed

Figure 31: Flow Rate profiles for a plate

# MultiBatchDS Sample Problems

## Example Files

The sample example files described below are provided with the package.

eth-unit.bdt - Simulation of a rectifier containing a mixture ethanol-water. This example was designed to show the use of non default units.

eth-wat.bdt  - Simulation of a rectifier using rigorous model (nonideal thermodynamics).

eth-wat1.bdt  - Simulation of a rectifier (zero holdup) (nonideal thermodynamics).

exam1a-exam1c.bdt  - Semirigourous simulation of a multicomponent mixture with the constant relative volatility thermodynamic option. Examples a and b use a recitifier while example c uses a stripper.

exam2a.bdt  - Shortcut simulation using the constant relative volatility thermodynamic option.

exam2b.bdt  - Semirigourous simulation using the constant relative volatility thermodynamic option. It uses the same mixture as exam2a.bdt.

exam3a-exam3e.bdt  - This series of examples represents the various parts of the tutorial example number 1.

exam4a-exam4f.bdt  - These examples represents the various parts of the tutorial example number 2.

heat1.bdt - Heat Balance calculation, constant reflux, semirigorous simulation, rectifier. 5 plates. Ideal thermodynamic calculation for both liquid and vapor. The mixture contains Pentane, Hexane and Heptane.

heat1t.bdt  - Heat Balance calculation, constant reflux, semirigorous simulation, rectifier. Ideal thermodynamic calculation for both liquid and vapor. The mixture contains Pentane, Hexane and Heptane. It is the same as heat1.bdt but the column has 10 plates instead of 5.

heat2.bdt  - Heat Balance calculation, constant reflux, rigorous simulation, rectifier, initial total reflux with NO equilibration time. Ideal thermodynamic calculation for both liquid and vapor. The mixture contains Pentane, Hexane and Heptane.

heat3.bdt  - Heat Balance calculation, constant reflux, rigorous simulation, rectifier, initial total reflux with equilibration time. Ideal thermodynamic calculation for both liquid and vapor. The mixture contains Pentane, Hexane and Heptane.

heatm1.bdt  - Heat Balance calculation, constant reflux, semirigorous simulation, middle vessel, 5 plates.  Ideal thermodynamic calculation for both liquid and vapor. The mixture contains Pentane, Hexane and Heptane.

heatm2.bdt  - Heat Balance calculation, constant reflux, rigorous simulation, middle vessel, no equilibration time.  Ideal thermodynamic calculation for both liquid and vapor. The mixture contains Pentane, Hexane and Heptane.

heats1.bdt  - Heat Balance calculation, constant reflux, semirigorous simulation, stripper.  Ideal thermodynamic calculation for both liquid and vapor. The mixture contains Pentane, Hexane and Heptane.

heats2.bdt  - Heat Balance calculation, constant reflux, rigorous simulation, stripper.  Ideal thermodynamic calculation for both liquid and vapor. The mixture contains Pentane, Hexane and Heptane.

inp1.bdt  - Shortcut simulation of a rectifier with constant reflux policy (constant relative volatility thermodynamics option)

inp2.bdt  - Shortcut simulation of a rectifier with variable reflux policy (constant relative volatility thermodynamics option)

inp3.bdt  - Design of a rectifier with constant reflux policy (constant relative volatility thermodynamics option)

inp4.bdt  - Design of a rectifier with variable reflux policy (constant relative volatility thermodynamics option)

inp5.bdt  - Semi-rigorous simulation of a rectifier with constant reflux policy (constant relative volatility thermodynamics option)

inp6.bdt  - Semi-rigorous simulation of a rectifier with variable reflux policy (constant relative volatility thermodynamics option)

inp7a.bdt  - Rigorous simulation of a rectifier with constant reflux policy (constant relative volatility thermodynamics option)
No startup simulation.

inp7b.bdt  - Rigorous simulation of a rectifier with constant reflux policy (constant relative volatility thermodynamics option)
Startup simulation.

inp8a.bdt  - Rigorous simulation of a rectifier with variable reflux policy (constant relative volatility thermodynamics option)
No Startup simulation.

inp8b.bdt  - Rigorous simulation of a rectifier with variable reflux policy (constant relative volatility thermodynamics option)
Startup simulation

inp9a.bdt  - Reduced order model simulation of a rectifier with constant reflux policy (constant relative volatility thermodynamics option)
No startup simulation.

inp9b.bdt  - Reduced order model simulation of a rectifier with constant reflux policy (constant relative volatility thermodynamics option)
Startup simulation.

inp10a.bdt  - Reduced order model simulation of a rectifier with variable reflux policy (constant relative volatility thermodynamics option)
No Startup simulation.

inp10b.bdt  - Rigorous simulation of a rectifier with variable reflux policy (constant relative volatility thermodynamics option)
Startup simulation

inpd1.bdt  - Shortcut simulation of a rectifier with constant reflux policy (with ideal thermodynamics, Cranium databank)

inpd2.bdt  - Shortcut simulation of a rectifier with variable reflux policy (with ideal thermodynamics, Cranium databank)

inpd5.bdt  - Semi-rigorous simulation of a rectifier with constant reflux policy (with ideal thermodynamics, Cranium databank)

inpd6.bdt  - Semi-rigorous simulation of a rectifier with variable reflux policy (with ideal thermodynamics, Cranium databank)

inpd7.bdt  - Rigorous simulation of a rectifier with constant reflux policy (with ideal thermodynamics, Cranium databank)
Startup simulation.

inpd8.bdt  - Rigorous simulation of a rectifier with variable reflux policy (with ideal thermodynamics, Cranium databank) 
Startup simulation

inpdm1.bdt  - Middle vessel semi-rigorous simulation with constant reflux/reboil mode.(with ideal thermodynamics, Cranium databank)

inpdm2.bdt  - Middle vessel rigorous simulation with constant reflux/reboil mode.(with ideal thermodynamics, Cranium databank)
Startup simulation

inpds1.bdt  - Stripper semi-rigorous simulation with constant reflux/reboil mode.(with ideal thermodynamics, Cranium databank)

inpds2.bdt  - Stripper rigorous simulation with constant reflux/reboil mode.(with ideal thermodynamics, Cranium databank)
Startup simulation

inp*sid*.bdt  - Semibatch operation with side feeds and/or side products.

inpsto.bdt  - Stochastic simulation demo.

inpther1.bdt  - Thermodynamic estimation demo1.

inpther2.bdt  - Thermodynamics estimation demo2.

ropt1.bdt  - Optimization demo.

# References

## References

Biegler L. T. and J. E. Cuthrell (1985), Improved infeasible path optimization for sequential modular simulators, Part II, Computers and chemical Engineering, 9. P. 257.

Devidyan A. G., V. N. Kiva, G. A. Meski, and M. Morari (1994), “Batch distillation in a column with a middle vessel”, Chemical Engineering Science, 49, P. 3033.

Diwekar U. M. (1995), Batch Distillation: Simulation, Optimal Design and Control, Taylor and Francis International Publishers, Washington D.C.

Diwekar U. M. (1996),  Understanding batch distillation principles with MultiBatchDS, to appear in Chemical Engineering Education.

Diwekar U. M. and K. P. Madhavan (1991a), Batch-dist a Comprehensive Package for Simulation, Design, Optimization, and Optimal Control of Multicomponent Batch Distillation Columns,’’ Comput. chem. Engng., 15, P. 833.

Diwekar U. M. and K. P. Madhavan (1991b), Multicomponent batch distillation column design, I&EC Res., 30, 713.

Diwekar U. M., K. P. Madhavan, and R. E. Swaney (1989), Optimization of multicomponent batch distillation column, I&EC Res., 28, 1011.

Logsdon  J. S.,  U. M. Diwekar, and L. T. Biegler (1990), On the simultaneous optimal design and opearation of batch distillation columns, Transactions of IChemE, 68,  P. 434.

# Appendix A

## Component list from the CRANIUM databank

| Component Categories | Category ID (CID) |
| --- | --- |
| ALL the needed data is available NO Boiling Point Temperature data NO Antoine Constants data NO Liquid Density data NO Enthalpy data (Liquid, Vapor or both phases) NO UniFac Parameters data NO data available (Missing all of the above: B - F) | A B C D E F M |

| Code | Name | CID | Code | Name | CID |
| --- | --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 | 1,1,1,2-Tetrachlorodifluoroethane 1,1,1,2-Tetrachloroethane 1,1,1,2-Tetrafluoroethane 1,1,1-Trichloroethane 1,1,1-Trichlorofluoroethane 1,1,1-Trifluoroethane 1,1,2,2-Tetrabromoethane 1,1,2,2-Tetrachlorodifluoroethane 1,1,2,2-Tetrachloroethane 1,1,2-Trichloroethane 1,1,2-Trichlorotrifluoroethane 1,1-Dibromoethane 1,1-Dichloroethane 1,1-Dichloroethylene 1,1-Dichloropropane | A A F A A A A A A A F A A A A | 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 | 1,1-Difluoroethane 1,1-Difluoroethylene 1,1-Dimethylcyclohexane 1,1-Dimethylcyclopentane 1,2,3,4-Tetrahydronaphthalene 1,2,3,5-Tetramethylbenzene 1,2,3-Trichloropropane 1,2,3-Trimethylbenzene 1,2,4,5-Tetramethylbenzene 1,2,4-Trimethylbenzene 1,2-Butadiene 1,2-Dibromoethane 1,2-Dibromotetrafluoroethane 1,2-Dichloroethane 1,2-Dichloropropane | F F A A DEF A A A A A F A A A A |

| Code | Name | CID | Code | Name | CID |
| --- | --- | --- | --- | --- | --- |
| 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 | 1,2-Dichlorotetrafluoroethane 1,2-Difluoroethane 1,2-Dimethoxyethane 1,2-Pentadiene 1,2-Propanediamine 1,2-Propylene glycol 1,2-Propylene oxide 1,3-Butadiene 1,3-Butanediol 1,3-Dichloropropane 1,3-Propylene glycol 1,3-Propylene oxide 1,4-Butanediol 1,4-Dioxane 1,4-Pentadiene 1,5-Hexadiene 1,5-Pentanediol 1,6-Hexanediol 1-Amino-2-propanol 1-Bromobutane 1-Bromoheptane 1-Bromopropane 1-Butene 1-Butyne 1-Chloro-1,1-difluoroethane 1--Decana 1-Decanol 1-Decene 1-Heptanal 1-Heptanol 1-Heptene 1-Hexanal 1-Hexanol 1-Hexene 1-Hexyne | A F A F F E F A A A A A A A A A A A A A A A A A A A A A A A A A A A A | 66 67 68 69 70 71 72 73 74 75 76 77 78 79 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 100 | 1-Nitropropane 1-Nonanal 1-Nonanol 1-Nonene 1-Octanol 1-Octene 1-Octylamine 1-Pentanol 1-Pentene 1-Pentyne 1-Propanethiol 2,2,3,3-Tetramethylpentane 2,2,3,4-Tetramethylpentane 2,2,3-Trimethylbutane 2,2,3-Trimethylpentane 2,2,4,4-Tetramethylpentan 2,2,4-Trimethylpentan 2,2,5-Trimethylhexane 2,2-Dimethyl-1-propanol 2,2-Dimethyl-3-ethylpentane 2,2-Dimethylbutane 2,2-Dimethylheptane 2,2-Dimethylhexane 2,2-Dimethyloctane 2,2-Dimethylpentane 2,3,3-Trimethyl-1-butene 2,3,3-Trimethylpentane 2,3,4-Trimethylpentane 2,3-Butanediol 2,3-Dichloropropene 2,3-Dimethyl-1-butene 2,3-Dimethyl-2-butene 2,3-Dimethylbutane 2,3-Dimethylhexane 2,3-Dimethylpentane | A A A A E A A A A A A A A A A A A A A A A A A E A A A A A A A A A A A |

| Code | Name | CID | Code | Name | CID |
| --- | --- | --- | --- | --- | --- |
| 101 102 103 104 105 106 107 108 109 110 111 112 113 114 115 116 117 118 119 120 121 122 123 124 125 126 127 128 129 130 131 132 133 134 135 | 2,3-Dimethylpyridine 2,3-Xylenol 2,4,4-Trimethyl-1-pentene 2,4,4-Trimethyl-2-pentene 2,4-Dimethyl-3-ethylpentane 2,4-Dimethylhexane 2,4-Dimethylpentane 2,4-Dimethylpyridine 2,4-Xylenol 2,5-Dihydrofurane 2,5-Dimethylhexane 2,5-Dimethylpyridine 2,5-Xylenol 2,6-Diethylaniline 2,6-Dimethyl-4-heptanol 2,6-Dimethylheptane 2,6-Dimethylpyridine 2,6-Xylenol 2-Bromobutane 2-Bromopropane 2-Butanol 2-Butyne 2-Chloroethanol 2-Chloropropene 2-Ethoxyethanol 2-Ethyl-1-hexanol 2-Ethyl-1-hexene 2-Ethyl-1-pentene 2-Ethyl-m-xylene 2-Ethyl-p-xylene 2-Ethylhexyl acetate 2-Heptanol 2-Heptanone 2-Hexanol 2-Hexyne | F A A A A A A F A F A F A A A A F A A A A A A A A E A A A A A A A A A | 136 137 138 139 140 141 142 143 144 145 146 147 148 149 150 151 152 153 154 155 156 157 158 159 160 161 162 163 164 165 166 167 168 169 170 | 2-Methoxyethanol 2-Methyl-1-butanol 2-Methyl-1-butene 2-Methyl-1-hexene 2-Methyl-1-pentene 2-Methyl-2-butanol 2-Methyl-2-butene 2-Methyl-2-pentene 2-Methyl-3-ethylpentane 2-Methylheptane 2-Methylhexane 2-Methylnonanone 2-Methyloctane 2-Methylpentane 2-Methylpyridine 2-Nitropropane 2-Nonanol 2-Octanol 2-Octanone 2-Pentanol 2-Pentanone 2-Pyrrolidone 3,3-Diethylpentane 3,3-Dimethyl-1-butene 3,3-Dimethylhexane 3,3-Dimethylpentane 3,4-Dimethylhexane 3,4-Dimethylpyridine 3,4-Xylenol 3,5-Dimethylpyridine 3,5-Xylenol 3-Amino-1-propanol 3-Chloropropene 3-Ethyl-1-pentene 3-Ethyl-o-xylene | A A A A A A A A A A A A A A F A E A A A A A A A A A A F A F A A A A A |

| Code | Name | CID | Code | Name | CID |
| --- | --- | --- | --- | --- | --- |
| 171 172 173 174 175 176 177 178 179 180 181 182 183 184 185 186 187 188 189 190 191 192 193 194 195 196 197 198 199 200 201 202 203 204 205 | 3-Ethylheptane 3-Ethylhexane 3-Ethylpentane 3-Hexyne 3-Methyl-1,2-butadiene 3-Methyl-1-butanol 3-Methyl-1-butene 3-Methyl-1-hexene 3-Methyl-1-pentene 3-Methyl-2-butanol 3-Methyl-3-ethylpentane 3-Methyl-cis-2-pentene 3-Methylheptane 3-Methylhexane 3-Methylnonanone 3-Methyloctane 3-Methylpentane 3-Methylpyridine 3-Pentanol 4-Ethyl-m-xylene 4-Ethyl-o-xylene 4-Methyl-1-hexene 4-Methyl-cis-2-pentene 4-Methyl-trans-2-pentene 4-Methylheptane 4-Methylnonanone 4-Methyloctane 4-Methylpyridine 5-Ethyl-m-xylene 5-Methyl-2-hexanone 5-Methylnonanone Acetaldehyde Acetamide Acetic acid Acetic anhydride | A A A A F A A A A A A A A A A A A F A A A A A A A A A F A A A A F A A | 206 207 208 209 210 211 212 213 214 215 216 217 218 219 220 221 222 223 224 225 226 227 228 229 230 231 232 233 234 235 236 237 238 239 240 | Acetone Acetonitrile Acetophenone Acetyl chloride Acetylene Acrolein Acrylamide Acrylic acid Acrylonitrile Adipic acid Allyl alcohol Allylamine alpha-Epichlorohydrine alpha-Methylstyrene Ammonia Aniline Anisole Argon Arsine Benzaldehyde Benzene Benzoic acid Benzonitrile Benzyl acetate Benzyl alcohol Benzyl chloride Benzyl ethyl ether Bromobenzene Bromochlorodifluoromethane Bromochloromethane Bromotrichloromethane Bromotrifluoroethylene Bromotrifluoromethane Butane Butanethiol | A A A F F A A A A A A A F A EF A A CDE CDEF A A A F A A F E A A A A F E A A |

| Code | Name | CID | Code | Name | CID |
| --- | --- | --- | --- | --- | --- |
| 241 242 243 244 245 246 247 248 249 250 251 252 253 254 255 256 257 258 259 260 261 262 263 264 265 266 267 268 269 270 271 272 273 274 275 | Butanol Butyl acetate Butyl acrylate Butyl amine Butyl butyrate Butyl chloride Butyl ethyl ether Butyl formate Butyl methacrylate Butyl propionate Butyl valerate Butylbenzene Butylcyclohexane Butyraldehyde Butyric acid Butyric anhydride Butyronitrile Carbon disulfide Carbon tetrachloride Carbon tetrafluoride Carbonyl fluoride Carbonyl sulfide Chlorobenzene Chlorodifluoroethylene Chlorodifluoromethane Chloroform Chloropentafluoroethane Chloroprene Chlorotrifluoroethylene Chlorotrifluoromethane cis-1,2-Dichloroethylene cis-1,2-Dimethylcyclopentane cis-1,3-Pentadiene cis-2-Butene cis-2-Heptene | A A F A A A A A F A E E A A A E E E A EF EF EF A F A A A A F A A A A A A | 276 277 278 279 280 281 282 283 284 285 286 287 288 289 290 291 292 293 294 295 296 297 298 299 300 301 302 303 304 305 306 307 308 309 310 | cis-2-Hexene cis-2-Pentene cis-3-Heptene cis-3-Hexene cis-Crotonitrile Cyanogen Cyanogen chloride Cyclobutane Cycloheptane Cyclohexane Cyclohexanol Cyclohexanone Cyclohexene Cyclopentane Cyclopentanone Cyclopentene Cyclopropane Decane Decanoic acid Di-sec-butyl ether Di-tert-butyl ether Diacetone alcohol Diallyl maleate Dibromodifluoromethane Dibromomethane Dibutyl ether Dibutylamine Dichlorodifluoromethane Dichlorofluoromethane Dichloromethane Dichlorosilane Diethyl disulfide Diethyl ether Diethyl ketone Diethyl maleate | A A A A F F F A A F A A A A A F A A A A F A F A A A A A A A CDEF A A A F |

| Code | Name | CID | Code | Name | CID |
| --- | --- | --- | --- | --- | --- |
| 311 312 313 314 315 316 317 318 319 320 321 322 323 324 325 326 327 328 329 330 331 332 333 334 335 336 337 338 339 340 341 342 343 344 345 | Diethyl malonate Diethyl succinate Diethyl sulfate Diethyl sulfide Diethylamine Diethylene glycol Difluoromethane Diiodomethane Diisobutyl ketone Diisobutylamine Diisopropyl ether Diisopropyl ketone Diisopropylamine Diketene Dimethyl disulfide Dimethyl ether Dimethyl maleate Dimethyl phthalate Dimethyl silane Dimethyl sulfate Dimethyl sulfide Dimethyl sulfoxide Dimethylamine Dipentyl ether Dipropyl ether Dipropylamine Ethane Ethanol Ethyl acetate Ethyl acrylate Ethyl benzoate Ethyl bromide Ethyl butyrate Ethyl chloride Ethyl chloroformate | EF A M A A A F A A A A F A F A A F F CDE CDEF A CDE A E A A E A A F F A A A F | 346 347 348 349 350 351 352 353 354 355 356 357 358 359 360 361 362 363 364 365 366 367 368 369 370 371 372 373 374 375 376 377 378 379 380 | Ethyl fluoride Ethyl formate Ethyl iodide Ethyl isobutyrate Ethyl isopropyl ketone Ethyl isovalerate Ethyl mercaptan Ethyl methacrylate Ethyl propionate Ethyl propyl ether Ethyl propyl ketone Ethyl vinyl ether Ethylamine Ethylbenzene Ethylcyclohexane Ethylcyclopentane Ethylene Ethylene glycol Ethylene oxide Ethylenediamine Ethyleneimine Fluorobenzene Formaldehyde Formamide Formic acid Fumaric acid Furan Furfural Glycerol Glyoxal Heptane Heptanoic acid Hexachloro-1,3-butadiene Hexachloroethane Hexafluoroacetone | F A A F A A A F A A A A A A A A EF E F A F A F F A A F A A A A A A A F |

| Code | Name | CID | Code | Name | CID |
| --- | --- | --- | --- | --- | --- |
| 381 383 383 384 385 386 387 388 389 390 391 392 393 394 395 396 397 398 399 400 401 402 403 404 405 406 407 408 409 410 411 412 413 414 415 | Hexane Hexanenitrile Hexanoic acid Hexyl acetate Hexylamine Hexylmercaptan Hydrazine Hydrogen bromide Hydrogen chloride Hydrogen cyanide Hydrogen fluoride Hydrogen iodide Hydrogen peroxide Hydrogen sulfide Indane Iodobenzene Isobutane Isobutanol Isobutene Isobutyl acetate Isobutyl acrylate Isobutyl formate Isobutyl isobutyrate Isobutylamine Isobutylbenzene Isobutyraldehyde Isobutyric acid Isobutyronitrile Isodecanol Isopentane Isopentyl acetate Isopentyl isovalerate Isopropanol Isopropyl acetate Isopropyl chloride | A A A A E A F DEF DEF F EF DEF E DEF CDEF A A A A A F A F A A A A F A A A E A A A | 416 417 418 419 420 421 422 423 424 425 426 427 428 429 430 431 432 433 434 435 436 437 438 439 440 441 442 443 444 445 446 447 448 449 450 | Isopropyl iodide Isopropyl mercaptan Isopropylamine Isopropylbenzene Isopropylcyclohexane Isoquinoline Isovaleric acid Ketene Krypton Lactic acid m-Cresol m-Cymene m-Dichlorobenzene m-Diethylbenzene m-Ethylphenol m-Ethyltoluene m-Methylstyrene m-Toluidine m-Xylene Maleic acid Maleic anhydride Mesityl oxide Mesitylene Methacrylonitrile Methane Methanol Methyl acetate Methyl acrylate Methyl amine Methyl benzoate Methyl bromide Methyl butyl ketone Methyl butyrate Methyl chloride Methyl chloroacetate | A F F A A CDE F F CDEF A A A A A A A A A A A F A A F EF A A F A EF A A A F F |

| Code | Name | CID | Code | Name | CID |
| --- | --- | --- | --- | --- | --- |
| 451 452 453 454 455 456 457 458 459 460 461 462 463 464 465 466 467 468 469 470 471 472 473 474 475 476 477 478 479 480 481 482 483 484 485 | Methyl chlorosilane Methyl cyanoacetate Methyl dichlorosilane Methyl ethyl ether Methyl ethyl ketone Methyl fluoride Methyl formate Methyl iodide Methyl isobutyl ether Methyl isobutyl ketone Methyl isocyanate Methyl isopropyl ether Methyl isopropyl ketone Methyl mercaptane Methyl methacrylate Methyl propionate Methyl salicylate Methyl sec-butyl ether Methyl silane Methyl tert-butyl ether Methyl trichlorosilane Methyl vinyl ether Methylacetylene Methylal Methylcyclohexane Methylcyclopentane Methylethanolamine Monoethanolamine Morpholine N,N-Diethylaniline N,N-Dimethylaniline N,N-Dimethylformamide N-Methylacetamide N-Methylaniline N-Methylformamide | CDEF F CDEF A A F A A A A DEF A A A F A F A CDE A EF A A A A A A A A A A A A A A | 486 487 488 489 490 491 492 493 494 495 496 497 498 499 500 501 502 503 504 505 506 507 508 509 510 511 512 513 514 515 516 517 518 519 520 | Naphthalene Neopentane Neopentyl glycol Nitric acid Nitrobenzene Nitroethane Nitrogen Nitromethane Nonane Nonanoic acid o-Chlorotoluene o-Cresol o-Cymene o-Dichlorobenzene o-Diethylbenzene o-Ethylphenol o-Ethyltoluene o-Methylstyrene o-Toluidine o-Xylene Octanal Octane Octanoic acid Octyl formate Oxazole Oxygen p-Bromotoluene p-Chlorotoluene p-Cresol p-Cymene p-Dichlorobenzene p-Diethylbenzene p-Ethylphenol p-Ethyltoluene p-Methylstyrene | EF A A F A A DE A A A A A A A A A A A A A A A A A F E A A A A A A A A A |

| Code | Name | CID | Code | Name | CID |
| --- | --- | --- | --- | --- | --- |
| 521 522 523 524 525 526 527 528 529 530 531 532 533 534 535 536 537 538 539 540 541 542 543 544 545 546 547 548 549 550 551 552 553 554 555 | p-Toluidine p-Xylene Pentachloroethane Pentafluorobenzene Pentafluoroethane Pentafluorophenol Pentane Pentyl acetate Pentyl formate Pentyl mercaptan Pentylamine Perchlorobenzene Perfluoro-2-methylpentane Perfluorobenzene Perfluorobutane Perfluorobutene Perfluorocyclobutane Perfluorocyclohexane Perfluoroethane Perfluoroethylene Perfluoroheptane Perfluorohexane Perfluoromethylcyclohexane Perfluoropentane Perfluoropropane Perfluoropropylene Perfluorotoluene Phenetole Phenol Phenyl mercaptan Phosgene Phosphine Phosphoric acid Phthalic anhydride Piperidine | A A E A F A A A A A A A F A F A A A E F A A F A E E A A E F F CDEF M DEF A | 556 557 558 559 560 561 562 563 564 565 566 567 568 569 570 571 572 573 574 575 576 577 578 579 580 581 582 583 584 585 586 587 588 589 590 | Propadiene Propane Propanol Propargyl alcohol Propargyl chloride Propionaldehyde Propionic acid Propionitrile Propyl acetate Propyl acrylate Propyl butyrate Propyl chloride Propyl formate Propyl iodide Propyl methacrylate Propyl propionate Propylamine Propylbenzene Propylcyclohexane Propylcyclopentane Propylene Propyleneimine Pyridine Pyrrole Pyrrolidine Quinoline sec-Butyl acetate sec-Butyl chloride sec-Butylbenzene Silane Silicon tetrachloride Styrene Succinic acid Succinic anhydride Succinonitrile | F A A A A A A A A F A A A A F A A A A A A F A F A CDEF A A A CDEF EF A A A A |

| Code | Name | CID | Code | Name | CID |
| --- | --- | --- | --- | --- | --- |
| 591 592 593 594 595 596 597 598 599 600 601 602 603 604 605 606 607 608 609 610 611 612 613 614 615 | Sulfolane Sulfur dioxide Sulfur hexafluoride Sulfuric acid tert-Butanol tert-Butyl acetate tert-Butyl chloride tert-Butylbenzene Tetrachloroethylene Tetrahydrofuran Tetrahydrothiophene Tetramethylsilane Thiophene Thiourea Toluene trans-1,2-Dichloroethylene trans-1,2-Dimethylcyclopentane trans-1,3-Pentadiene trans-2-Butene trans-2-Heptene trans-2-Hexene trans-2-Octene trans-2-Pentene trans-3-Heptene trans-3-Hexene | CDEF DEF DEF M A A A E A A A BCDE A M A A A A A A A A A A A | 616 617 618 619 620 621 622 623 624 625 626 627 628 629 630 631 632 633 634 635 636 637 638 639 640 | trans-3-Octene trans-4-Octene trans-Crotonitrile Tribromomethane Trichloroethylene Trichlorofluoromethane Trichlorosilane Triethylamine Trifluoroacetic acid Trifluoromethane Trimethyl silane Trimethylamine Trioxane Urea Valeraldehyde Valeric acid Vinyl acetate Vinyl bromide Vinyl chloride Vinyl fluoride Vinyl formate Vinylacetonitrile Vinylacetylene Water Xenon | A A F A A A CDEF A A F CDE A F EF A E A A A F A A A E CDEF |

# Appendix B

## Keyword Input

The text input to MultiBatchDS  is also user-friendly and is based on the concept of keywords. A typical input file is shown in Table 2.

SAMPLE KEYWORD INPUT C* Policy Commands POLICY= CONSTANT REFLUX; C* Thermodynamic options and data bank COMPONENT-CODE= 10,20,30,70; THERMO-MODEL= 0,8; C* Task Commands TASK=SHORT CUT SIMULATION; C* Column Specifications FEED-COMPOSITION= 0.35,0.10,0.30,0.25; REFLUX-RATIO = 1.0; LIGHT-KEY =1110; HEAVY-KEY = 117; INIIAL-REFLUX=TOTAL; NUMBER OF -INTERNAL-STAGES = 10; PRESSURE OF -COLUMN (MM HG) = 760; VAPOR-RATE=100;FEED = 100; BATCH-TIME= 1; NUMBER OF -FRACTIONS= 1; C* End Commands END-TASK = 1; END-POLICY = 1;

Table 2: Sample Input

The boldface characters (first three letters) represent the (“keyword”) which are recognized by the program. Parameters assigned to the keywords are separated by a comma (“,”). The commands are terminated by a semicolon (“;”).

The input must be given in the following order (with the exception of the comment commands):

- Policy Commands

- Thermodynamic Option Commands

- Task Commands

- Column Specification Commands

- End Commands

Any number of commands can be given on a line. Any number of spaces may also be included in a command. A new command can be continued on a new line starting from first, second, or third column. A command can be continued on a line provided the new line has blanks in first three columns.

A special comment command can be used for self documentation. This command can be inserted anywhere in the input and allows you to make notes or remarks. To use this feature, simply enter C* starting at the left margin, and enter your comment (see Table 2).

Currently the units for Temperature is in celcius and for pressure in bars or atms.

## Description of Keywords:

### Policy Commands

POL = mode of column operation. The options are constant reflux, variable reflux, and optimal reflux with the following keywords.

CON = constant reflux/constant reboil

VAR = variable reflux/variable reboil

OPT = optimal reflux/optimal reboil (regular)

LIN = linear optimal reflux/optimal reboil

STE = step optimal reflux/optimal reboil

EXP = exponential optimal reflux/ optimal reboil

### Thermodynamic Option Commands

REL-VOL = relative volatility of components.

COM-COD = component numbers from the data bank. We are using the CRANIUM databank which has 640 components. The names of the components with component codes are listed at the back of the manual. The user can append new components to this databank. The databank files are:

- PURE.CON

- PURE.DEN

- PURE.ENT

- PURE.PAR

- UNIFAC.FIL

THE-MOD = thermodynamic models. The first for vapor phase fugacity and second for liquid phase fugacity.  The keywords for different models are described in Table 3a and b. Thermodynamic model interaction parameters can be supplied by the user as described below.

| A. Vapor Phase METHOD | KEYWORDS |
| --- | --- |
| Ideal | 0 |
| Virial Equation | 1 |
| Redlich-Kwong | 2 |
| Soave-Redlich-Kwong | 3 |
| Peng-Robinson | 4 |
| Lee-Kesler-Plocker | 5 |
| GEOS | 6 |

Table 3a:Thermodynamic Options and Key Words

| B. Liquid Phase METHOD | KEYWORDS |
| --- | --- |
| Ideal | 0 |
| Margules | 1 |
| Van-Laar | 2 |
| Wilson | 3 |
| Renon (NRTL) | 4 |
| UNIQUAC-1 param | 5 |
| UNIQUAC-2 param | 6 |
| UNIQUAC-3 param | 7 |
| UNIFAC-1 param | 8 |
| UNIFAC-2 param | 9 |
| UNIFAC-3 param | 10 |
| Redlich-Kwong | 11 |
| Soave-Redlich-Kwong | 12 |
| Peng-Robinson | 13 |
| Lee-Kesler-Plocker | 14 |
| GEOS | 15 |

Table 3b:Thermodynamic Options and Key Words

### Task Commands

TAS = by this command user can specify any task to be performed. The options and keywords are shown in Table 4.

| OPTIONS | KEYWORDS |
| --- | --- |
| Short-cut method simulation | SHO |
| Design | DES |
| Optimization | OPT |
| Semi-rigorous simulation | SEM |
| Reduced order simulation | RED |
| Rigorous simulation | RIG |

Table 4: Task Command and Key Words

### Column Specification Commands

#### Design Related

COL = type of column. REC - rectifier, STR - stripper, MID - middle vessel. The default is rectifier.

REF-RAT= reflux ratio. Overall reflux ratio for all the fractions. If reflux ratio for each fraction is specified in the operation related parameters then that overrides this specification.  To be specified for POL=CON and TAS=SHO, MOD, SEM, RED, RIG and for COL = REC and MID.

NUM-INT-STA= number of internal stages in the top section of the column, excluding reboiler and condenser. This is not required for stripper (COL=STR).

REB-RAT = reboil ratio. required for stripper and middle vessel.

BOT-PLA = number of plates in the bottom section of the column. Required for stripper and middle vessel.

INI-RAT-RMI = initial ratio of R/Rmin required for DES task option for variable reflux policy (should be > 1).  Applicable only for the rectifier.

INI-PLA-RAT = initial ratio of $N/Nmin$ required for DES task option for constant reflux policy (should be > 1). Applicable only for the rectifier.

#### Operation Related

INI-REF = initial reflux, this command is used to indicate that the package should simulate the initial total reflux condition. The command should be specified as follows: INITIAL-REFLUX =TOTAL;

EQU-TIM= equilibriation time specification, this command indicates the modified short-cut, reduced order model, and the rigorous model to simulate the total reflux operation by actual integration.

FEE = feed (moles)

FEE-COM=feed composition in the same order as REL-VOL or COM-COD.

CON-HOL = condenser holdup (moles)

PLA-HOL = plate holdup (moles)

TEM-FEE(degrees C)= feed temperature. May be given to make the initial guess of temperature for VLE calculations.

PRE-COL (bar) = pressure of the column.

VAP-RAT = vapor rate (moles/hr)

DIS-RAT = distillate rate (moles/hr)

VAP-BOT-RAT = vapor bottom rate (mole/hr)

NUM-FRA = number of fractions (default=1)

For Each Fraction:

STO-CRI = stopping criterion, the options available are batch time 1, final still composition 2, average distillate composition 3, and total distillate 4 to be given sequentially for each fraction.

TOT-DIS = total distillate, to be specified if STO-CRI=4.

BAT-TIM = for the stopping criterion TIM, batch time for each fraction in hrs, to be given sequentially for each fraction.

KEY-COM-FRA = key component for each fraction. This keyword is used when the stopping criteria is 2 or 3, or the average distillate composition is specified and the compositions are specified using the keywords FIN-STI-COM or AVE-PRO-COM. To be given sequentially for each fraction.

FIN-STI-COM = final still composition of the key component specified by the keyword KEY-COM-FRA. To be given sequentially for each fraction.

AVE-PRO-COM = average product composition of the key component. To be given sequentially for each fraction.

FRA-REF-RAT = reflux ratio for each fraction (overides the overall reflux ratio specification).

ADD-FEE = this command is used to signal whether additional feed is added after the fraction or not (default=0.0), to be sequentially for each fraction after first.

ADD-FEE-COM = composition of the additional feed. To be given sequentially for each component and for each fraction.

For Each Side Feed:

IND-FEE-PLA: index of the side feed plate.

PLA-FEE : side feed in moles.

PLA-FEE-COM : side feed composition in mole fraction, to be given sequentially for each component.

FOR EACH SIDE STREAM (PRODUCT)

IND-SID-STR:index of the side stream

SID-STR : side product in moles.

#### Calculation Related

LIG-KEY = initial light key component, required for SHO, MOD, and DES options.

HEA-KEY = initial heavy key component, required for SHO, MOD, and DES options.

NUM-COL-POI = number of collocation points required for the Reduced order model (RED) simulation (default=1)

#### Optimization Related

COS-IND = cost model index. if index equal to 1 then the following profit function is used.

Where C1 is the amortized incremental cost in $/sq.m/plate/y, C2 is the amortized incremental cost of tubular equipment in $/sq.m/yr, and C3 is the cost of steam or coolant to vaporize and condense respectively, per Kgmole of the distillate. For details please refer to Diwekar et al., 1989.

for cost index= 2:

where K1, K2, and K3 represent the cost coefficients and HRs represents the hours per year.

COS-VAL = cost variables required for the optimization and optimal control options. C1, C2, C3 (index=1) or K1, K2, K3 (index=2) values in the same sequence.

VEL-CON = velocity constants required for index=1 (values of Ga and Gb). Where Ga is allowable vapor velocity in kgmoles/hr and Gb is vapor handling capacity of tubular equipment in kgmoles/hr/sq.m.

SET-TIM = set up or shutdown time ts in hrs.

FEE-MAT-COS = cost of feed $/moles

BOT-MAT-COS = cost of bottoms $/moles

DES-VAR-MAT = decision variable matrix required for optimization. The options for variable reflux are: initial R/Rmin, final still composition or stopping criterion. Options for the constant reflux optimization are: initial R/Rmin, initial N/Nmin. For optimal reflux all the three options can be chosen. The initial value and upper and lower bound for these decision varibles can be specified though this keyword. The order is decision varible (1= selected, 0=not selected), upper bound, lower bound, and initial value.

PER-IND = performance index. Required for optimal reflux problems. 1, maximum distillate; 2, minimum time, and 3, maximum profit.

PER-SIZ= perturbation size for derivative calculations (default=0.001)

KUH-TUC-ERR = Kuhn-Tucker error tolerance (default=0.0001)

MAX-ITE = maximum number of iterations (default=20)

OPT-MET= optimization method. 1=feasible path; 2=infeasible path. (default=2)

For Each Fraction

PRO= Reflux profile equations for optimal reflux problems. 1, regular; 2, linear, 3, step; and 4, exponential.

COE-MAT = coefficients of the profile equation matrix. Upper, lower bounds and initial values.

PRO-VAL = product sales values in \$/kgmole, to be given sequentially for each fraction, required for OPT.

# Index

A

About 16

Add-ons 5

Arrange Icons 15

B

BDT File

New 8

Open 8

Save 8–9, 41–46, 48

Save As 9, 46

C

Calculate

Run Calculation 13

Show Errors 14, 41

Show Results 14, 41, 49

Calculate Menu 13

Calculation Related 26, 32, 48, 82

Cascade 15

CDF Plot 15

Clear All 11

Close All 15

Close Text File 9

Column Specification Commands 78, 81

Column Specifications 26, 77

Copy 11

Copy Plot to Clipboard 15

D

Data Bank 12, 19, 39, 77, 79

Delete 11

Design 1–3, 17, 18, 24–27, 34, 37–38, 39, 47–49, 60, 63, 80–81

Design Related 26–27, 35, 39, 49, 81

E

Edit

Clear All 11

copy 11

Delete 11

Paste 11

Undo 10–11

Edit Menu 10

Edit Side Feed 29, 31

Edit Side Product 29, 32

Estimation Related 26, 35

Example Files 59

Exit 10

F

File

Close Text File 9

Exit 10

New BDT File 8

New Text File 9

Open BDT File 8

Print Preview 10

Print Setup 10

Save BDT File 8–9, 41–46, 48

Save BDT File As 9, 46

Save Text File 9

Save Text File As 9

File menu 7

Find 12

G

Graphical input 3, 7–8, 7, 17

H

Hardware/Software Requirements 4

Help

About 16

Contents 16

Using Help 16

Help topic contents 16

K

Keyword input 3, 7, 10, 13, 77

M

Main Window 7, 15

Middle Vessel Sub-Policies 27

Model Options 26, 39, 47

N

New BDT File 8

New Text File 9

Next 12

O

Open BDT File 8

Open Text File 9

Operation Related 26, 28, 39, 47, 81

Optimization 2, 5, 17–18, 25–26, 32, 36–37, 39, 62, 63, 80, 83

Optimization Related 26, 32, 83

P

Parameters Menu 12, 18

Paste 11

Plot

CDF Plot 15

Copy Plot to Clipboard 15

Print Plot 15

View Simulation Plot 14, 42

Plot Menu 14

Policy Commands 77–78

Policy Options 12, 17, 27, 38, 47

Print 10, 15

Current Document 10

Preview 10

Setup 10

Print Plot 15

Print Preview 10

Print Setup 10

R

Reduced Order Model 26, 32, 61, 81, 83

References 63

Replace 12

Rigorous Model 26, 28, 37, 46, 59, 81

Run Calculation 13

S

Save BDT File 8–9, 41–46, 48

Save BDT File As 9, 46

Save Text File 9

Save Text File As 9

Search

Find 12

Next 12

Replace 12

Search Menu 11

Semi-Rigorous Model 26

Short Cut Model 26

Show Errors 14, 41

Show Results 14, 41, 49

Side Feed

Edit 29, 31

Side Product

Edit 29, 32

Simulation 2–3, 5, 12–14, 17, 25, 37–38, 39–42, 47–48, 59, 63, 77, 80, 83

Stochastic Analysis 13, 15, 23

System Requirements 4

T

Task Commands 77, 80

Task Specification 12, 41, 48

Text file

Close 9

Open 9

Save 9

Save As 9

Text File

New 9

Thermodynamic Option Commands 78–79

Thermodynamic Options 12, 17, 19, 38, 47, 77, 79

Thermo-Estimation 25

Tile 15

Tutorial 16, 37

U

Undo 10–11

Units 13, 23, 40, 78

Using Help 16

V

View Simulation Plot 14, 42

W

Window

Arrange Icons 15

Cascade 15

Close All 15

Tile 15

Window Menu 15