MultiBatchDS Tutorial

MultiBatchDS is a general-purpose package for the design, simulation and optimization of multicomponent batch distillation columns operating under different modes. This document is intended to show some of its features and applications by using two simple examples. It will be assumed that the user just started a session with MultiBatchDS and he/she is ready for the definition of the parameters required for the solution of the problem(s).

EXAMPLE 1

A mixture containing 60 moles of pentane and 40 moles of hexane is to be distilled in a batch distillation column. The column has 5 theoretical plates with a holdup of  2 moles per plate and a condenser holdup of  2 moles. The vapor boilup rate of the reboiler is 100 moles/hr.  Assume an initial total reflux condition. The column is operating under a constant reflux mode with reflux ratio equal to 1.0.  It can be assumed that the ideal model gives a good estimation for the fugacity of both the liquid phase and the vapor phase. Assume a feed temperature equal to 20 C and a constant pressure operation with the pressure equal to 1 atm.

a) Simulate a 1 hr operation of the column using MultiBatchDS without specifying the equilibration time.

b) Repeat a) but now set the value of the equilibration time to 1 hr.

c) Change slowly the plate holdup for the column specified in a) and find out the approximate switching point from the rigorous model to  the semirigorous model.

d) Simulate a 1 hr operation for the column specified in a) using the semirigorous model (zero holdup model)

SOLUTION

The solution to each part of this problem (as any other simulation or design in MultiBatchDS) can be divided in  three main tasks:

- To define the parameters of the problem

- To perform the calculations required for the solution.

- To analyze the results obtained from the simulation

Hence, here the attention will be focused in three of the buttons of the main menu of MultiBatchDS:

- the Parameters button

- The Calculate button.

- The Plot button

## STEP BY STEP PROCEDURE

Part (a)

## The Parameters Menu

In order to perform a simulation, sufficient information should be provided in each of three submenu options of the parameters menu:

- Policy Options

- Thermodynamic Options

- Task Specification

preferably in that specific order.  The numerical values and options used in this example will be in agreement with the statement of the problem given above.

Figure 1.  The Parameters menu

I.  POLICY SPECIFICATION

- Hit the Parameters button (See Figure 1).

- Select Policy Options. The Policy Options window shown in Figure 2. will appear. There are 3 possible policy options: Constant Reflux/Reboil, Variable Reflux/Reboil and Optimal Reflux/Reboil.

- Select Constant Reflux/Reboil

- Hit the OK button.

Figure 2.  Policy options window

II.  THERMODYNAMIC OPTIONS

- Hit the Parameters button.

- Select Thermodynamic Options. There are two main things that should be defined here: Thermo Settings and Fugacity Options. Thermo Settings is used to define the number of components and the relative volatility (for constant  models) or the Components Codes (for components appearing in the data bank).  Fugacity Options is ignored for the case of constant  models. There is also a small box in which the user can specify that energy balance calculations must be performed. Since in this example energy balance calculations are not going to be required, that box will be ignored.

- Under Thermo Settings select Component Codes.

- Define Number of Components equal to 2.

- Define the component 1 to be the component pentane.

- Define the component 2 to be the component hexane.

- Under Fugacity Options and Liquid Phase select ideal. If you do not define this option, the default is ideal.

- Under Fugacity Options and Vapor Phase select ideal.

- Hit the OK button.

In Figure 3, the Thermodynamic Options window with the data specified in steps 3 through 9 is shown.

Figure 3. Thermodynamic Options window

III.  TASK SPECIFICATIONS

- Hit the Parameters button.

- Select Task Specifications

- Define Number of Tasks to be 1. If you do not define this number, the default value of 1 is assumed. It can be a number in between 1 and 10. Figure 4 shows the result of this step.

- Hit the Task 1 button. The task 1 specification window presented in Figure 5 will appear.

- Under Tasks, select Simulation. Other options are Design, Optimization and Thermo Estimation.

- Under Model Options, select Rigorous. Shortcut, Semirigorous and Reduced Order Dynamic are also available.

- Figure 4.  Task Specification window

- Figure 5.  Specification of a specific task

- Under Column Specification, hit the Design Related button.

- Select Rectifier. As it can be seen, Stripper and Middle Vessel are also available.

- Define Reflux Ratio to be 1. Note that since Rectifier was already set, the only options now available are Reflux Ratio and Top Plates.

- Define Top Plates to be 5. You will get the specifications shown in Figure 6.

- Hit the OK button. You will return to the Task 1 specification window.

- Figure 6.  Design Related specifications

- Under Column Specification, hit the Operation Related button. The Operation Related window will appear. Note: In this section you will realize that the default units for pressure are atmospheres, Celsius degrees for temperature, hours for time and moles for flow.

- Define Initial Reflux to be Total.

- For this part DO NOT define Equilibration Time.

- Under Pressure, select Constant and define the value to be 1.

- Define Feed Temperature to be 20

- Define Feed to be 100.

- Define Feed Compositions for each component. Component 1 is 0.6 and component 2 is 0.4

- Assign a value of  2 to both the Condenser Holdup and the Plate Holdup.

- Select Vapor Rate and define a value of 100.

- Compare your data with that presented in figure 7.

- Define Number of Fractions to be 1.

- Figure 7.  Operation Related window

- Hit the Edit Fraction button. The Edit Fraction will appear. Note in Figure 8 that the user must select  one of the following Stopping Criterion

- Batch Time

- Final Bottom Composition

- Average Distillate Composition

- Total Distillate

- and the data corresponding to the Stopping Criterion selected above:

- Batch time

- Key Component Bottom

- Final Bottom Composition of the key component in the bottoms

- Key Component Distillate

- Average Distillate Composition of the key component in the distillate

- Final Distillate

- For instance, if the total Distillate is defined as Stopping Criterion, then the Final Distillate has to be given.

- The average distillate composition stopping criterion can not be used for variable reflux or reboil ratio operations.  On the other hand, the average distillate composition of the key component in the distillate for variable reflux operating mode, and average bottom composition of the key component in bottoms for variable reboil ratio operating mode, need to be specified.

- According to the problem statement, under Stopping Criterion select Batch Time.

- Assign the value of 1 to the Batch Time.

- Figure 8.  Edit Fraction window

- Hit OK button. You will return to the Operation Related window. Note that Side Feeds and Side Products are also available, but they are not used in this example. See the section of this tutorial corresponding to these optional features.

- Hit OK button. You will return to the Task 1 window.

- Hit OK button. You will return to the Task Specification window.

- Hit OK button. The definition of the problem is complete. Now you are ready to ask for the execution of the calculations. However, you may want to save all this information first. You can do that by choosing the Save BDT File As option under the button File in the main menu.

## The Calculate Menu

Once the parameters required for the simulation has been given, the user can proceed to perform the numerical calculations by using the Calculate Menu. See Figure 9.

Figure 9.  The Calculate menu

IV.  PERFORMING THE CALCULATIONS

- Hit the Calculate button.

- Select Run Calculations. A small window will appear showing the remaining portion of the simulation as shown in Figure 10.

- Figure 10.  Displaying the progress of a simulation

- Once the calculations have been performed, you will want to see the results. Hit the Calculate button.

- Select Show Results. A window as the one in Figure 11 displaying the data and the results will appear. Click in that window and examine the results.

- There is another option under the Calculate button. This option is Show Errors. MultiBatchDS will find out the cases on which you are not providing enough information for the solution of a problem. Error messages are saved in temporal files. You can access to this information by clicking the Show Errors button.

- Figure 11.  Numerical Result

## The Plot Menu

The result of the simulation can also be analyzed graphically. MultiBatchDS allows to plot several dynamic variables against each. other. This capability is provided under the Plot Menu presented in Figure 12.

Figure 12.  The Plot Menu

V.  PLOTTING RESULTS

- Hit the Plot button.

- Select View Simulation Plot.

- As it can be seen in Figure 13, several variables can be plotted against each other. Select your X and Y variable. Try several combinations.

- Figure 13.  The View Plot window

- Hit OK button.

The graph you asked for will appear in a separate window. Note that both the results window and any graph window can be closed at any time. This graph can be modified accordingly to the preferences of the user.  A typical graph and the tool bar provided for its modification is  shown in Figure 14.

Figure 14. Graphical Results

Note that, for this example, the number of task and the number of fraction are both 1 and, therefore, these values did not have to be modified in the Simulation Plot window.

Part b)

The procedure is the same as in part a) , the only difference is in III-15. For this case the Equilibration Time should be set to 1. Of course you can repeat all of the steps  for solving this part, but a much easier approach is to modify the BDT generated in part a) and to save this new file with a new name. Again, to do that use Save BDT File As under the File option in the main menu.

Part c)

Same procedure as in a), but the values for the Condenser Holdup and Plate Holdup (step III-19) will be less than 2. A Value of 0.01 works slowly, but it is near the solution.  If you reduce the holdup further, the rigorous model will not be able to integrate and you are forced to use the Semirigorous model.

Part d)

Same procedure as in part a), but in III-6 select Semirigorous. Semirigorous implies a zero holdup model. You can take the file generated in part a) and modify it. If you do that, you will find out that the values for the holdup defined in part a) are deactivated, since they do not make sense any more.

EXAMPLE 2

A mixture containing 40 percent A, 40 percent B and 20 percent C is to be distilled in a batch distillation column. Assume A = 1.5, B=1.25 and C=1.0.

a) Use MultiBatchDS to find the feasibility region for the variable reflux policy, if the column is operating in such a way that XD(A) remains constant at 0.9 throughout the operation. The operation is stopped when 40 percent of the original feed is distilled. Assume N=10 for the initial calculation.  Use the semirigorous model and then the shortcut model.  Selection of  initial light key and heavy key components is very important in the shortcut model.  Assume initial light key equal to 2 and initial heavy key equal 2.

b) Use MultiBatchDS to find the feasibility region for constant reflux condition if the average distillate composition of A is equal to 0.9. Use the shortcut model.

SOLUTION

The general procedure is the same as that in Example 1. The difference lies, of course, in different selection for Tasks, Model Options, specific values for the parameters, etc.

## STEP BY STEP PROCEDURE

Part a)

## The Parameters Menu

I.  POLICY SPECIFICATION

- Hit the Parameters button.

- Select Policy Options. There are 3 possible policy options: Constant Reflux/Reboil, Variable Reflux/Reboil and Optimal Reflux/Reboil.

- Select Variable Reflux/Reboil

- Hit the OK button.

II.  THERMODYNAMIC OPTIONS

- Hit the Parameters button.

- Select Thermodynamic Options.

- Under Thermo Settings select Relative Volatility.

- Define Number of Components equal to 3.

- Define a for the component 1 to be 1.5.

- Define a for the component 2 to be 1.25

- Define a for the component 3 to be 1.0

- Hit the OK button.

III.  TASK SPECIFICATIONS

- Hit the Parameters button.

- Select Task Specifications

- Define Number of Tasks to be 1.

- Hit the Task 1 button.

- Under Tasks, select Simulation.

- Under Model Options, select Semirigorous in the first part and for the second part use Shortcut.

- Under Column Specification, hit the Design Related button.

- Select Rectifier.

- Define Top Plates to be 10.

- Hit the OK button. You will return to the Task 1 window.

- Under Column Specification, hit the Operation Related button.

- Define Feed to be 100.

- Define Feed Compositions for each component. Component 1 is 0.4, component 2 is 0.4 and component 3 is 0.2

- Select Vapor Rate and define a value of  50.

- Define Number of Fractions to be 1.

- Hit the Edit Fraction button.

- Under Stopping Criterion, select Final Distillate.

- Assign the value of  40 to the Final Distillate.

- Assign the value of  0.9 to the Average Distillate Composition.

- Hit OK button. You will return to the previous window (Operation Related). If you are using Semirigorous model then go to step 26.

- Hit the Calculation Related button.

- Define Initial Light Key  to be 2.

- Define Initial Heavy Key to be 2.

- Hit OK button. You will return to the previous window (Operation Related).

- Hit OK button. You will return to the Task 1 window.

- Hit OK button. You will return to the Task Specification window.

- Hit OK button. The definition of the problem is complete. Now you are ready to ask for the execution of the calculations. However, you may want to save all this information first. You can do that by choosing the Save BDT File As option under the button File in the main menu.

## The Calculate Menu

IV.  PERFORMING THE CALCULATIONS

- Hit the Calculate button.

- Select Run Calculations. The error window will appear telling you to check your design specifications for both the Semirigorous model and the Shortcut model. See, for instance, the error presented in Figure 15.

- Figure 15.  Showing the errors in the input data

- Now take the Shortcut model. What you can do to solve the problem is to go to step III-5 and select Design instead of Simulation. Select Run Calculations again. A window will appear showing the remaining portion of the simulation. The Design option is iterative. Once the calculations have been performed, you will want to see the results. The Design option will give the feasible values for the Reflux Ratio and Number of plates.

- Hit the Calculate button.

- Select Show Results. A window displaying the data and the results will appear. Click in that window and examine the results.

Part b)

Repeat Part a) with the following modifications:

- I-3 is now  Constant Reflux/Reboil.

- III-18 Stopping Criterion is now Average Product Composition.

- Most of the numerical values for these parameters remain the same as in part a).  However, since we have learnt from the above exercise , the Number of  Plates needs to be higher, so let us try number of plates equal to 14.

- For Constant Reflux case, we also need to specify Reflux Ratio. Use Reflux Ratio equal to 1. Specify these parameters using Design Related menu (steps III-22 to III-24).

- Hit the Calculate button.  The column is again not feasible.

- For solving this part, it will be required a Design step similar to that in IV-3. To check what is wrong with this design, use the design option.  Here we need to re-specify the Stopping Criteria as  Final Distillate (steps III-18 and III-19) as Average Product Composition is now your specification.

MultiBatchDS Tutorial

Side Streams

A simulation involving continuous Side Feeds and/or continuous Side Products to the column is possible using MultiBatchDS. While defining the specifications of the simulation Task, the number of Side Feeds and the number of Side Products are defined in the Operation Related window. This is illustrated in Figure 1, where the Number of Side Feeds and the Number of Side Products have both been specified to be 1. These specification are both optional and independent from each other. In the simulation of a batch distillation column, both specifications, of anyone of them, or none of them may be given, according to the requirements of the user.

Figure 1. Definition of the Number of Side Streams

When these features are used, for each side feed and side product, the input can be supplied using the Edit Side Product window and Edit Side Feed window.

## DEFINITION OF SIDE PRODUCTS

For the specification of a Side Product the following steps should be taken:

- Define the Number of Side Products in the Operation Related window as explained above. For each of the Side Products:

- Select the number of the side product to be specified (if more than 1).

- Click in the button Edit Side Product.

- The Side Product window shown in Figure 2 will appear indicating the number of the side product being defined. Then, 3 parameters will have to be defined:

- The number of tray from which the side product will be extracted.

- The amount of the Side Product

- The vapor fraction of the side Product.

- Hit the OK button to return to the Operation Related window.

Figure 2.  Side Product Window

## DEFINITION OF SIDE FEEDS

The specification of side feeds is very similar to the definition of side products. For the specification of a Side Feed :

- Define the Number of Side Feeds in the Operation Related window as explained above. For each of the Side Feeds:

- Select the number of the side feed to be specified (if more than 1).

- Click in the button Edit Side Feed.

- The Side Feed window will appear indicating the number of the side product being defined.  See Figure 3.  Then, the following parameters have to be given:

- The number of tray to which the side is introduced.

- The amount of the Side Feed.

- If energy balance calculation are going to be performed, define the pressure and temperature of the side feed.  The user will not be able to define these two values if the Heat Balance option is not enabled.

- The condition of the feed. That is, if the feed is liquid or vapor.

- Finally, the composition of the side feed for each of the components.

- Hit the OK button to return to the Operation Related window.

Figure 3.  Side Feed Window

MultiBatchDS Tutorial

Multi-Fractional Operation

A batch distillation column can be operated in such a way that several fractions or cuts ca be obtained. MultiBatchDS allows the user to specify a maximum of 10 different fractions. The number of fractions can be specified in the Operation Related window:

- Select the number of the fraction that is going to be specified.

- Figure 1 Operation Related window

- In Figure 1, note that a number of fraction equal to 2 has been specified in the box corresponding to “Number of Fractions”.

- Hit the button “Edit Fraction”. The Edition Fraction screen will appear showing the number of the fraction being currently edited.

After the number of fractions has been defined, the input for each fraction must be specified by using the Edit Fraction window (Figure 2). For each fraction, the definition of the parameters will be basically the same as the definition in a single fraction simulation:

Figure 2. Edit Fraction window for fraction number 1

The user must select

- One of the following Stopping Criterion

- Batch Time

- Final Bottom Composition

- Average Distillate Composition

- Total Distillate

- The data corresponding to the Stopping Criterion selected above

- Batch time

- Key Component Bottom

- Final Bottom Composition of the key component in the bottoms

- Key Component Distillate

- Average Distillate Composition of the key component in the distillate

- Final Distillate

- For instance, if the total Distillate is defined as Stopping Criterion, then the Final Distillate has to be given.

- The average distillate composition stopping criterion can not be used for variable reflux or reboil ratio operations.  On the other hand, the average distillate composition of the key component in the distillate for variable reflux operating mode, and average bottom composition of the key component in bottoms for variable reboil ratio operating mode, need to be specified.

- Besides the stopping criterion related data, the user can specify a reflux ratio for each fraction. This specification is optional and, if given, it will override the overall reflux ratio specified in the design related variables.

- As an optional specification, MultiBatchDS lets the user define an additional feed introduced to the column after the current fraction. If this option is used, the flow and the composition of this additional feed have to be specified in the boxes provided.

- Recycling a waste cut/fraction (or fraction of this waste cut) is a common practice in batch distillation.  How much of the current fraction is to be recycled back to the column can be specified using the percent waste fraction recycled box provided in this window.  If this value is defined, then the number of the fraction after which this waste cut is recycled to the reboiler needs to be specified as well.

Only items 4 and 5 are specific to the Multi-fractional operation capability provided by MultiBatchDS. The rest of the specifications are the same already explained before in this tutorial document. Also, both of them are optional and may not be needed to perform a multi-fractional operation simulation.

After executing the calculations of a multifractional operated batch distillation column, the numerical results will be displayed separately for each fraction.. The information about the kind of simulation performed and the initial values for the compositions will separate the numerical results among the different fractions. See Figure 3.

In the same way, the graphical results are shown for the fraction specified by the user in the View Plot Simulation window. Note that the number fraction specified in Figure 4 is 2. Therefore, the simulation plots will be those corresponding to fraction number 2.

Figure 3. Header of the numerical result for each fraction

Figure 4.  View Simulation Plot Window

MultiBatchDS Tutorial

Heat Balance Calculation

One of the most remarkable features of MultiBatchDS is its capability to perform heat balance calculations during the simulation of a batch distillation column.

A very usual simplification while performing the simulation of a distillation column is the assumption of constant molar flows. That means that the molar amounts of the vapor going upstream and the liquid going downstream do not change from tray to tray along the column. For mixtures containing components with similar heat of vaporization, this is a very reasonable assumption.

MultiBatchDS allows the user removing the assumption of constant molar flow. As a consequence, energy balance calculations have to be performed in order to determine the values of the liquid and vapor flows in each tray as well as the temperature profiles along the distillation column. This is computationally expensive, but it is definitely required when the chemical species present in the mixture are different in nature. An obvious restriction for the execution of heat balance calculation in MultiBatchDS is the fact that the components present in the mixture must be “real” components, so that the physical properties needed in the calculation are available.

In this section, we will explain the steps that the user has to follow in order to use energy balance; we will focus, however, only in those steps which are particular to this kind of simulation.

## HEAT BALANCE SPECIFICATION

- Under the Parameters window, select Thermodynamic Options. The Thermodynamic Options window is used to specify that the simulation will be executed with energy balance calculations on it. Just click in the small box belonging to the option Heat Balance. The definitions of the components of the mixture and the thermodynamic model is the same as before. This is shown in Figure 1.

- The second step in the specification of energy balance is optional. Going further in the definition of the problem, follow the step already explained for the specification of the task:

- Under the Parameter menu, select Task

- Select the number of tasks of the simulation

- Hit the OK button

- Hit the Task 1 button

- Click in the Design Related button and define the parameters  of the problem corresponding to this window

- Hit the OK button.

- Figure 1. Heat Balance Specification in Thermodynamic Options

- Click the Operation Related button. In this window, shown in Figure 2, there is another important step in the specification of a energy balance calculation. In a heat balance calculation, the rate of the vapor going up from the reboiler can be selected in two possible ways. The first one is explicitly select the option Vapor Rate and define its value, as it has been done in all of the examples presented before in this tutorial. The second option the definition of the Heat Duty in the reboiler. The user may notice that there is a button Edit Heat Duty that will only be available if the Heat Duty option is selected. The use of this button is optional as it can be better to define explicitly the vapor rate, but it will be explained here to give an example of its application.

- Click the Heat Duty option.

- Click the Edit Heat Duty button. The window presented in the Figure 3 will appear.

- Figure 2. Example of Heat Duty specification

- Select the Heat Duty option and give the value of the heat duty of the reboiler in the corresponding box. An important point here is that, while performing energy balance calculations, MultiBatchDS assumes that the feed is at its bubble point temperature. Therefore, since the amount of the feed is know, by defining the heat duty in the reboiler, the vapor rate is implicitly defined.

- Hit the OK button to return to the Operation Related window.

- The rest of the specifications in the Operation Related window are done as usual.

## NUMERICAL RESULTS IN ENERGY BALANCE CALCULATION

- Besides the usual profiles of compositions against time, after performing energy balance calculations the profiles of temperature and flows of liquid and vapor along the column are available and reported. This is illustrated in Figure 4. It is obvious that these profiles are not required when the simplification of constant molar flow is assumed.

- Figure 3. Edit Heat Duty Window

Figure 4. Numerical results in energy balance calculations

## GRAPHICAL RESULTS IN ENERGY BALANCE CALCULATIONS

Since the numerical profiles of temperature and flows with respect to the tray number and time are available for energy balance calculations, MultiBatchDS will also allow the user to graphically analyze those results. To obtain the plots of temperature and flows of each of the trays against time, use the following steps:

- Under the Plot Menu, select Flow Rates. The Flow Rate (Figure 5) window will appear asking for the number of tray to be analyzed.

- Select the number of tray of your convenience. In the example shown in Figure 5, the tray number one is selected.

- Figure 5.  Selection of the Plate to be graphically analyzed

- After the number of tray is selected, the graphical profiles are displayed as in Figure 6.

Figure 6 Flow Rate profiles for a plate

MultiBatchDS Tutorial

Units Conversion

MultiBatchDS provides the user with the capability of using different unit systems for the main variables of a Batch Distillation calculation: temperature, pressure, area and flow.

To select among the different choices for each of the variables:

- Go to the Parameters menu and click in the option Units. The Units window will appear showing the alternatives for each of the variables (See Figure 1).

- Click in the options of your convenience.

- Click in the OK button.

In particular, for the case of Flow Units, the units to be used depend on the basis used for the calculation:

- Moles

- k moles

- lb moles

- Volume

- m3

- gallons

- liter

- ft3

- Weight

- kg

- lb

The defaults defined by MultiBatchDS are:

- Degrees Celsius for Temperature.

- Bars for Pressure.

- m2 for Area.

- moles for Flow.

Figure 1 The Units window

Internally, MultiBatchDS always performs its calculations in the default units, but the definition of the data and the graphical and numerical report of the results will be given in the units defined by the user.

The internal conversion of units for variables like temperature, pressure and area is straightforward.  On the other hand, the conversion of the units in the case of the flow becomes more complicated for a number of reasons.  First of all, the conversion of units for the flow is available only for “real” components in a mixture; that is because such a conversion requires the use of the molecular weight of each component. The second and main reason of the complexity appears while working with volume flows and compositions. In such a case, the calculation of the density of the components and the mixture for the vapor phase is required. A basic approximation to that problem is using the ideal gas law.