MultiBatchDS Tutorial

Heat Balance Calculation

One of the most remarkable features of MultiBatchDS is its capability to perform heat balance calculations during the simulation of a batch distillation column.

A very usual simplification while performing the simulation of a distillation column is the assumption of constant molar flows. That means that the molar amounts of the vapor going upstream and the liquid going downstream do not change from tray to tray along the column. For mixtures containing components with similar heat of vaporization, this is a very reasonable assumption.

MultiBatchDS allows the user removing the assumption of constant molar flow. As a consequence, energy balance calculations have to be performed in order to determine the values of the liquid and vapor flows in each tray as well as the temperature profiles along the distillation column. This is computationally expensive, but it is definitely required when the chemical species present in the mixture are different in nature. An obvious restriction for the execution of heat balance calculation in MultiBatchDS is the fact that the components present in the mixture must be “real” components, so that the physical properties needed in the calculation are available.

In this section, we will explain the steps that the user has to follow in order to use energy balance; we will focus, however, only in those steps which are particular to this kind of simulation.

## HEAT BALANCE SPECIFICATION

- Under the Parameters window, select Thermodynamic Options. The Thermodynamic Options window is used to specify that the simulation will be executed with energy balance calculations on it. Just click in the small box belonging to the option Heat Balance. The definitions of the components of the mixture and the thermodynamic model is the same as before. This is shown in Figure 1.

- The second step in the specification of energy balance is optional. Going further in the definition of the problem, follow the step already explained for the specification of the task:

- Under the Parameter menu, select Task

- Select the number of tasks of the simulation

- Hit the OK button

- Hit the Task 1 button

- Click in the Design Related button and define the parameters  of the problem corresponding to this window

- Hit the OK button.

- Figure 1. Heat Balance Specification in Thermodynamic Options

- Click the Operation Related button. In this window, shown in Figure 2, there is another important step in the specification of a energy balance calculation. In a heat balance calculation, the rate of the vapor going up from the reboiler can be selected in two possible ways. The first one is explicitly select the option Vapor Rate and define its value, as it has been done in all of the examples presented before in this tutorial. The second option the definition of the Heat Duty in the reboiler. The user may notice that there is a button Edit Heat Duty that will only be available if the Heat Duty option is selected. The use of this button is optional as it can be better to define explicitly the vapor rate, but it will be explained here to give an example of its application.

- Click the Heat Duty option.

- Click the Edit Heat Duty button. The window presented in the Figure 3 will appear.

- Figure 2. Example of Heat Duty specification

- Select the Heat Duty option and give the value of the heat duty of the reboiler in the corresponding box. An important point here is that, while performing energy balance calculations, MultiBatchDS assumes that the feed is at its bubble point temperature. Therefore, since the amount of the feed is know, by defining the heat duty in the reboiler, the vapor rate is implicitly defined.

- Hit the OK button to return to the Operation Related window.

- The rest of the specifications in the Operation Related window are done as usual.

## NUMERICAL RESULTS IN ENERGY BALANCE CALCULATION

- Besides the usual profiles of compositions against time, after performing energy balance calculations the profiles of temperature and flows of liquid and vapor along the column are available and reported. This is illustrated in Figure 4. It is obvious that these profiles are not required when the simplification of constant molar flow is assumed.

- Figure 3. Edit Heat Duty Window

Figure 4. Numerical results in energy balance calculations

## GRAPHICAL RESULTS IN ENERGY BALANCE CALCULATIONS

Since the numerical profiles of temperature and flows with respect to the tray number and time are available for energy balance calculations, MultiBatchDS will also allow the user to graphically analyze those results. To obtain the plots of temperature and flows of each of the trays against time, use the following steps:

- Under the Plot Menu, select Flow Rates. The Flow Rate (Figure 5) window will appear asking for the number of tray to be analyzed.

- Select the number of tray of your convenience. In the example shown in Figure 5, the tray number one is selected.

- Figure 5.  Selection of the Plate to be graphically analyzed

- After the number of tray is selected, the graphical profiles are displayed as in Figure 6.

Figure 6 Flow Rate profiles for a plate