# Data Entry Capability

This section illustrates yet another modeling capability of MultiBatchDS, the Data Entry button (In the Thermodynamic Options window). This capability is mainly designed for providing the user with the ability of entering the thermodynamic data for user defined components. There are 3 basic applications of the Data Entry button:

- Assume that you have a mixture containing components which are not included in the data bank provided by MultiBatchDS (CRANIUM data bank). If the user has available the thermodynamic data required for the calculations, MultiBatchDS still can be used. What it should be done is to provide MultiBatchDS with the values for the constants required for the calculation of vapor pressure (Antoine constant), normal boiling point temperature, liquid and vapor enthalpy constants (if heat balance calculations are used), liquid volume (if volume units used), and molecular weight. This can be done through the use of the Data Entry button.

- This capability can also be used to modify a property of a component included in the data bank. Note: Whatever value the user defines through this button overrides the values given by the data bank.

- The last application of this window is to provide binary interaction parameters when the thermodynamic model selected for the liquid phase is Wilson.

SUMMARY OF THE DATA REQUIRED by MultiBatchDS

- Antoine Constants (T is in degrees Celsius, vapor pressure in mm Hg and log10 is used)

- Liquid and Vapor Enthalpy constants (a, b, c, d, e). The constants are used in an expression like (T is temperature in Celsius):

H = a + b T + cT2 + dT3 + eT4

Where H is calculated in Kcal/kmol.

- Liquid molar volume. It is given in m3/kmol.

- Boiling Point temperature (in Celsius).

- Molecular Weight (kg/kmol).

- If Wilson method is used for the liquid phase, binary interaction parameters for the components of the mixture have to be given.

## Example 6

Assume that the mixture to be distillate contains Pentane and two components which are not included in the data bank provided by MultiBatchDS (CRANIUM data bank). Assume also that the user has the information required by the MultiBatchDS calculations. Use the Data Entry button to enter such information.

### Solution

For simplicity, rather than repeating the whole step by step procedure for a new example, this section will focus exclusively on the Data Entry button.

#### II  Thermodynamic Options

- Hit the Parameters button.

- Select Thermodynamic Options.

- Under Settings select Component Codes.

- Define Number of Components equal to 3.

- Define Pentane as the first component.

- Define USER 1 as the second component (at the end of the list).

- Define USER 2 as the third component (at the end of the list).

- Select the fugacity option for the Liquid Phase. The fugacity option for the vapor phase is ideal.

- Hit the Data Entry button. The Data Entry window appears.

- Select the component USER 1.

- Hit the Edit button. The Data Entry Component USER 1 window appears.

- Enter the data for the component USER 1 (Antoine constants and Liquid Molar Volume). The Binary interaction parameters are given only if Wilson method is used.

- Hit the OK button. The Data Entry window reappears.

- Select User Data. Enter the corresponding data for component USER 1 (molecular weight, normal boiling point temperature, and enthalpy constants).

- Hit the OK button. The Data Entry window reappears.

- Select the component USER 2.

- Repeat from 11 to 15 for the component USER 2.

- Hit the OK button. The Thermodynamic Options window reappears.

- Hit the OK button.