Cranium

## User’s Guide

Molecular Knowledge Systems, Inc. makes no claims or warranties with respect to the contents or accuracy of this publication, including any warranties of fitness or merchantability for a particular purpose. Information in this document is subject to change without notice.

The software described in this document is furnished under a license agreement or nondisclosure agreement. The software may be used or copied only in accordance with the terms of that agreement. It is against the law to copy the software except as specifically allowed in the license agreement or nondisclosure agreement.

© Copyright 1997 Molecular Knowledge Systems, Inc. All rights reserved. No part of this manual may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the written permission of Molecular Knowledge Systems, Inc.

Cranium is a trademark of Molecular Knowledge Systems, Inc. Molecular Knowledge Systems and Synapse are registered trademarks of Molecular Knowledge Systems, Inc.

Microsoft and MS-DOS are registered trademarks of Microsoft Corporation. Windows is a trademark of Microsoft Corporation.

April 1997 Edition

Chapter 1: Introduction

Welcome to the Cranium physical property estimation system from Molecular Knowledge Systems, Inc. Cranium is designed to quickly and easily provide you with high quality physical property estimates. We hope Cranium will dramatically reduce your time spent on physical property tasks.

If you are in a rush to start using Cranium and are familiar with Windows software read the System Concepts section below and skip to Chapter 3.

Installation

Cranium is distributed with an installation program which ensures the software is placed in the correct directory and the appropriate initialization files are updated.

Note: You can not copy Cranium or its knowledge bases directly to your computer from the floppy disks. You must use the installation program.

Follow the directions on the accompanying installation card to properly install Cranium.

Starting Cranium

Cranium currently runs under Windows 95 or Windows NT.

•	To start Cranium under Windows 95 or Windows NT

1.	Press the Start button.

Windows displays a menu of application groups.

2.	Point to the Programs menu item.

A hierarchical menu of program groups is displayed.

3.	Point to the Cranium menu item.

Another hierarchical menu is displayed.

4.	Choose the Cranium program.

Cranium initially displays a welcome screen followed by the Open File dialog. You can choose an existing knowledge base from the dialog by selecting its file name with the mouse and pressing the Open button.

System Concepts

Cranium collects information on elements, molecular substructures, materials, mixtures, estimation techniques, and references into an electronic reference book we call a knowledge base. A knowledge base is like a database in that it does contain data and information on a set of entities. But in addition to data, Cranium knowledge bases contain estimation techniques and applicability rules enables them to create new information and new data.

The analogy to a reference book is carried through the user interface. Knowledge base windows are divided into chapters. Each chapter contains a number of pages. Information for a single material, mixture, technique, etc. is presented on each page. Finding information in Cranium therefore typically involves “changing” chapters and “turning” pages.

Figure 1 shows a knowledge base window.

Figure 1: Layout of a knowledge base window

Each window is divided into several “panes”. The chapter pane, located at the window’s top, displays tabs representing each chapter contained in the current knowledge base. Pressing the left mouse button on any tab activates that chapter displaying information from one of its pages in the window.

Immediately below the chapter pane is the window’s identifier pane. This displays the identifier, typically a name, acronym, or abbreviation, which Cranium uses to find the object.

The next pane is the data pane. This large, central window displays the bulk of data for each object. The data pane is typically organized into sections with bold headings describing the presented information.

Below the data pane is the control pane. The control pane displays buttons for navigating to the next or previous page as well as buttons for performing specific operations on the current object.

The status pane is the last pane at the bottom of the window. It presents the current page number as well as the total number of pages contained in that section.

The Introduction chapter has a slightly different interface than other chapters. This is because the Introduction chapter contains only a single page. The identifier, control, and status panes are not needed and not displayed. Figure 2 shows the Introduction chapter’s different layout.

Figure 2: Layout of the Introduction chapter’s panes

Nomenclature

The following list contains those expressions used in this guide which are unique to Cranium:.

Knowledge Base: a collection of data, rules, and procedures stored in computer executable format. We differentiate a knowledge from a data base because of the former’s ability to extend and reason about its contents.

Chapter: a collection of objects of the same class. Cranium contains seven chapters: Introduction, Elements, Groups, Materials, Mixtures, Techniques, and References.

Knowledge Base Window: a top-level window displaying the information contained in a knowledge base.

Pane: a child window of a knowledge base window.

Field: a visual grouping of labels and data entry boxes used to present information to the user.

Data Entry Box: a rectangular visual control used to display knowledge base information. Data entry boxes are always display blue text on a white background. Pressing the left mouse button anywhere within a data entry box will activate the associated dialog used for editing data.

This User’s Guide

The remaining chapters document Cranium’s operation in various detail. Chapter 2 provides a quick tour of Cranium. Chapter 3 provides details for some of the typical tasks Cranium can assist with. Chapters 4 through 10 are detailed references for each of Cranium’s object classes. Chapter 11 documents Cranium’s menu commands and auxiliary dialogs. Finally Appendix A provides a conceptual background of physical property estimation.

Chapter 2: Tour

This chapter provides an overview of Cranium’s user interface and capabilities.

Note: Any changes made to a knowledge base will not be saved until you choose the Save command from the File menu.

Starting Cranium

•	To start Cranium under Windows 95 or Windows NT

1.	Press the Start button.

Windows displays a menu of application groups.

2.	Point to the Programs menu item.

A hierarchical menu of program groups is displayed.

3.	Point to the Cranium menu item.

Another hierarchical menu is displayed.

4.	Choose the Cranium program.

Cranium displays a welcome screen followed by the Open file dialog.

5.	Select a knowledge base, denoted by a file with a .crn extension, and press the Open button. The chosen knowledge base is opened and displayed on the screen (see Figure 3).

Figure 3: Newly opened knowledge base

Navigation

Cranium manages information on elements, molecular substructures, materials, mixtures, estimation techniques, and references. Objects of each class are collected into “chapters”. The tabs at the top of each knowledge base window correspond to each of these chapters.

To change to the Elements chapter

1.	Move the mouse over the Elements tab at the window’s top and press the left mouse button.

Cranium displays the one page of the Elements chapter. Figure 4 shows an example page displaying information on the element Carbon.

Figure 4: One page of the Elements chapter

Each page of a chapter displays information on a single object. In our example page above, information on the element Carbon is displayed. Since more information is present than is visible in the window a scroll bar is activated at the window’s right side. Manipulating the scroll bar enables you to view the full page of information on the element.

Since each object is displayed on a separate page in a chapter, navigating through Cranium primarily involves changing chapters and turning pages.

To view elements on other pages

1.	Press the Next button located on the right side of the window’s control pane or press the right facing, hollow arrow located in Cranium’s toolbar or choose Next from the Navigation menu or press the ctrl-F accelerator keystroke. The ctrl-F keystroke is generated by holding down the control key and then pressing the F key.

All four approaches are equivalent resulting in the display of the next element stored in the knowledge base. Navigating to the previous element is done in the same manner by either pressing the Prev button, pressing the left facing, hollow arrow, choosing Prev from the Navigation menu, or pressing the ctrl-B accelerator keystroke.

Adding New Pages

One of the advantages an electronic reference book has over a printed version is the ease with which you can add new information.

To add a new element

1.	Press the Add toolbar button (the large plus sign). Alternatively you can choose the Add command from the Edit menu or press the alt-A keyboard accelerator.

Cranium adds a new, empty element after the current element.

Editing Data

Consider the various types of information Cranium manages: text values for material names and element symbols; numerical values for the atomic weight and normal boiling point; lists of atoms and bonds for each molecular structure; compiled code for each estimation technique. In addition whereas the Gibbs energy of formation is either a positive or negative real number, an element’s atomic number must be a positive integer, and a material’s boiling point must be greater than 0K.

One of Cranium’s most powerful yet subtle features is its tailored interface for data entry. For data entry into most fields, Cranium displays a customized dialog which not only facilitates input but also checks the entered data for completeness and correctness.

Consider the dialogs used to add data to our newly created element.

To enter a name for our new element

1.	Click the left mouse button in the identifier’s field data entry box.

Cranium displays a simple text entry dialog.

2.	Type in a name for our new element. and press the OK button.

Cranium now displays the name in the element’s identifier field.

To enter the atomic weight for our new element

1.	Click the left mouse button in the data entry box following the Atomic Number label. Cranium displays the Atomic Number dialog:

This dialog is one example of the tailored entry interface discussed previously. In addition to the value for the element’s atomic number, the dialog queries for the reference from which the value was obtained. The List button to the right of the reference entry displays those references currently present in the knowledge base.

In addition to tailored entry dialogs, Cranium checks all data entry for correctness and completeness.

2.	Enter a value of -1 into the dialog and press OK. Cranium checks the entered value against its programmed constraints and signals an error if a violation occurs. In this case Cranium requires the entered atomic number to be a positive integer and displays an error message.

3.	Reenter a 100 for the value and Smith90 for the reference. Press the OK button.

These values are acceptable and Cranium displays the entry in the Atomic Number field.

To enter the element’s electron configuration

1.	Click the left mouse button in the data entry box following the Electrons label. Cranium displays the Electron Configuration dialog.

2.	Enter values for the configuration and press the OK button. Cranium checks for positive values with no gaps between entries.

These three examples were used to demonstrate the variety tailored dialogs provides to facilitate accurate data entry. Chapters 4 through 10 discuss each of these dialogs in greater detail.

Removing Pages

To remove the current element

1.	Press the Remove toolbar button (the large minus sign). Alternatively you can choose the Remove command from the Edit menu or press the alt-R keyboard accelerator.

Cranium prompts you for confirmation and then removes the current element. The next element in the knowledge base is then displayed.

Note: There is currently no Undo function for the Remove operation so use it carefully.

Identifying Substructures

Identifying substructures is an essential step in the application of many physical property estimation techniques. Cranium uses a network matching algorithm to identifying groups within molecular structures. The Groups chapter contains the substructures used by all the knowledge base’s estimation techniques. It also contains an interface for evaluating the substructure matching algorithm.

Change to the Groups chapter

1.	Move the mouse over the Groups tab at the window’s top and press the left mouse button.

Cranium displays a page of the Groups chapter. Figure 5 shows an example page displaying information on the methyl group.

Figure 5: The Groups chapter displaying the methyl group page

To find the current group in a molecular structure

1.	Use the Next and Prev control pane buttons to page to the group you wish to locate with a structure. We will use the methyl group shown in Figure 5.

2.	Press the Occurrences button.

Cranium activates the Substructure Occurrences Dialog.

3.	Press the Choose Structure button to select a structure for examination. In our example we chose o-Xylene.

Cranium redraws the matching methyl groups in blue and reports the number of occurrences.

Working with Materials

Cranium manages over 25 physical properties and attributes for pure component materials. Cranium also provides estimation techniques for the majority of these properties.

To view a material

1.	Change to the Materials chapter. Click the left mouse button on the Materials chapter tab.

The Materials chapter is organized in the standard window layout. The material’s identifier, its name, is located near the top of the window. The data pane contains the fields displaying the material’s physical properties and attributes. In addition to the standard Prev and Next buttons, the window’s control pane contains a Compute button which, when pressed, estimates all of the current material’s properties.

Scroll the data pane downward to the Constant Properties section

Figure 6: The Materials chapter’s Constant Properties section

The fields in the Constant Properties section are representative of most of the Materials chapter’s fields. Each field consists of a label, a data entry box, and estimates display box, and a units entry box. In the Constant Properties section these items are aligned in columns.

To estimate a material’s properties

1.	In the Materials chapter go to the page displaying the desired material.

2.	Press the Compute button located in the center of the window’s control pane.

Cranium displays the Estimating Properties dialog which details the progress of the estimation. When the estimation is complete Cranium displays the Estimation Details dialog. This dialog is used to investigate which estimation technique was used to estimate which property and also provides details about why particular techniques failed. The operation of this dialog is detailed in Chapter 7.

3.	Exit the Estimation Details dialog by pressing the Done button.

Cranium displays the material’s estimated properties in the second column of display boxes. This side by side display makes it easy to compare estimates with data.

Editing Molecular Structure

Many estimation techniques require knowledge of a material’s structure. Some of these techniques require only information about the material’s chemical family, e.g., is the material chlorinated. Other techniques, e.g.,  group contribution techniques, require a material’s structure to be completely dissected in molecular fragments.

Cranium uses a simple connectivity model to represent molecular structure. Atoms and bonds are connected into a simple graph. Cranium does not currently support stereochemical bonds nor do the majority of estimation techniques use such information.

To enter a new molecular structure

1.	First create a new material. While in the Materials chapter press the Add toolbar button (the large plus sign).

Cranium creates a new material page and adds it to the chapter.

2.	Scroll the data pane to the Structure field located near the top of the pane.

3.	Press the left mouse button anywhere within the large data entry box following the Structure label.

Cranium displays its molecular structure editor.

The editor is designed to be analogous to standard drawing software packages. The large central window is the “canvas” on which you draw structures. The tools palette on the dialog’s left side is used to change “drawing” modes. The controls at the dialog’s top enable you to specify the current element, the current bond, and editing options.

To add atoms to our new structure

1.	Press the tools palette’s Atom button (the large A).

Cranium now adds an atom wherever you click the mouse in the canvas pane.

2.	Move the mouse onto the canvas pane and click the left mouse button several times in several different locations.

Cranium creates and adds several new atoms.

Figure 7: Adding atoms to a new structure

Note: Cranium does not allow you to add a new atom on top of an existing one.

To connect atoms by bonds

1.	Press the tools palette’s Bond button (the diagonal line).

Cranium now creates bonds between two atoms.

2.	Click the left mouse button on the first atom to be bonded.

Cranium redraws the atom in green and surrounds it with four small rectangles indicating it is the first bonding atom.

3.	Click the left mouse button on the second atom to be bonded.

Cranium creates a bond, of the currently selected bond type, and attaches it to this atom and the first bonding atom.

4.	Continue the process, selecting pairs of atoms to bond, until all atoms are bonded.

5.	You have now completed editing your new structure. Press the OK button.

Cranium adds the structure to our new material and displays it in the structure field.

You can now press the Compute button and have Cranium generate estimates for the new structure.

Chapter 3: Example Tasks

This chapter presents a number of example tasks which Cranium can assist with. We hope some of these tasks match those you need to accomplish so you can follow the same procedures modifying only the data specific to your problem.

Finding a Material in the Knowledge Base

Materials can be found by name, synonym, or chemical formula.

To find MEK in the Learning Organics KB

1.	Open the Learning Organics KB.

Choose the Open command from the File menu. Cranium displays the Open dialog. Select the learnorg.crn file and press the Open button.

2.	Change to the Materials chapter. Press the left mouse button on the Materials chapter tab.

3.	Choose the Goto command from the Navigation menu.

Cranium displays the Go To dialog listing the name of each material contained in the knowledge base.

Because MEK is a synonym it is not listed.

4.	Check the Synonyms option. Cranium updates the list to display names and synonyms. The entries are sorted alphabetically.

5.	Scroll to the entry for MEK, select the entry and press the OK button.

Cranium changes to the page displaying information for methyl ethyl ketone, MEK’s full name.

Estimating Constant Properties

Estimating constant properties can be accomplished in a few simple steps.

To estimate the properties of methyl ethyl ketone

1.	Open the Learning Organics KB.

Choose the Open command from the File menu. Cranium displays the Open dialog. Select the learnorg.crn file and press the Open button.

2.	Change to the Materials chapter and find methyl ethyl ketone. (See the previous section for an example of finding a material.)

3.	Press the Compute button located in center of the window’s control pane.

Cranium documents the estimation of each property and displays the Estimation Details dialog when finished.

4.	Press the Done button to exit the Estimation Details dialog.

5.	Scroll to the Constant Properties section of the window’s data pane.

Estimating T-Dependent Properties

Cranium only estimates the properties of the currently selected temperature dependent property.

To estimate the vapor pressure of methyl ethyl ketone

1.	Open the Learning Organics KB.

Choose the Open command from the File menu. Cranium displays the Open dialog. Select the learnorg.crn file and press the Open button.

2.	Change to the Materials chapter and find methyl ethyl ketone. (See the previous section for an example of finding a material.)

3.	Scroll down to the data pane’s Temperature Dependent section.

4.	Click the left mouse button anywhere in the data entry box following the Property label.

Cranium displays a dialog listing temperature dependent properties.

5.	Select the vapor pressure entry and press the OK button.

You must now specify the temperatures for which you want estimates generated.

6.	Click the left mouse button in either the Temp or Data columns of the temperature dependent display table. Click in the white, grid area of the table not in the column headings or row labels.

Cranium displays the Temperature Dependent Property Dialog.

7.	Click the left mouse button in any row and press the Edit button.

Cranium displays the Temperature Data Reference dialog.

8.	Enter a value for the temperature and press the OK button. Repeat the procedure entering temperatures for several other rows.

9.	Once you have entered several temperatures press the OK button.

Cranium displays the entered temperatures.

10.	Press the Compute button located in center of the window’s control pane.

Cranium documents the estimation of each property, including the vapor pressure, and displays the Estimation Details dialog when finished.

11.	Press the Done button to exit the Estimation Details dialog.

Cranium displays the generated estimates in the table’s rightmost column.

Importing Mol Files

To take advantage of the large number of existing molecular structure libraries, Cranium’s structure editor enables you to import Mol files.

To import the steroid.mol Mol file

1.	Open the Learning Organics KB.

Choose the Open command from the File menu. Cranium displays the Open dialog. Select the learnorg.crn file and press the Open button.

2.	Change to the Materials chapter and add a new material.

3.	Scroll to the data pane’s structure field and click the left mouse button on the large data entry box below the Structure label.

Cranium displays its molecular structure editor.

4.	Press the tools palette’s menu button (the button with a rectangle enclosing several horizontal lines).

Cranium displays a menu of commands.

5.	Choose the Import Structure command from the menu.

Cranium activates the Open file dialog.

6.	Open the MolFiles directory and select the steroid.mol file. Press the Open button.

Cranium imports the structure into the editor.

7.	Press the OK button.

Cranium displays the imported structure in the data pane’s Structure field.

Note: You can now press the Compute button to generate estimated properties for the imported steroid.