MultiBatchDS Tutorial

Multi-Fractional Operation

A batch distillation column can be operated in such a way that several fractions or cuts ca be obtained. MultiBatchDS allows the user to specify a maximum of 10 different fractions. The number of fractions can be specified in the Operation Related window:

- Select the number of the fraction that is going to be specified.

- Figure 1 Operation Related window

- In Figure 1, note that a number of fraction equal to 2 has been specified in the box corresponding to “Number of Fractions”.

- Hit the button “Edit Fraction”. The Edition Fraction screen will appear showing the number of the fraction being currently edited.

After the number of fractions has been defined, the input for each fraction must be specified by using the Edit Fraction window (Figure 2). For each fraction, the definition of the parameters will be basically the same as the definition in a single fraction simulation:

Figure 2. Edit Fraction window for fraction number 1

The user must select

- One of the following Stopping Criterion

- Batch Time

- Final Bottom Composition

- Average Distillate Composition

- Total Distillate

- The data corresponding to the Stopping Criterion selected above

- Batch time

- Key Component Bottom

- Final Bottom Composition of the key component in the bottoms

- Key Component Distillate

- Average Distillate Composition of the key component in the distillate

- Final Distillate

- For instance, if the total Distillate is defined as Stopping Criterion, then the Final Distillate has to be given.

- The average distillate composition stopping criterion can not be used for variable reflux or reboil ratio operations.  On the other hand, the average distillate composition of the key component in the distillate for variable reflux operating mode, and average bottom composition of the key component in bottoms for variable reboil ratio operating mode, need to be specified.

- Besides the stopping criterion related data, the user can specify a reflux ratio for each fraction. This specification is optional and, if given, it will override the overall reflux ratio specified in the design related variables.

- As an optional specification, MultiBatchDS lets the user define an additional feed introduced to the column after the current fraction. If this option is used, the flow and the composition of this additional feed have to be specified in the boxes provided.

- Recycling a waste cut/fraction (or fraction of this waste cut) is a common practice in batch distillation.  How much of the current fraction is to be recycled back to the column can be specified using the percent waste fraction recycled box provided in this window.  If this value is defined, then the number of the fraction after which this waste cut is recycled to the reboiler needs to be specified as well.

Only items 4 and 5 are specific to the Multi-fractional operation capability provided by MultiBatchDS. The rest of the specifications are the same already explained before in this tutorial document. Also, both of them are optional and may not be needed to perform a multi-fractional operation simulation.

After executing the calculations of a multifractional operated batch distillation column, the numerical results will be displayed separately for each fraction.. The information about the kind of simulation performed and the initial values for the compositions will separate the numerical results among the different fractions. See Figure 3.

In the same way, the graphical results are shown for the fraction specified by the user in the View Plot Simulation window. Note that the number fraction specified in Figure 4 is 2. Therefore, the simulation plots will be those corresponding to fraction number 2.

Figure 3. Header of the numerical result for each fraction

Figure 4.  View Simulation Plot Window