MultiBatchDS Tutorial

Units Conversion

MultiBatchDS provides the user with the capability of using different unit systems for the main variables of a Batch Distillation calculation: temperature, pressure, area and flow.

To select among the different choices for each of the variables:

- Go to the Parameters menu and click in the option Units. The Units window will appear showing the alternatives for each of the variables (See Figure 1).

                - Click in the options of your convenience.

                - Click in the OK button.

In particular, for the case of Flow Units, the units to be used depend on the basis used for the calculation:

                - Moles

                - k moles

                - lb moles

                - Volume

                - m3

                - gallons

                - liter

                - ft3

                - Weight

                - kg

                - lb

The defaults defined by MultiBatchDS are:

                - Degrees Celsius for Temperature.

                - Bars for Pressure.

                - m2 for Area.

                - moles for Flow.

Figure 1 The Units window

Internally, MultiBatchDS always performs its calculations in the default units, but the definition of the data and the graphical and numerical report of the results will be given in the units defined by the user.

The internal conversion of units for variables like temperature, pressure and area is straightforward.  On the other hand, the conversion of the units in the case of the flow becomes more complicated for a number of reasons.  First of all, the conversion of units for the flow is available only for “real” components in a mixture; that is because such a conversion requires the use of the molecular weight of each component. The second and main reason of the complexity appears while working with volume flows and compositions. In such a case, the calculation of the density of the components and the mixture for the vapor phase is required. A basic approximation to that problem is using the ideal gas law.