# Chapter 11. Accuracies

## Simulations

In Chapter 11 the accuracy with which the β’s of the model equation and the E{μ}s of each data unit are estimated is determined by simulation. The basic idea is that the originally fitted values are taken to be a set of true E{μ}’s of an imagined populations and the estimates  E2{μ}/b are taken as their true V{μ}’s. Using these, for each data element (road segment or intersection)  a μ is randomly generated from a Gamma distribution. These μ’s then used to generate accident counts from a Poisson distribution. Using the new set of accident counts Solver obtains new parameter estimates and new fitted values. Repeating such simulation runs several times, sample variances of both parameters and fitted values interest can be computed.

This problem serves to elucidate the tasks of randomly generating μ’s and accident counts.

### Generate μ’s from a Gamma PDF

To be specific, the task here is to generate 10 random μ’s from a gamma distribution with E{μ}=5 accidents and V{μ}=9 accidents2. The so-called ‘Inverse Transformation Method’ will be used. To explain, Figure 1 is a screen capture of the ‘Mu’s from Gamma.xls’ file.

Figure 1

The a and b in D1:D2 were computed from the E{μ} and V{μ} in B1:B2 and then used as parameters in the GAMMADIST function in B5:B25. The corresponding Gamma distribution is in Figure 2.

Figure 2

Thus P(5<μ≤10)=0.932-0.580. Let U be a number from a uniform distribution on 0 to 1. If 0.580<U≤0.932 this corresponds to the μ from a Gamma distribution with E{μ}=5 and V{μ}=9 being between 5 and 10. This would obtain if when U=0.580 one would say the μ=5, when U= 0.932 one would say that μ=10, etc.

All that remains is to find a way of determining what μ from a Gamma distribution corresponds to what U. The Excel function GAMMAINV does just that. Thus, as shown in Figure 3, when U is, say, 0.282 then with the same a and b as before, the GAMMAINV returns the value 3.00.

Figure 3

### Generate accident counts from the Poisson distribution

As explained at the beginning of this problem, once a μ has been generated for each data unit, the next task is to generate a Poisson accident count for each such μ. The Inverse Transformation Method cannot be used for discrete random variables.  One way to generate a Poisson random variable with μ as mean is to generate independent uniform random variables U and keep multiplying them till, after the Nth multiplication the product is for the first time less than e-μ; the value of the Poisson random variable is N-1. A Visual Basic for Applications function for the job is in Figure 4.

Figure 4

Use the 10 μ’s generated in ‘a’ and, for each, generate a Poisson accident count.

## Parameter and Fitted value accuracies with and without ‘Area’ variable.

The NB fits for the Edmonton signalized intersection data without and with ‘Area’ as a variable are in the ‘ Edmonton All, Urban, and Suburban for Chapter 11.xls’ file.

The task is to estimate the accuracy of the model equation parameters and of the fitted values.

To do that replace the C-F spreadsheet in the ‘Accuracy Simulation.xls’ file by the Edmonton C-F spreadsheets adapted the VBA code to this data.