Rudder Roll Stabilization with NGMV control – MATLAB files.

- Change the extension of the ship.doc file to .zip

- Extract its contents, i.e. the folder RRS

- NGMV Toolbox and RS Toolbox (for the LQG controller) are needed

In Matlab:

- Set the current directory to \RRS

- Type

>>rrs

in the command window. This loads ship model parameters from rrs_load file, and opens the Simulink model and the RRS Design GUI (the GUI can be also opened independently by typing rrstool).

- Design GUI. Used to design PID, MPC, GMV and LQG controllers for the RRS problem. The initial screen looks like this:

- Menu File can be used to load any saved models and settings or to save the current session. Load the rrs1511 session by selecting File -> Load Session…

- Change the controller to LQG, uncheck the Rate feedback checkbox and press UPDATE button to redesign the controller. The front panel should now look like this:

It is possible to add and remove poles and zeros of the weightings (all transfers are in continuous time). Pressing the UPDATE button updates the plots accordingly. Also, the calculated controller parameters are copied to MATLAB workspace for use in Simulink model.

- Menu Ship invokes an editor for viewing and modifying ship components:

- After designing the controller, you can simulate the system using the Simulink model. In the model, double-click the LQG button to connect the LQG controller. The ‘none’ button corresponds to no rudder-roll feedback but with the fin controller in the local loop. After the simulation is finished, you can plot the results by pressing Time plots/One button, as shown below.

The open-loop and closed-loop variance of roll angle is printed in the command window, together with the controller roll reduction ratio.

The controllers can also be designed from by double-clicking the Design! button in the model. This button runs the rrs_ctrl script and the controller design parameters can be changed in that script.

- Rate feedback. I assume that only PID and MPC (as well as the fin controller) use it. The GMV and LQG are designed assuming roll angle measurement. This has to be remembered in the GUI by accordingly checking and unchecking the checkbox.

- Wave model. The original model didn’t include the differentiator but I added it to better reflect the wave spectrum. It can be seen and plotted in the GUI -> Ship menu by pressing the Diagram/Responses toggle button (yellow plot):

The actual disturbance model used for the controller design is obtained by filtering this model through the roll closed-loop transfer (incl. the fin controller). That corresponds to the ‘output disturbance’.

- Fin stabilization. The roll reduction achieved by using fins can be seen by pressing the PLOT button under the fin controller transfer function (BTW I added a pole there to make the system proper – otherwise Simulink won’t run).
Figure 7 in paper actually shows the sensitivity from the waves to e - shown in black below. The ‘true’ sensitivity to roll is different and shown in red (higher frequencies attenuated)

- GMV design. I suggest you arrange the windows as follows:

The design procedure is more or less as follows:

- design the weights by adding/removing poles and zeroes to the error and control weightings

- Press UPDATE button to update the plots. The ‘Stable?’ and max. eigenvalue textboxes are also updated

- When satisfied, run the Simulink model and watch the Roll angle scope – any problems will be apparent there

- You can stop the simulation at any time and plot the results by pressing Time plots -> One button on the Simulink model. The roll reduction ratio will be displayed in the figure, as well as in the command window

You can load the above example from File -> Load… -> rrs_at_least_it_is_stable.mat

- To simplify things, I’m using only discrete-time models in Simulink, although the design (weightings) is done in continuous-time domain, and then discretized. The LQG controller is actually continuous but I’m using its discretized version in the simulation.