README

- Include folders (Data, Planta,  Images and folders with MPC examples) in the Matlab path.

- Edit  the script file “Main” to set the controller parameters.

- Open the Simulink file corresponding to the microgrid, there are three Simulink files corresponding to different scenarios: a sunny day, a cloudy day and a windy day.

- Run the selected Simulink file

Observations:

- To modify the weights and matrix of the model used by the MPC, modify the "Main" file  as appropriate (the code is commented, each part is easily identified). You can modify the matrix of the state space  model depending on the sampling times you want to use.

- To play the sunny day we select the file 'P_sunny.mat' in the Solar-Power block. You can also scale this curve by means of a gain that is in the same block designated by 'Sun Multiplicator'. The format of the file must be in timeseries.

- To play the cloudy day we select "in the Solar-Power block the file 'Pcloudy.mat'. In this case, it is not necessary to scale the generation curve for any gain, although if you wanted it you could do.

- To reproduce the day of moderate wind we select in the Wind-Power block the file 'WindPower.mat'. You can also scale this curve by means of a gain that is in the same block designated by 'Wind Multiplicator'.

- In a recent version of the Simulink model of the Microrred, the demand block has been modified, as well as the generation block, in which you can select the demand data file that you want to implement that is in the Data folder / demand.

- The user manual explains how to create the generation and / or demand file for new data.

- With the "graphic" script, you can make two graphic representations of all the powers of the equipment, the demand and the generation. Like another graph with the state of charge of the equipment.