API README for FEMAP Version 7.1

© 1985-1999 Enterprise Software Products, Inc.

API scripts can be accessed in two ways, by running the script through the program file commands or by creating a user defined menu that is placed as a new entry on the menu bar and references the script.

User defined Menu's

User defined menus are defined in the menu.esp file.  This file contains the structure of the new menus and the full path ( i.e. c:\Femap71\Api\Splittriangles.bas) to the script files that are used to perform user created functions.  Once the menu.esp file is created, place it in the FEMAP directory and restart FEMAP.  The contents of the file are described in the API_REF.doc located in the FEMAP directory.  An example menu.esp file is located in the API sub directory.

Groupsolids.bas

This script brings up the standard entity selection box that allows you to pick multiple solids and automatically builds a new group with associated surfaces, curves, and points.

Grpfreend.bas

This script is intended to help with fixing meshes that came from bad geometry.

First a free edge plot is created and then FEMAP prompts you to pick the nodes on the free edge (select Methods and then select Free Edge) then graphically pick the free edges on the screen.  This will create a new group that contains nodes and elements directly in the area of the free edge

Splittriangles.bas

This script is intended to refine surface triangles. Choose surface triangles that you wish to split.  FEMAP will then split the original triangle into 4 smaller triangles, but will not merge coincident nodes formed by the creation of new triangles.  Check your mesh carefully for connectivity.

Surffromels.bas

This script creates surface patches from any selected existing triangles or quads.  These surface patches can then be stitched using Geometry-Solid-Stitch, then used for further modeling purposes.   Note the geometric facets of the geometry can only be as fine as the original surface mesh.

Masselement.bas

This script creates multiple point mass elements at a single time using the standard entity selection dialog box.  You must define a Mass property prior to running the command.