# PART I - GENERAL

1.01	SCOPE

A.	This section includes providing all materials and labor required for installation of all posts, braces, rails, chain link fabric and gates indicated on the drawings and as here specified.

  - RELATED WORK SPECIFIED ELSEWHERE

- Concrete Work, Section 03300

  - QUALITY ASSURANCE

- Use adequate numbers of skilled workmen who are thoroughly trained and experienced in necessary crafts and who are completely familiar with the specified requirements and the methods needed for proper performance of the work.

  - SUBMITTALS

A.	Submittals shall be made in accord with “SUPPLEMENTAL REQUIREMENTS FOR VENDOR DOCUMENTS/MATERIALS.”

- The Contractor shall examine the areas to receive chain link fencing and notify Owner of any conditions that may be detrimental to the finished work or may interfere with the timely completion of the work.  Beginning work constitutes acceptance of existing conditions and full responsibility of a finished job.

PART II – MATERIALS

  - Chain link fabric shall be woven into a 2” diamond shape mesh in which the individual pickets are helically woven and interwoven in the form of a continuous chain-like mesh.  Salvages shall be knuckled.

  - Galvanized-after-weaving chain link fabric shall be 11-gauge steel wire.  Galvanize in compliance with ASTM A 392, Class II.

  - FRAMEWORK

- Post, rails, braces and gate frames shall be fabricated of hot dip galvanized steel.  Galvanizing shall comply with ASTM A 123.

- Line post shall be 1.90-inch diameter Schedule 40 pipe.

- Terminal posts 2.375-inch OD, Schedule 40 pipe.

- Post tops shall be designed as to exclude moisture from the post.

- Top tension wire shall be 7-gauge wire.

- Gateposts, 2.875-inch OD, Schedule 40 pipe.

- Gates.  Six foot and fourteen foot leaf width double gates (see plans for width locations), frame formed from 2” OD pipe.  Fabric same as fence.

PART III – EXECUTION

  - ERECTION

- The chain link fence, gates, and accessories shall be installed in strict accordance with plans and specifications in a workmanlike manner.

- Encase the posts in concrete as follows: Extend concrete 6” below bottom of posts.  Provide 10” diameter encasement for line posts and 12” diameter for end, corner and gateposts.  Encasement depth shall be 36” for line posts and 42” for end, corner and gateposts.

- After posts are installed and concrete has set firmly, place top tension wire and securely anchor before fabric is hung.

- Secure ends of fabric with tension bars threaded through loops in fabric and attach to posts.

- Place fabric by securing one end and applying sufficient tension with mechanical fence stretchers to remove slack then attach to top tension wire a 24” on center.

- Gates shall be installed plumb and level and all hardware shall be provided for smooth operation.

- Clean up.  The Contractor shall promptly remove from site all excess material and other debris resulting from fence construction.

- Schlage brand padlocks by Contractor; furnish 3 sets of keys to the Owner.  All locks to be keyed the same.

END OF SECTION