Copyright 1997, The American Institute of Architects (AIA)

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes the following:

Adjust list below to suit Project.

          - Protecting existing trees and vegetation to remain.

          - Removing trees and other vegetation.

          - Clearing and grubbing.

          - Topsoil stripping.

          - Removing above-grade site improvements.

          - Disconnecting, capping or sealing, and abandoning site utilities in place.

          - Disconnecting, capping or sealing, and removing site utilities.

        - Related Sections include the following:

List below only construction that the reader might expect to find in this Section but is specified elsewhere.

          - Division 2 Section "Earthwork" for soil materials, excavating, backfilling, and site grading.

          - Division 2 Section "Landscaping" for finish grading, including placing and preparing topsoil for lawns and planting.

      - DEFINITIONS

Revise below to define topsoil according to local practices if required.  Topsoil defined below is the raw material that may need screening and amending before meeting landscaper's definition of topsoil.

        - Topsoil:  Natural or cultivated surface-soil layer containing organic matter and sand, silt, and clay particles; friable, pervious, and black or a darker shade of brown, gray, or red than underlying subsoil; reasonably free of subsoil, clay lumps, gravel, and other objects more than 2 inches (50 mm) in diameter; and free of weeds, roots, and other deleterious materials.

      - MATERIALS OWNERSHIP

        - Except for materials indicated to be stockpiled or to remain Owner's property, cleared materials shall become Contractor's property and shall be removed from the site.

      - SUBMITTALS

        - Photographs or videotape, sufficiently detailed, of existing conditions of trees and plantings, adjoining construction, and site improvements that might be misconstrued as damage caused by site clearing.

Revise paragraph below to suit Project.

        - Record drawings according to Division 1 Section "Contract Closeout."

          - Identify and accurately locate capped utilities and other subsurface structural, electrical, and mechanical conditions.

      - QUALITY ASSURANCE

Delete below if work of this Section is not extensive or complex enough to justify a preinstallation conference.  If retaining, coordinate with Division 1 Section "Project Meetings."

        - Preinstallation Conference:  Conduct conference at Project site to comply with requirements in Division 1 Section "Project Meetings."

      - PROJECT CONDITIONS

        - Traffic:  Minimize interference with adjoining roads, streets, walks, and other adjacent occupied or used facilities during site-clearing operations.

          - Do not close or obstruct streets, walks, or other adjacent occupied or used facilities without permission from Owner and authorities having jurisdiction.

          - Provide alternate routes around closed or obstructed traffic ways if required by authorities having jurisdiction.

Delete paragraph below if not required.  Coordinate with Owner.

        - Improvements on Adjoining Property:  Authority for performing indicated removal and alteration work on property adjoining Owner's property will be obtained by Owner before award of Contract.

Delete paragraph below if none.  Indicate items to be salvaged on Drawings or in schedules.

        - Salvable Improvements:  Carefully remove items indicated to be salvaged and store on Owner's premises where indicated.

        - Notify utility locator service for area where Project is located before site clearing.

- PRODUCTS (Not Applicable)

Delete "(Not Applicable)" above and retain "Soil Materials" Article below when soil backfill is required in below-grade areas after demolition.  Add other materials to suit Project.

      - SOIL MATERIALS

Revise paragraph and subparagraph below to include soil materials if required.  Delete if earthwork requirements remain in Division 2 Section "Earthwork."

        - Satisfactory Soil Materials:  Requirements for satisfactory soil materials are specified in Division 2 Section "Earthwork."

          - Obtain approved borrow soil materials off-site when satisfactory soil materials are not available on-site.

- EXECUTION

      - PREPARATION

        - Protect and maintain benchmarks and survey control points from disturbance during construction.

Delete paragraph below if erosion and sedimentation controls are included elsewhere in Division 2 or in Division 1 "Construction Facilities and Temporary Controls."

        - Provide erosion-control measures to prevent soil erosion and discharge of soil-bearing water runoff or airborne dust to adjacent properties and walkways.

        - Locate and clearly flag trees and vegetation to remain or to be relocated.

        - Protect existing site improvements to remain from damage during construction.

          - Restore damaged improvements to their original condition, as acceptable to Owner.

      - TREE PROTECTION

Delete this Article if not required.

        - Erect and maintain a temporary fence around drip line of individual trees or around perimeter drip line of groups of trees to remain.  Remove fence when construction is complete.

          - Do not store construction materials, debris, or excavated material within drip line of remaining trees.

          - Do not permit vehicles, equipment, or foot traffic within drip line of remaining trees.

        - Do not excavate within drip line of trees, unless otherwise indicated.

Retain paragraph below if construction excavation within drip line of trees is unavoidable.  Most roots of trees and shrubs occur in top 18 inches (450 mm) of soil.

        - Where excavation for new construction is required within drip line of trees, hand clear and excavate to minimize damage to root systems.  Use narrow-tine spading forks, comb soil to expose roots, and cleanly cut roots as close to excavation as possible.

Retain applicable subparagraphs below.

          - Cover exposed roots with burlap and water regularly.

          - Temporarily support and protect roots from damage until they are permanently relocated and covered with soil.

          - Coat cut faces of roots more than 1-1/2 inches (38 mm) in diameter with an emulsified asphalt or other approved coating formulated for use on damaged plant tissues.

          - Cover exposed roots with wet burlap to prevent roots from drying out.  Backfill with soil as soon as possible.

        - Repair or replace trees and vegetation indicated to remain that are damaged by construction operations, in a manner approved by Architect.

          - Employ a qualified arborist, licensed in jurisdiction where Project is located, to submit details of proposed repairs and to repair damage to trees and shrubs.

Revise subparagraph above and below to suit Project.

          - Replace trees that cannot be repaired and restored to full-growth status, as determined by the qualified arborist.

      - UTILITIES

Retain paragraph below if Owner arranges this work.

        - Owner will arrange for disconnecting and sealing indicated utilities that serve existing structures before site clearing when requested by Contractor.

          - Verify that utilities have been disconnected and capped before proceeding with site clearing.

Delete paragraph and subparagraph above and retain paragraph and applicable subparagraph below if Contractor arranges or performs this work.

        - Locate, identify, disconnect, and seal or cap off utilities indicated to be removed.

          - Owner will arrange to shut off indicated utilities when requested by Contractor.

Retain subparagraph above or below.

          - Arrange to shut off indicated utilities with utility companies.

Delete paragraph below if no existing utilities.  Coordinate with requirements specified in Division 1 Section "Construction Facilities and Temporary Controls" for temporary utilities.

        - Existing Utilities:  Do not interrupt utilities serving facilities occupied by Owner or others unless permitted under the following conditions and then only after arranging to provide temporary utility services according to requirements indicated:

Edit subparagraph below to suit Project

          - Notify Architect not less than two days in advance of proposed utility interruptions.

          - Do not proceed with utility interruptions without Architect's written permission.

        - Excavate for and remove underground utilities indicated to be removed.

Delete paragraph above or below.  If retaining above, coordinate removal of utilities with provisions for excavation, backfill, and compaction in Division 2 Section "Earthwork." if retaining below, coordinate with Divisions 15 and 16.

        - Removal of underground utilities is included in Division 15 mechanical or Division 16 electrical Sections.

      - CLEARING AND GRUBBING

        - Remove obstructions, trees, shrubs, grass, and other vegetation to permit installation of new construction.  Removal includes digging out stumps and obstructions and grubbing roots.

          - Do not remove trees, shrubs, and other vegetation indicated to remain or to be relocated.

          - Cut minor roots and branches of trees indicated to remain in a clean and careful manner where such roots and branches obstruct installation of new construction.

          - Completely remove stumps, roots, obstructions, and debris extending to a depth of 18 inches (450 mm) below exposed subgrade.

          - Use only hand methods for grubbing within drip line of remaining trees.

Delete paragraph below if not required.  Coordinate with Division 2 Section "Earthwork."

        - Fill depressions caused by clearing and grubbing operations with satisfactory soil material, unless further excavation or earthwork is indicated.

          - Place fill material in horizontal layers not exceeding 8-inch (200-mm) loose depth, and compact each layer to a density equal to adjacent original ground.

      - TOPSOIL STRIPPING

        - Remove sod and grass before stripping topsoil.

        - Strip topsoil to whatever depths are encountered in a manner to prevent intermingling with underlying subsoil or other waste materials.

Revise subparagraph below if other materials are known to be mixed into unsuitable surface soil.

          - Strip surface soil of unsuitable topsoil, including trash, debris, weeds, roots, and other waste materials.

Revise paragraph below if topsoil is to be removed from site.

        - Stockpile topsoil materials away from edge of excavations without intermixing with subsoil.  Grade and shape stockpiles to drain surface water.  Cover to prevent windblown dust.

          - Limit height of topsoil stockpiles to 72 inches (1800 mm).

          - Do not stockpile topsoil within drip line of remaining trees.

          - Dispose of excess topsoil as specified for waste material disposal.

Select subparagraph above or below.

          - Stockpile surplus topsoil and allow for respreading deeper topsoil.

      - SITE IMPROVEMENTS

Edit paragraph below and add specific items to suit Project.

        - Remove existing above- and below-grade improvements as indicated and as necessary to facilitate new construction.

        - Remove slabs, paving, curbs, gutters, and aggregate base as indicated.

          - Unless existing full-depth joints coincide with line of demolition, neatly saw-cut length of existing pavement to remain before removing existing pavement.  Saw-cut faces vertically.

      - DISPOSAL

        - Disposal:  Remove surplus soil material, unsuitable topsoil, obstructions, demolished materials, and waste materials, including trash and debris, and legally dispose of them off Owner's property.

END OF SECTION