PART I - GENERAL

  - Furnish and install all plumbing fixtures with all trim, fittings, wall hangers, carriers and other devices for a complete installation.  All fixtures and accessories shall be by the same manufacturer and all trim and fittings shall be by the fixture manufacturer, unless otherwise noted.

# PART II - PRODUCTS

  - FIXTURES

- Plumbing fixtures as called for in "Plumbing Fixture Schedule".

# PART III - EXECUTION

  - After plumbing fixtures are set in place, the cracks between the fixture and wall shall be caulked carefully with "Tube-Tite" or approved equal.

  - All exposed metal trim shall be chromium plated and polished.

  - Provide closets with white bolt caps and retainer clip.

  - Angle valves at fixtures shall have ½" IPS inlets, 3/8" O.D. risers, and chromium plated tubing.

  - Escutcheons shall have depth to conceal all pipe and fittings that are not chromium plated and its diameter shall cover wall openings.  All escutcheons shall have set screw.  Tolerance type fit, or split ring escutcheons will not be accepted.

  - All interior and exterior hose bibbs and hydrants to have ball valve installed at takeoff from main water supply.

## END OF SECTION