# PART 1 - GENERAL

1.01 RELATED DOCUMENTS

A.	Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

1.02 SUMMARY

A.	This Section includes meter, backflow prevention system, valves, piping, sprinklers, specialties, accessories, controls, and wiring for lawn and planting irrigation systems.

B.	Related Sections:  The following Sections contain requirements that relate to this Section:

1.	Division 2 Section "Water Distribution”

C.	The term "Contractor" as used in this section shall refer to the Irrigation Contractor unless otherwise indicated.

D.	The Contractor shall review the drawings and specifications to coordinate work of this section with all other related work.

E.	Contractor shall visit the site and fully inform himself as to all existing conditions and limitations that may apply to the work.

1.03 DESCRIPTION

A.	Extent: Contractor is responsible for providing all services, labor, equipment, materials, and supplies to install a complete, fully operational automatic irrigation system as per the Drawings and per local and state requirements.

B.	Limits of Work: Drawings show project site boundary lines for the purpose of identification, within which the irrigation work is to be confined, unless noted otherwise.

1.04	REQUIREMENTS BY REGULATORY AGENCIES

A.	Ordinances and Regulations: Local, municipal and state laws,

rules and regulations governing or relating to any portion of this work are hereby incorporated into and made a part of these specifications and their provisions shall be carried out by Contractor.  However, when these specifications and drawings call for or describe materials, workmanship or construction of a better quality, higher standard or larger size, they shall take precedence over the requirements or such rules and regulations.

B.	Permits and Inspections: Permits for the installation or construction of the work which are required by the legally constituted authorities having jurisdiction, shall be obtained and paid for by the Contractor, each at the proper time.  Contractor shall also arrange for and pay costs in connection with inspections and examinations required by these authorities.

1.05 SYSTEM PERFORMANCE REQUIREMENTS

A.	Provide Owner with a complete, fully operational automatic irrigation system with all system components including, but not limited to:  Meter, backflow preventer, controller(s), piping, valves, heads, drip system, rain and freeze sensors and wiring.

B.	Minimum Water Coverage:  Not less than:

1.	Turf Areas: 100 percent.

2.	Precipitation rates must be consistently matched throughout any one zone.

C.	Components and Installation:  Capable of producing piping systems with the following minimum working pressure ratings except where indicated otherwise.

1.	Pressure Piping (Mainline): Schedule 80, 200 psig (1380 kPa).

2.	Circuit and Drain Piping: Schedule 40, 160 psig (1104 kPa).

1.06 SUBMITTALS

A.	General:  Submit the following according to the Conditions of the Contract and Division 1 Specification Sections.

B.	Product data including pressure rating, rated capacity, settings, and electrical data of selected models for the following:

1.	Water meter.

2.	Backflow preventers, including test equipment.

3.	Pressure regulator (if required).

4.	Valves, including general-duty, underground, manual and automatic control, and types, and valve boxes.

5.	Sprinkler heads and drip equipment, including emitter tubing, drip tubes and emitters for spot irrigation, and devices.

6.	Controls, including controller wiring diagrams.

7.	Rain cut-off switch.

8.	Wiring and splicing.

9.	Automatic drain valve.

10.	Enlosures, valve boxes.

C.	Wiring diagrams for electrical controllers, valves, and devices.

D.	Submit project record (as built) drawings as specified below:

1.	Maintain one set of working prints for annotating installation additions, changes, relocations and corrections to the installation plan.  Include all wiring as routed from controller to valves on the working prints. These shall be carefully recorded and kept up to date throughout the progress of the job to completion of the project, whereupon they shall be returned to the Owner, providing a permanent record of this work.

E.	Maintenance data and operating instructions for inclusion in "Operating and Maintenance Manual" to be provided to the Owner and shall include the following:

1.	Water meter.

2.	Backflow preventers, instructions for testing.

3.	Pressure regulators.

4.	Automatic and manual control valves.

5.	Drip irrigation emitters, filters, flush valves and vacuum relief valves.

6.	Sprinklers.

7. 	Controllers

1.07 QUALITY ASSURANCE

A.	Comply with requirements of authority with jurisdiction for irrigation systems.

B.	Installer Qualifications:  Engage an experienced Installer who has completed irrigation systems similar in material, design, and extent to that indicated for Project that have resulted in construction with a record of successful in-service performance.

C.	Listing/Approval Stamp, Label, or Other Marking:  On equipment, specialties, and accessories made to specified standards.

1.	The Terms "Listed" and "Labeled":  As defined in "National Electrical Code," Article 100.

- Product Options:  Irrigation system piping, specialties, and accessories are based on specific types, manufacturers, and models indicated.  Components with equal performance characteristics produced by other manufacturers may be considered, provided deviations in dimensions, operation, and other characteristics do not change design concept or intended performance as judged by the Architect.  The controller and automatic control valves are to be of a single acceptable manufacturer as listed in the construction documents.  The burden of proof of product equality is on the Contractor.

E.	Manufacturer's Specifications: The latest printed specifications of approved manufacturer of materials shall become part of these specifications.

1.08 PROJECT CONDITIONS

A.	Perform site survey, research public utility records, and verify existing utility locations.  Verify that irrigation system piping may be installed in compliance with original design and referenced standards.

B.	Site Information:  Reports on subsurface condition investigations made during design of the Project are available for informational purposes only; data in reports are not intended as representations or warranties of accuracy or continuity of conditions (between soil borings).  Owner assumes no responsibility for interpretations or conclusions drawn from this information.

C.	Damage by Leaks: Contractor shall be responsible for all damages to: the grounds; walks; roads; foundations; building piping system; electrical systems, their equipment and contents; persons or property from accidents or injuries directly or indirectly caused by leaks in piping systems being installed or having been installed by Contractor.  Repair, at Contractor's expense, all damages so caused.  Repair work shall be done as directed by the Owner's Representative.

1.09 SEQUENCING AND SCHEDULING

A.	Coordinate irrigation systems work with landscape work specified in Division 2 Section "Landscaping."

1.10 EXTRA MATERIALS

A.	Deliver extra materials to Owner.  Furnish extra materials matching products installed as described below.  Package them with protective covering for storage and label clearly describing contents.

1.	Sprinklers:  Furnish quantity of units equal to 10 percent of amount of each type installed.

2.	Sprinkler Nozzles: Furnish quantity of units equal to 10 percent of amount of each type installed.

3.	Valve Keys:  Furnish quantity of tee-handle units equal to 25 percent of amount of each type key-operated, control valve installed.

4.	Emitters, drip tube, and devices: Furnish quantity of units equal to 10 percent of amount of each type installed.

# PART 2 - PRODUCTS

2.01 MANUFACTURERS

A.	Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated in the Work include, but are not limited to, the following:

1.	Automatic Control Valves:

a.	Rainbird

2.	Control Valve Boxes:

a.	Ametek by Plymouth Products Div., AMETEK.

b.	NDS

3.	Sprinklers:

a.	Rainbird

4.	Emitters, Drip Tubes and Devices (No substitutions):

a.	Netafim Irrigation

5.	Controllers:

a.	Rainbird

6.	Rain cut-off Switch/Moisture Sensor

a.	Rainbird

7.	Freeze cut-off switch:

a.	Rainbird

2.02 AUTOMATIC IRRIGATION SYSTEMS COMPONENTS.

A.	General:  Control valves, sprinkler heads, and controller shall be by a single manufacturer per the manufacturer's specifications.  All drip irrigation components shall be of a single manufacturer per the manufacturer's specs.

B.	Controller shall be a solid state hybrid type with dual programming for lawn and shrub areas; 14 day programming (Minimum); capable of 3 start times per program minimum; watering delay; water conservation control; manual start of each station or entire program; non-volatile memory and fuse protected circuitry.  Controls shall be installed in a lockable wall-mount steel enclosure.

C.	Pressure piping shall be:  PVC schedule 80, rated 200 psig, with socket type pipe fittings and solvent cemented joints.

D.	Circuit piping shall be:  PVC Schedule 40, rated 160 psig, with socket type pipe fittings and solvent cemented joints; OR polyethylene pipe with 100 psig minimum pressure rating.

E.	Wiring: Electrical system equipment shall comply with the Electric Code most recently adopted by the local authorities, and shall be UL-approved.

1.	Wiring between automatic controllers and remote control valves shall be direct burial copper wire, #14 AWG, Type UF, UL-approved 600 volt.

2.	Where more than 1 wire are grouped together in a trench, bundle together with tape at 10' intervals.

# PART 3 - EXECUTION

3.01 EXAMINATION

A.	Investigate and determine available water supply water pressure and flow characteristics.

3.02 PREPARATION

A.	Inspection: Examine the areas and conditions under which landscape irrigation system is to be installed and notify the Contractor of conditions detrimental to the proper installation of the work.  Do not proceed with the work until unsatisfactory conditions have been corrected in a manner acceptable to the Owner's Representative.

B.	Sleeves and Conduits: Contractor for work under this Section shall be required to determine and/or verify the location of sleeves through walls/foundations, under walks, under paving and similar places and set sleeves. Contractor shall be required to determine and/or verify the location of conduits through walls and to the roof for the sensors and coordinate this work with the Electrical and General Contractors.

C.	Existing Utilities: Verify with the Architect, Owner and local utilities the location of all utility lines, in the project area, before irrigation system installation begins.  Damage caused by this work to these utilities, shall be repaired at no additional cost to Owner.

3.03 SYSTEM DESIGN

A.	Design Pressures: Verify irrigation supply pressure and volume before installing equipment.  Notify Architect if inconsistent with that shown on drawings.

B.	Location of Heads: Design location is approximate.  Make minor adjustments as necessary to avoid plantings and other unforeseen obstructions.  Do not compromise coverage of irrigation design.

3.04 EARTHWORK

A.	Trenching: Excavate trenches to uniform widths to provide a working clearance on each side of pipe or conduit.  Excavate trench walls vertically from trench bottom.

1.	Clearance: 6 inches (minimum) each side of pipe or conduit.

2.	Trench Bottoms:  Excavate and shape trench bottoms to provide uniform bearing and support of pipes and conduit.  Shape subgrade to provide continuous support for bells, joints, and barrels of pipes and for joints, fittings, and bodies of conduits.  Remove stones and sharp objects to avoid point loading.

3.	Pipe depth:

Pressure piping: 18 inches (300 mm) to top of pipe or sleeve, unless otherwise indicated.

Lateral piping: 12 inches (300 mm) to top of pipe or conduit, unless otherwise indicated.

4.	For pipes or sleeving less than 6 inches in nominal diameter, hand-excavate trench bottoms and support pipe and sleeving on an undisturbed subgrade.

5.	Where encountering rock or another unyielding bearing surface, carry trench excavation 3 inches below pipe invert elevation and install bedding course. Top of pipe to be no less than 12" below finish grade.

B.	Underground warning tape shall be installed to protect system wiring and mainline.

C.	Backfill excavations promptly, but not before completing the following:

1.	Surveying locations of underground utilities for record documents.

2.	Testing, inspecting, and approval of underground utilities.

3.	Removal of trash and debris from excavation.

4.	Removal of temporary shoring and bracing, and sheeting.

5.	Pressure testing system for a minimum of four hours as specified below, not to exceed 75% of pressure rating of pipe.  System may be tested in portions as approved by Architect.  Notify General Contractor 24 hours in advance of system pressure testing for inspection.  General Contractor shall verify results of system pressure test.

D.	Compact backfill and fill materials in layers not more than 4 inches in loose depth.

1.	Percentage of Maximum Dry Density Requirements:  Compact soil to not less than the following percentages of maximum dry density according to ASTM D 1557:

2.	Under lawn or unpaved areas, compact the top 6 inches  below subgrade and each layer of backfill or fill material at 90 percent maximum dry density.

3.05 FIELD QUALITY CONTROL

A.	Testing:  Perform hydrostatic test of piping and valves before backfilling trenches.  With prior approval, piping may be tested in sections to expedite work.

1.	Cap and subject the piping system to a static water pressure of 150 psig (345 kPa).  Isolate test source and allow to stand for 4 hours.  Leaks and loss in test pressure constitute defects that must be repaired.

2.	Repair leaks and defects with new materials and retest system or portion thereof until satisfactory results are obtained.

3.	Sprinkler heads shall be installed and tested for operation in accordance with design requirements under normal operating pressure.  All necessary adjustments to the sprinkler equipment as required shall be made to assure efficient operation.

4.	Drip System: Prior to backfilling, open the remote control valve and operate each circuit to check for leakage around both barbed and threaded PVC fittings.  Make necessary corrections to stop leaks.

3.06	CLEANING AND ADJUSTING

A.	Flush dirt and debris from piping before installing sprinklers and other devices.

B.	Adjust automatic control valves to provide flow rate of rated operating pressure required for each sprinkler circuit.

C.	Carefully adjust lawn sprinklers so they will be flush with finish grade after completion of landscape work.

D.	Adjust settings of controllers and automatic control valves.

3.07 COMMISSIONING

A.	Starting Procedures:  Follow manufacturer's written procedures.  If no procedures are prescribed by manufacturers, proceed as follows:

1.	Verify that specialty valves and their accessories have been installed correctly and operate correctly.

2.	Verify that specified tests of piping are complete.

3.	Check that sprinklers and devices are correct type.

4.	Check that damaged sprinklers and devices have been replaced with new materials.

5.	Check that potable water supplies have correct type backflow preventers.

6.	Energize circuits to electrical equipment and devices.

7.	Adjust operating controls.

B.	Operational Testing:  Perform operational testing after hydrostatic testing is completed, backfill is in place, and sprinklers are adjusted to final position.

3.08 SYSTEM OPERATION AND ACCEPTANCE

A.	Operate the completed system for the Architect and the Owner's inspection for final  acceptance at which time each sprinkler head shall be visually checked for coverage, adjustment, installation and proper automatic or manual control.  As built "working prints" will be checked for accuracy.  Cleanup of the site will be checked; and general items such as manual drain valves, controller, and automatic control valves and quick coupler valves will be inspected for proper installation and operation.  All loose equipment, including controller keys, and related equipment as required or as necessary for the Owner's operation of the system will be turned over prior to final acceptance.

B.	Instruct Owner's personnel or designated representative in complete operation and maintenance of the irrigation system including but not limited to the controller operation, maintenance of the systems filters and adjustment of heads.

C.	Maintenance: After final acceptance of the completed installation of irrigation related materials, Contractor for work under this section shall be made responsible for systems maintenance for 60 days and shall guarantee the design, parts and workmanship of irrigation system for a period of one year after final acceptance.

D.	Provide 7 days' written notice in advance of demonstration.

3.09 GUARANTEE

A.	After final acceptance of the completed installation of all irrigation related materials, Contractor for work under this section shall guarantee the design, parts and workmanship of irrigation system for a period of one calendar year.

B.	All materials and equipment shall be warranted in writing against defect in materials and workmanship by the respective manufacturers.  These written warranties shall be assembled and given to the Owner at the time of final acceptance.  All installation work shall be guaranteed for one calendar year after final acceptance.  No claims under warranty shall be considered for materials damaged or destroyed by vandals or damages caused by unauthorized operation of the system.

C.	Contractor shall meet with Owner's Representative two weeks prior to the termination of the one year guarantee and run the system one more time and make all necessary changes, repairs, etc. as are needed to deliver a final working system to the Owner.

END OF SECTION