PART 1 - GENERAL

1.01		RELATED DOCUMENTS

A.	Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

B.	See also description of alternative #3 on the Construction Drawings, sheets C3 and C4.

1.02		SUMMARY

A.	This Section includes the following:

1.	Lawns.

2.	Topsoil and soil amendments.

3.	Fertilizers and mulches.

B.	Related Sections: The following Sections contain requirements that relate to this Section:

1.	Division 2 Section "Site Clearing" for protection of existing trees and planting, topsoil stripping and stockpiling, and site clearing.

2.	Division 2 Section "Earthwork" for excavation, filling, rough grading, and subsurface aggregate drainage and drainage backfill.

1.03		SUBMITTALS

A.	General: Submit each item in this Article according to the Conditions of the Contract and Division 1 Specification Sections.

B.	Product certificates signed by manufacturers certifying that their products comply with specified requirements.

1.	Manufacturer's certified analysis for standard products.

C.	Certification of grass seed from seed vendor for each grass-seed mixture stating the botanical and common name and percentage by weight of each species and variety, and percentage of purity, germination, and weed seed.  Include the year of production and date of packaging.

1.	Certification of each seed mixture for sod, identifying sod source, including name and telephone number of supplier.

D.	Material test reports from qualified independent testing agency indicating and interpreting test results relative to compliance of the following materials with requirements indicated.

1.	Analysis of existing surface soil.

2.	Analysis of imported topsoil.

E.	Maintenance instructions recommending procedures to be established by Owner for maintenance of landscaping during an entire year.  Submit before expiration of required maintenance periods.

1.04		QUALITY ASSURANCE

A.	Installer Qualifications: Engage an experienced Installer who has completed landscaping work similar in material, design, and extent to that indicated for this Project and with a record of successful landscape establishment.

1.05		DELIVERY, STORAGE, AND HANDLING

A.	Packaged Materials: Deliver packaged materials in containers showing weight, analysis, and name of manufacturer.  Protect materials from deterioration during delivery and while stored at site.

B.	Seed: Deliver seed in original sealed, labeled, and undamaged containers.

1.06		PROJECT CONDITIONS

A.	Utilities: Determine location of above grade and underground utilities and perform work in a manner which will avoid damage.  Hand excavate, as required.  Maintain grade stakes until removal is mutually agreed upon by parties concerned.

B.	Excavation: When conditions detrimental to plant growth are encountered, such as rubble fill, adverse drainage conditions, or obstructions, notify Architect before planting.

1.07		WARRANTY

A.	General Warranty: The special warranty specified in this Article shall not deprive the Owner of other rights the Owner may have under other provisions of the Contract Documents and shall be in addition to, and run concurrent with, other warranties made by the Contractor under requirements of the Contract Documents.

1.08		LAWN MAINTENANCE

A.	Begin maintenance of lawns immediately after each area is planted and continue until acceptable lawn is established, but for not less than the following periods:

1.	Seeded Lawns: 60 days after date of Substantial Completion.

a.	When full maintenance period has not elapsed before end of planting season, or if lawn is not fully established at that time, continue maintenance during next planting season.

B.	Maintain and establish lawns by watering, fertilizing, weeding, mowing, trimming, replanting, and other operations.  Roll, re-grade, and replant bare or eroded areas and re-mulch to produce a uniformly smooth lawn.

C.	Watering: Provide and maintain temporary piping, hoses, and lawn-watering equipment to convey water from sources and to keep lawns uniformly moist to a depth of 4 inches (100 mm).

1.	Water lawn at the minimum rate as indicated on the Construction Drawings.

D.	Mow lawns as soon as there is enough top growth to cut with mower set at specified height for principal species planted.  Repeat mowing as required to maintain specified height without cutting more than 40 percent of the grass height.  Remove no more than 40 percent of grass-leaf growth in initial or subsequent mowing.  Do not delay mowing until grass blades bend over and become matted.  Do not mow when grass is wet.

E.	Post-fertilization: Apply fertilizer to lawn after first mowing and when grass is dry.

1.	Use fertilizer that will provide actual nitrogen of at least 1 lb per 1000 sq. ft. (0.5 kg per 100 sq. m) of lawn area.

PART 2 - PRODUCTS

2.01		GRASS MATERIALS

A.	Grass Seed: Fresh, clean, dry, new-crop seed complying with the Association of Official Seed Analysts' "Rules for Testing Seeds" for purity and germination tolerances.

1.	Seed Mixture: Provide seed of grass species and varieties, proportions by weight, and minimum percentages of purity, germination, and maximum percentage of weed seed as indicated on drawings.

2.02		TOPSOIL

A.	Topsoil: ASTM D 5268, pH range of 5.5 to 7, 4 percent organic material minimum, free of stones 1 inch (25 mm) or larger in any dimension, and other extraneous materials harmful to plant growth.

1.	Topsoil Source: Reuse surface soil stockpiled on the site.  Verify suitability of surface soil to produce topsoil meeting requirements and amend when necessary.  Supplement with imported topsoil when quantities are insufficient.  Clean topsoil of roots, plants, sods, stones, clay lumps, and other extraneous materials harmful to plant growth.

2.03		SOIL AMENDMENTS

A.	Lime: ASTM C 602, Class T, agricultural limestone containing a minimum 80 percent calcium carbonate equivalent, with a minimum 99 percent passing a No. 8 (2.36 mm) sieve and a minimum 75 percent passing a No. 60 (250 micrometer) sieve.

1.	Provide lime in the form of dolomitic limestone.

B.	Aluminum Sulfate: Commercial grade, unadulterated.

C.	Sand: Clean, washed, natural or manufactured sand, free of toxic materials.

D.	Perlite: Horticultural perlite, soil amendment grade.

E.	Peat Humus: Finely divided or granular texture, with a pH range of 6 to 7.5, composed of partially decomposed moss peat (other than sphagnum), peat humus, or reed-sedge peat.

F.	Sawdust or Ground-Bark Humus: Decomposed, nitrogen-treated, of uniform texture, free of chips, stones, sticks, soil, or toxic materials.

1.	When site treated, mix with at least 0.15 lb (2.4 kg) of ammonium nitrate or 0.25 lb (4 kg) of ammonium sulfate per cu. ft. (cu. m) of loose sawdust or ground bark.

G.	Manure: Well-rotted, unleached stable or cattle manure containing not more than 25 percent by volume of straw, sawdust, or other bedding materials; free of toxic substances, stones, sticks, soil, weed seed, and material harmful to plant growth.

H.	Herbicides: EPA registered and approved, of type recommended by manufacturer.

I.	Water: Potable.

2.04		FERTILIZER

A.	Bonemeal: Commercial, raw, finely ground; minimum of 4 percent nitrogen and 20 percent phosphoric acid.

B.	Commercial Fertilizer: Commercial-grade complete fertilizer of neutral character, consisting of fast- and slow-release nitrogen, 50 percent derived from natural organic sources of urea-form, phosphorous, and potassium in the following composition:

1.	Composition: Nitrogen, phosphorous, and potassium in amounts recommended in soil reports from a qualified soil-testing agency.

C.	Slow-Release Fertilizer: Granular fertilizer consisting of 50 percent water-insoluble nitrogen, phosphorus, and potassium in the following composition:

1.	Composition: Nitrogen, phosphorous, and potassium in amounts recommended in soil reports from a qualified soil-testing agency.

2.05		MULCHES

A.	Fiber Mulch: Biodegradable dyed-wood cellulose-fiber mulch, nontoxic, free of plant growth- or germination-inhibitors, with maximum moisture content of 15 percent and a pH range of 4.5 to 6.5.

B.	Non-asphalt Emulsion Tackifier: Nontoxic and free of plant growth- or germination-inhibitors.

PART 3 - EXECUTION

3.01		EXAMINATION

A.	Examine areas to receive landscaping for compliance with requirements and for

conditions affecting performance of work of this Section.  Do not proceed with installation until unsatisfactory conditions have been corrected.

3.02		PLANTING SOIL PREPARATION

A.	Before mixing, clean topsoil of roots, plants, sods, stones, clay lumps, and other extraneous materials harmful to plant growth.

B.	Mix soil amendments and fertilizers with topsoil at rates indicated.  Delay mixing fertilizer if planting does not follow placing of planting soil within a few days.

C.	For planting beds and lawns, mix planting soil either prior to planting or apply on surface of topsoil and mix thoroughly before planting.

1.	Mix lime with dry soil prior to mixing fertilizer.  Prevent lime from contacting roots of acid-tolerant plants.

2.	Apply phosphoric acid fertilizer, other than that constituting a portion of complete fertilizers, directly to subgrade before applying planting soil and tilling.

3.03		LAWN PLANTING PREPARATION

A.	Limit subgrade preparation to areas that will be planted in the immediate future.

B.	Loosen subgrade to a minimum depth of 4 inches (100 mm).  Remove stones larger than 1-1/2 inches (38 mm) in any dimension and sticks, roots, rubbish, and other extraneous materials.

C.	Spread planting soil mixture to depth required to meet thickness, grades, and elevations shown, after light rolling and natural settlement.  Do not spread if planting soil or subgrade is frozen.

1.	Place approximately 1/2 the thickness of planting soil mixture required.  Work into top of loosened subgrade to create a transition layer and then place remainder of planting soil mixture.

D.	Preparation of Unchanged Grades: Where lawns are to be planted in areas unaltered or undisturbed by excavating, grading, or surface soil stripping operations, prepare soil as follows:

1.	Remove and dispose of existing grass, vegetation, and turf.  Do not turn over into soil being prepared for lawns.

2.	Till surface soil to a depth of at least 6 inches (150 mm).  Apply required soil amendments and initial fertilizers and mix thoroughly into top 4 inches (100 mm) of soil.  Trim high areas and fill in depressions.  Till soil to a homogenous mixture of fine texture.

3.	Clean surface soil of roots, plants, sods, stones, clay lumps, and other extraneous materials harmful to plant growth.

4.	Remove waste material, including grass, vegetation, and turf, and legally dispose of it off the Owner's property.

E.	Grade lawn and grass areas to a smooth, even surface with loose, uniformly fine texture.  Roll and rake, remove ridges, and fill depressions to meet finish grades.  Limit fine grading to areas that can be planted in the immediate future.  Remove trash, debris, stones larger than 1-1/2 inches (38 mm) in any dimension, and other objects that may interfere with planting or maintenance operations.

F.	Moisten prepared lawn areas before planting when soil is dry.  Water thoroughly and allow surface to dry before planting.  Do not create muddy soil.

G.	Restore prepared areas if eroded or otherwise disturbed after fine grading and before planting.

3.04		SEEDING NEW LAWNS (OUTSIDE OF SOCCER FIELD PERIMETER)

A.	Sow seed with a spreader or a seeding machine.  Do not broadcast or drop seed when wind velocity exceeds 5 mph (8 km/h).  Evenly distribute seed by sowing equal quantities in 2 directions at right angles to each other.

1.	Do not use wet seed or seed that is moldy or otherwise damaged in transit or storage.

B.	Sow seed at the following rates:

1.	Seeding Rate: 3 to 4 lb. per 1000 sq. ft. (1.5 to 2 kg per 100 sq. m).

C.	Rake seed lightly into top 1/8 inch (3 mm) of topsoil, roll lightly, and water with fine spray.

D.	Protect seeded areas with slopes less than 1:6 against erosion by spreading straw mulch after completion of seeding operations.  Spread uniformly at a minimum rate of 2 tons per acre (45 kg per 100 sq. m) to form a continuous blanket 1-1/2 inches (38 mm) loose depth over seeded areas.  Spread by hand, blower, or other suitable equipment.

E.	Hydroseeding is an acceptable alternative, at Contractor's option.

3.05		SPRIGGING AREA WITHIN SOCCER FIELD PERIMETER

- See Construction Drawing for base bid sprigging requirements.

3.06		RECONDITIONING LAWNS

A.	Recondition existing lawn areas damaged by Contractor's operations, including storage of materials or equipment and movement of vehicles.  Also recondition lawn areas where settlement or washouts occur or where minor regrading is required.

B.	Remove vegetation from diseased or unsatisfactory lawn areas; do not bury into soil.  Remove topsoil containing foreign materials resulting from Contractor's operations, including oil drippings, fuel spills, stone, gravel, and other construction materials, and replace with new topsoil.

C.	Remove waste and foreign materials, including weeds, soil cores, grass, vegetation, and turf, and legally dispose of it off the Owner's property.

D.	Till stripped, bare, and compacted areas thoroughly to a depth of 6 inches (150 mm).

E.	Apply required soil amendments and initial fertilizers and mix thoroughly into top 4 inches (100 mm) of soil.  Provide new planting soil as required to fill low spots and meet new finish grades.

F.	Apply seed as required for new lawns.

G.	Water newly planted areas and keep moist until new grass is established.

3.07		CLEANUP AND PROTECTION

H.	During landscaping, keep pavements clean and work area in an orderly condition.

I.	Protect landscaping from damage due to landscape operations, operations by other contractors and trades, and trespassers.  Maintain protection during installation and maintenance periods.  Treat, repair, or replace damaged landscape work as directed.

3.08		DISPOSAL OF SURPLUS AND WASTE MATERIALS

J.	Disposal: Remove surplus soil and waste material, including excess subsoil, unsuitable soil, trash, and debris, and legally dispose of it off the Owner's property.

3.09		SEED MIXTURES SCHEDULE

A.	Sun and Partial Shade: Provide certified grass-seed blends or mixes, proportioned by weight, as follows:

Min.	Min.	Max.

Pct.	Pct.	Pct.

Proportion	Name	Germ.	Pure Sd.	Weed Sd.

50 %	Kentucky 31	80%	85%	0.5%

30 %	Chewings red fescue	85%	98%	0.5%

10 %	Perennial rye grass	90%	98%	0.5%

10 %	Redtop	85%	92%	1%

END OF SECTION 02900