PART I - GENERAL

1.01	Provide air handling equipment of types and capacities as shown on drawings.

PART II - DRAWINGS

  - IN-LINE EXHAUST FANS

- Provide in-line centrifugal exhaust fans or belt or direct drive as indicated on plans.

- Fan casing to be heavy gauge galvanized steel with access panels to allow motor and drive to be removed without removing fan casing.

- Bearings to be permanently lubricated pillow block type.

- Wheel to be backward inclined, statically and dynamically balanced.

- Fan to be approved for mounting arrangement as shown on plans.

## PART III - EXECUTION - NOT USED

# END OF SECTION