Chapter 11. Spherical Aberration.

PA1.Calculation of  LSA = xi1 - xi1sal for a single Spherical Surface and fixed Value of the object Distance.(see p.408)

Go to FileFig.(A1SPHASS) and use as input data

ro =2, r = 10, xo = -30, and n1 = 1.5 and also n2 = 3.4.

a. Calculate the image distance xi.

b. Calculate the image distance with correction of spherical aberration.

c. Calculate  the longitudinal spherical aberration L.S.A.

d. Chang n and see that for real objects the L.S.A. is never zero.

PA2.Calculation of LSA = xi1 - xi1sal for a single spherical Surface for a Range of object Distances.(see p.408)

Go to FileFig.(A2SPHSSS) and use as input data ro = 6, r = 8,  and n1 = 1.7

a. Determine the object focal point.

b. Plot the graph from the object focal point to –100 and see that L.S.A. is never zero.

c. Go to the third part of FileFig.(A2SPHSSS) and use the same ro and look at the plots over nn. Change rr, and use negative xxo and see that for real object distances the L.S.A. is never zero.

c. Go to the fourth part of FileFig.(A2SPHSSS) and use the same ro and look at the plots over rrr from 0 to 20 in steps of 0.1. Use negative xxo = -30 and see that only for a singularity the value of L.S.A. is zero. Determine the rrr value of the singularity from the formula of xi and read it off the graph.

PA3. Spherical Aberration of a thin Lens.(see p.412)

In Jenkins and White, fourth Ed, p.156 several lenses are shown with different shapes. Consider three lenses, a plane-convex lens (  = -1), a bi-convex lens (  = 0), and a convex-plane lens (  = 1). Consider FileFig.(A4SPHLSIPIS) and use  n = 1.5,

 = -.333, ro = 4 and f = 10 and fill in the Table. The calculation of Y serves as a measure of aberration. Find out which of the lenses has the smallest value. For that lens, is the side of the incident light flat or curved, and what is the second side?

r1	r2		Y

inf	-5	-1

10	-10	0

5	inf	1

PA4. Spherical Aberration and Coma depending on the Shape Factor.(see p.412)

Calculate the correction of spherical aberration Y() and the tangential coma CT() = 3Cs() for fixed  and the range of  from –2 to 2.

Assume

Plot Y() and the tangential coma CT() in one graph. Observe that close to   =1 there is a point of no coma and minimum spherical aberration.

PA5. Coma of a thin Lens.(see p.415)

In Jenkins and White, fourth Ed, p.165 several lenses are shown with different shapes, spherical aberration and coma is given for  values from –3 to+2. Look at

FileFig.( A5COMAS) and consider three lenses, a plane-convex lens (  = -1), a bi-convex lens (  = 0), and a convex-plane lens (  = 1). Use  n = 1.5,  = -0.333, ro = 1.5 and f = 10  and fill in the table. Take for the spherical aberration Y( Sp)  from problem PA3, and go to eqs. 11.31 and 11.32 and calculate ZZ = W + G, as a measure of coma. Find the shape factor for zero Coma. (A lens with   = 1 is close to zero coma and has minimum spherical aberration).

r1	r2		ZZ	Y

inf	-5	-1

10	-10	0

5	inf	1

PA6. Calculation of Coma for the aplanatic Lens.(see p.417)

Consider a farsighted eye and assume that the focal length is f = 2.2cm and the distance from eye to retina d = 2cm. Design an aplanatic lens for correction, that is bringing the focal point of the combination of eye and correcting lens to the retina.

PA7. Astigmatism of a thin Lens.(see p.419)

Go to FileFig.(A8ASTISMSP) and plot the astigmatic difference ASD depending on the angle  = 1 to 10 degrees , for xo = - 400  and n = 1.5. Consider the 3 types of lenses discussed for spherical aberration and coma and fill in the table

Are there any changes when changing xo or n ?

PA8. The achromatic Doublet. Determination of Radii of Curvature and Focal Length.(see p.421)

Go to FileFig.(A9ACROMS) and calculate the radii of curvature for an achromatic double.

a. Assume values for nB, nR, and nD, for one and two and calculate V1 and V2. Choose f1 and calculate f2 and f.

b. A doublet of two lenses should have flat surfaces in the middle. The doublet should have the final focal length f = 50cm. Use the V values of FileFig.(A9ACROMS) and calculate r1 and r4.

PA9. The achromatic Doublet. Determination of Materials and Focal Length.(see p.422)

Do the calculation of PA8 for a chosen wavelength range in the visible and for f = 15cm. Use two different materials. The corresponding refractive indices may be obtained from Handbooks or from Jenkins and White, p.177. Find f1 .