Chapter 10. Imaging using Wave Theory.

PW1. Fourier Series.(see p.370)

Go to FileFig.(W1FTSERIS) and change the following settings

such that one has for g0(x) one maximum, g1(x) three maxima and g2(x) five maxima.

Observe the generation of a pulse function for N = 1, 2, 4, 8, 16 and so on until one gets a nice picture. At what N do we first see the Gibb’s phenomena.

PW2. Two Bars.(see p.381)

Go to FileFig.(W4IMTWOB) and consider an object made of two bars. The image is obtained by convolution of each object function with the spread function. The spread function is the Fourier transformation of the pulse, representing the cylindrical lens.

a. Change the distance between the two object bars and study “resolution”. If the distance is too small, the image is just one peak.

b. Change the calculation and apply the spread function to the total image over a large range of integration. The image is now “more similar” to the object.

PW3. Two round Objects.(see p.383)

Go to FileFig.(W6IMTWORO) and consider two round objects and a circular lens. The image is obtained by convolution of each object function with the spread function. The spread function is the Fourier transformation of the pulse, representing the lens.

a. Change the distance between the two object bars and study “resolution”. If the distance is too small, the image is just one peak.

b. Change the calculation and apply the spread function to the total image over a large range of integration. The image is now “closer” to the object.

PW4. Demonstration of the use of Spread Function (siny/y)2 as a Transfer Function.(see p.386)

Go to FileFig.(W8TRASI) and consider an object of a grid. The Fourier transformation of the object is shown and the transfer function, which is the Fourier transformation of the spread function (siny/y)2. The product of the Fourier transformation of the object and the transfer function is called  . The inverse Fourier transformation of   is the result of the imaging process and should look more or less like the object.

Change the f# and observe through the changing triangle that more or less frequencies are used for image formation.

PW5. Demonstration of the use of Spread Function (J1(y)/y)2 as a Transfer Function.(see p.386)

Go to FileFig.(W9TRAJ1) and consider an object of a grid. The Fourier transformation of the object is shown and the transfer function, which is the Fourier transformation of the spread function (J1(y)/y)2. The product of the Fourier transformation of the object and the transfer function is called . The inverse Fourier transformation of   is the result of the imaging process and should look more or less like the object.

Change the f# and observe through the changing transfer function that more or less frequencies are used for image formation.

PW6. Rayleigh Distance and Resolution with incoherent Light.(see p.389)

Go to FileFig.(W11TWOROJ1)  and consider two round objects at diameter 0.0005 and center to center distance of 0.0061. The convolution with the spread function (J1(y)/y)2  results in a resolved image.

a. Read off the graph the distance of the peaks of the image.

b. Change the distance between the objects so that the two are just not resolved.

c. Change the distance between the objects so that the two are just separated.

d. Increase the size of the object until they are no longer resolved.

PW7. Rayleigh Distance and Resolution with coherent Light.(see p.392)

Go to FileFig.(W12TWOROCOH) and consider two round objects at diameter 0.0005 and center to center distance of 0.0061. The convolution with the spread function (J1(y)/y) {not squared} results in an image of one peak, not resolved.

Introduce the center to center distance of 0.0082 between the two objects, b3 = 0.00795 and b4 = 0 .00845 and show that the two objects appear resolved.

PW8. Transfer Function for (sinx/x) of the Coherent Case.(see p.394)

Go to FileFig.(W13TRANCOHSI) and consider a grid as an object. The Fourier transformation ω of the object is shown and the transfer function, which is the Fourier transformation τ of the spread function (siny/y) {not squared}. The product of the Fourier transformation ω of the object and the transfer function τ is called  . The inverse Fourier transformation of  is the result of the imaging process and should look more or less like the object.

a. Change the f#, that is changing the width of the spread function and observe that more or less frequencies are used for image formation.

b. Compare the incoherent and coherent case for the same f#. Choose one large and one smaller  f#.

PW9. Transfer Function for (Bess/arg) of the Coherent Case.(see p.394)

Go to FileFig.(W14TRANJ1) and consider a grid as an object. The Fourier transformation ω of the object is shown and the transfer function, which is the Fourier transformation τ of the spread function (Bess/arg) {not squared}. The product of the Fourier transformation ω of the object and the transfer function τ is called  . The inverse Fourier transformation of  is the result of the imaging process and should look more or less like the object.

a. Change the f#, that is changing the width of the spread function and observe that more or less frequencies are used for image formation.

b. Compare the incoherent and coherent case for the same f#, choose one large and one smaller f#.

PW10. Blocking Function for removing high Frequencies.((see p.399)

Go to FileFig.(W15HOGRBLHI) and consider a grid as an object. The transfer function is removing certain higher frequencies of the first Fourier transformation. The extend of the blocking function depends on the width parameter a.

Observe the changes of the final image by modification of the blocking function, that is changing a.

PW11. Blocking Function passing a band of Frequencies.(see p.399)

Go to FileFig.(W16HOGRBLLO) and consider a grid as an object. The transfer function is a blocking function passing just one portion of the frequencies of the first Fourier transformation. The extend of the blocking function depends on the parameters n and a.

Observe the changes of the final image by modification of the blocking function, that is changing n and a.

PW12. Blocking Function passing periodic portion of all Frequencies.(see p.400)

Go to FileFig.(W17HOGRPER) and consider a grid as an object. The transfer function is a grid type blocking function, blocking off periodic parts of the first Fourier transformation. The width of the peaks and the extend of the blocking function depends on q and a.

Observe the changes of the final image by modification of the blocking function, that is changing q and a.

PW13. Size of Hologram and Quality of Image.(see p.402)

Go to FileFig.(W18HOSTEPCOM) and consider the object as a composition of step functions. The hologram corresponds to the first Fourier transformation. The transfer function passes all lowest frequencies, positioned on both sides of the hologram. The product (hologram times filter) is the resulting image. The number of passing lowest frequencies is monitored by the parameters a  and b. The size of the hologram corresponds to the amount of low frequencies passing. Observe the changes of the “size of the hologram” when making the range of passing low frequencies larger or smaller.