EPRLL VERSION 1.6b UPDATE

David E. Budil

Department of Chemistry

Cornell University

Oct., 1994

This file describes the cumulative changes through Version 1.6b that

have been made in EPRLL and related programs since the publication of

Version 1.0 by D.J. Schneider and J.H. Freed in Chapter 1 of

Biological Magnetic Resonance, Vol. 8, Spin Labeling (L.J. Berliner

and J. Reuben, eds., Plenum Press, 1989)

The suffix "L" in the program names refers to the "Limited" basis set

used to carry out the calculation. The EPRLL family of programs is a

subset of the more general EPRLF ("Full" basis set) programs

originally written by G. Moro [see Meirovitch et al., J. Chem. Phys.

77:3915 (1982) ]. The original EPRLL programs were somewhat less general

in that:

(1) they required the high field approximation

(2) only one tilt angle (e.g. between magnetic and diffusion tensors)

was allowed instead of all three Euler angles

(3) a nuclear Zeeman interaction could not be included

(4) the g- and A- tensors were assumed to coincide.

This sacrifice of generality permits a very substantial reduction of

the matrix dimension needed to carry out the calculation, making it

very efficient for running on small systems.

In Version 1.6 of EPRLL, the last three limitations listed above have

been eliminated. Thus, it is no longer necessary to use EPRLF in

cases where the g- and A- axes do not coincide, where more than one

tilt angle is needed to describe the relative orientation of the

magnetic vs. diffusion axes, or where the nuclear Zeeman interaction

is significant. EPRLL is now able to increase the basis set as needed

for a given problem, and reduces fully to the minimal basis set in

the absence of tilt angles and the nuclear Zeeman term. (EPRLF did

not reduce to the minimal set in this case; it always required a

basis set that was about a factor of two larger than the minimal

set). EPRLF is still required, however, when the high-field

approximation breaks down.

Implementation and use of the EPRLL programs is generally documented

in the Schneider and Freed book chapter cited above. What follows is

a specific list of changes in the way the programs operate from what

is described in the book chapter.

SUMMARY OF CHANGES IN VERSION 1.6b

Changes made to correct an error where in the absence of magnetic

or diffusion tilt, k and m may be negative.  This should not happen.

Also changes to use dabs on double precision variables are included.

These should not affect results significantly.

From the unix diff command (The lines with < are from the new version,

the lines following > are the old version:

Version 1.6b:

skeeve>diff matrll.f ftp/EPRLL/matrll.f

80,81c80,81

<       integer lr,lrprod,kr,mr,ipnr,iqnr,lc,ld,ldabs,ls,kc,kd,ks,jkr,

<      #        jkc,jmr,jmc,kdabs,ksabs,mc,md,ms,mdabs,msabs,ipnc,iqnc,

---

>       integer lr,lrprod,kr,mr,ipnr,iqnr,lc,ld,ls,kc,kd,ks,jkr,jkc,

>      #        jmr,jmc,kdabs,ksabs,mc,md,ms,mdabs,msabs,ipnc,iqnc,

239c239

<                if(dabs(pl).gt.RNDOFF) c2=c2**pl

---

>                if(abs(pl).gt.RNDOFF) c2=c2**pl

250c250

<                if(dabs(pkzz).gt.RNDOFF) c4=c4**pkzz

---

>                if(abs(pkzz).gt.RNDOFF) c4=c4**pkzz

253c253

<                if(dabs(pkxy).gt.RNDOFF) c4=c4**pkxy

---

>                if(abs(pkxy).gt.RNDOFF) c4=c4**pkxy

318d317

<                ldabs=abs(ld)

478,480c477,479

<                if((ipt.ne.0).and.(ldabs.le.lband).and.(md.eq.0).and.

<      #            ((kdabs.le.kband).or.(ksabs.le.kband)).and.

<      #            (ipar(ks).eq.1).and.(jkd.eq.0)) then

---

>                if((md.eq.0).and.(ipt.ne.0).and.(ipar(ks).eq.1)

>      #              .and. ((kdabs.le.kband).or.(ksabs.le.kband))

>      #              .and. jkd.eq.0) then

492,493c491

<      #                   cdffu=cjkc*cplkc*xlk(li,ksi)*

<      #                         w3j(lr,l,lc,kr,-ks,kc)

---

>      #                   cdffu=cplkc*xlk(li,ksi)*w3j(lr,l,lc,kr,-ks,kc)

SUMMARY OF MAJOR NEW FEATURES

Version 1.6

1) The program now includes the following new parameters:

(a) "Magnetic tilt" between the g- and A- tensors, specified by

the three Euler angles alm, bem, and gam

(b) Full "diffusion tilt"; i.e., the ability to specify the

tilt between the g- and diffusion tensors with a full set

of three Euler angles, ald, bed, and gad (ald and gad are

new parameters)

(c) Nuclear Zeeman interaction, specified by the nuclear gyro-

magnetic ratio, gamman.

2) A new program, BSSL, has been included which provides an ASCII

listing of the basis set information contained in the <filename>.bss

file.

Version 1.5

1) The way in which inhomogeneous linewidth is added to the spectral

lineshape has been modified and made more flexible. There are now

two ways to specify inhomogeneous linewidth: The first is the "W"

tensor, which has the normal Cartesian components Wxx, Wyy, Wzz,

and is set in LBLL. This specifies an orientation-dependent Lorentzian

linewidth that is related to the molecular (g-tensor) frame. The

second type of linewidth is Gaussian inhomogeneous broadening, which

is also orientation-dependent, but is related to the director frame. It

is specified by the parameters gib0 (isotropic term) and gib2 either

in the LBLL program, or in TDLL (see below).

Version 1.4

1) All calculations now have the capability of using a basis set "pruning"

method which dramatically enhances calculation speed for repetitive

calculations with similar magnetic and dynamic parameters. Two examples

of this type of calculation are the MOMD calculation described below and

nonlinear least-squares spectral fitting (not available in this package).

2) Calculations based on the MOMD (Microscopic Order Macroscopic Disorder)

model are now available as part of the program set. The MOMD calculation

was originally described by Meirovitch et. al [J. Phys. Chem. 88, 3454

(1984). ] It assumes microscopic molecular ordering with respect to a

local director that is itself randomly oriented, and is particularly

useful for samples consisting of membrane dispersions.

Version 1.3

1) The programs now accommodate a fully anisotropic rotational diffusion

tensor with Dx .ne. Dy .ne. Dz, in place of the axially symmetric

diffusion tensor (Dxy, Dzz) used in previous versions.  This feature

has proven useful for studying motion using very high-freqency EPR

(250 GHz). The theoretical expressions needed for this update are

given in Appendix A of [K.A.Earle, D.E. Budil, and J.H. Freed,

J.Phys.Chem., 97:13289 (1993) ]

Versions 1.1 and  1.2

1)   A new matrix storage scheme has been introduced that reduces by

50% the required storage for off-diagonal matrix elements.

2)   The capability of calculating a first-derivative spectrum directly

from the continued-fraction expression has been added to the TDLL

program.

3)   The convention for naming files associated with a given calculate

has been updated in Version 1.2. See "User Interface" below for

details.

4)   The updates to this program include several improvements in the user

interface (detailed below) that make input of the calculation

parameters easier and output of the programs more informative. Among

these are a re-write of the user prompt routine, adaptations for

specifying filenames and options switches on the command line for

operating under UNIX, updates of the routines that output the matrix

and starting vector elements, and an update of the calculation of order

parameters from orienting potential coefficients.

CHANGES IN THE USER INTERFACE

This section details the important changes in the operation of the EPRLL

family programs in Versions 1.1 through Version 1.5. In general, features

described in an earlier version are also retained in later versions, except

as noted.

Version 1.6

1)   The LBLL program now contains an input query for the magnetic tilt

angles alm, bem, and gam (alpha-, beta-, and gamma-magnetic).

These specify the rotation that takes the g-tensor axes into the

A-tensor axes, according to the following series of operations:

(i)   a rotation about gz through the angle gamma

(ii)  a rotation about gy through the angle beta

(iii) a rotation about gz through the angle alpha

A positive angle produces a counterclockwise rotation when viewed

along the axis of rotation (looking from the + end).

Similarly, the angles ald, bed, and gad (alpha-, beta-, and gamma-

diffusion) specify the rotations that take the diffusion axes

into the g-tensor frame, according to the same definitions given

above.

It should be noted that, according to these definitions, the alpha

angle is not required if the tensor(s) in the reference frame is(are)

axially symmetric. LBLL recognizes such cases and resets the alpha

angle.

The final floating-point parameter requested by LBLL is gamman,

which should be input in units of radians/(sec*gauss).

These new physical quantities require two additions to the integer

parameters used to specify the basis set. The new parameters are

kmn and mmn, and they correspond to the minimum K and M values to

be used in the basis set. They are expected to be less than or

equal to zero. (In versions 1.5 and earlier, kmn and mmn were

implicitly zero). In general, a nonzero kmn is required only when

one of the set {ald,gad,alm,gam} is nonzero, whereas a nonzero

mmn is required only in the presence of a nuclear Zeeman interaction.

It is not necessary that |kmn| be equal to |kmx| (and similarly

for |mmn| and |mmx|, but |kmn| and |mmn| must both be less than

or equal to lemx. In fact, for reasonably small tilt angles, |kmn|

may be significantly smaller than |kmx|, so that the additional

tilt angles may be included without a significant penalty in

basis set size. Similarly, since the nuclear Zeeman is often

relatively small, it may be possible to choose |mmn| much smaller

than |mmx|. The best method to determine these new truncation

indices for a given problem is still to use the swept-field

calculation in EPRBL, which now accommodates negative K and M

indices.

2)   The BSSL program simply accepts the name of an existing basis set

weighting file (<name>.bss, produced by EPRBL) and outputs it

to the screen. The information can be captured by using the DOS

or UNIX redirection feature, as follows:

bssl sample > sample.info

Version 1.5

The LBLL program now contains an input query for the W tensor, which

specifies the Lorentzian inhomogeneous linewidth tensor that is related

to the molecular (g-tensor) frame. Units are Gauss (derivative peak-to-

peak linewidth).

In the case of a MOMD calculation, LBLL also requests the parameters

gib0 and gib2, which specify the Gaussian inhomogeneous width related

to the director frame, in units of Gauss (derivative peak-to-peak

width). The linewidth as a function of tilt angle psi is given by

gib0 + sin(psi)*gib2

For non-MOMD calculations, Gaussian broadening may be added to the

spectrum by specifying gib0 and/or gib2 in program TDLL. The broaden-

ing added to the spectrum is calculated for the given tilt angle

as in the above equation.

Version 1.4

1)   EPRBL program and basis pruning

The field-swept conjugate gradient programs used to determine an

appropriate basis set for a given calculation (i.e., program EPRBL)

has changed substantially from its implementation in previous versions.

Each element of the basis set used by an EPR calculation is specified

by five quantum numbers: L,K,M,pI,and qI. Through Version 1.3, the

basis sets used by EPRLL family programs were specified by five

"truncation parameters", lemx, lomx, kmx, mmx, and ipnmx, that specify

the maximum values that each quantum number may assume (separate maxima

are specified for even and odd L values, and none is needed for qI).

When these parameters are set to the minimum necessary for acceptable

convergence of the calculated spectrum, they are referred to as

Minimum Truncation Set (MTS) parameters. It is highly desirable to limit

the basis for an EPRLL family calculation to the MTS wherever possible

to optimize its performance.

The previous strategy for optimizing a given calculation was to find

the MTS using the field-swept conjugate gradients method provided by the

EPRBL program. Briefly, EPRBL determined a list of "significance

factors" for each vector in a basis set with truncation parameters

chosen slightly larger than the anticipated MTS. The significance factor

for a given basis vector is the maximum of the projection of that vector

on the solution vector, evaluated at a selection of fields across the

spectrum. Each significance factor and the field corresponding to the

point in the spectrum where the maximum projection was found are stored

in the file <fileid>.bss.

After EPRBL was run, the MTS could then determined using program TNLL.

Basically, TNLL chose a series of minimum tolerances used to define

a "significant" basis vector, and for each tolerance searched through the

basis to find the largest value of each index for the set of vectors

that satisfied the significance criterion. Using this method, Schneider

[Ph.D. Dissertation, Cornell University; see also Schneider and Freed

(1989) and  Vasavada et al., will beJ. Chem. Phys. 86, 647 (1987).]

derived some simple empirical rules for estimating the appropriate MTS

parameters for a given set of dynamical parameters.

However, practical experience has shown that significant further

reduction of the size of the basis below that of the MTS is possible.

That is, not every member of a MTS is needed for a given calculation.

However, the pattern of "extinctions" is rather irregular and cannot

easily be formulated as a simple set of rules. The alternate strategy

implemented in program EPRBL is simply to keep a list of all the relevant

basis indices. The list is first built to include the full truncation set

specified by the truncation indices in the <fileID>.par file, and to

delete any vector from that set that falls below the "minimum

significance" tolerance. This procedure will be  referred to as

"basis set pruning".

The Version 1.4 EPRBL program has been modified to perform the pruning

function. Specifically, it takes the place of both the EPRBL and the TNLL

programs in previous versions. As before, EPRBL carries out a field-swept

conjugate gradient calculation to determine a set of "significance

factors" which it writes into the file <fileID>.bss. In addition,

it will produce a basis index file, <fileID>.ind with lists of the

indices of basis vectors whose significance factors are greater than

the cutoff value, "btol", specified in the parameter file. Subsequent

calculations using EPRLL, EPRCGL, MOMDL, or EPRBL will use this list

as the basis set.

The new EPRBL also handles the case of MOMD calculations. In this

case, many spectra need to be calculated at different director

tilt angles, each of which may correspond to a different basis set.

To account for this, EPRBL performs a swept-field calculation for

a number of tilt angles specified by the "nort" parameter (see below)

and combines the results. The <fileID>.bss file produced for a MOMD

calculation contains the maximum projection of a given basis vector

for all fields and tilt angles calculated. This calculation is often

very time-consuming.

CAVEAT: It is possible to "over-prune" the basis to the point where

it works only for a very narrow range of physical parameters. In

addition, the symptoms of an insufficient basis set are often somewhat

less apparent than in the case where the truncation parameters are

too small. It is advisable to check the calculation for a couple of

different values of the pruning tolerance "btol". So that the pruned

basis may be changed without repeating an entire EPRBL calculation,

the PRUNEL program has been supplied (see below).

2)   PRUNEL program

The PRUNEL program is used to generate a new pruned basis set in

file <fileid>.ind starting from the basis projection file <fileid>.bss

using the pruning tolerance "btol" specified in the <fileid>.par file.

To use the program, simply reset "btol" to the desired value using LBLL,

and invoke PRUNEL with the desired filename(s).

3)   MOMDL program

The MOMDL program performs an integration of the spectrum over the

director tilt angle, "psi", from zero to pi/2. The integration method

is a simple trapezoidal rule using a number of different orientations

specified by the "nort" parameter (see LBLL documentation below).

The program usually integrates by taking steps equally spaced in

cos(psi); however if the +a switch is activated, it will take steps

that are equally spaced in psi and weight them by sin(psi). MOMDL

produces the spectrum, averaged over orientation, directly into the

output file <fileID>.spc. For this reason, it is necessary to specify

several parameters in the LBLL program related to the output spectrum,

including "fieldi", "fieldf", and  "nfield"  for the first and last

field points and the number of field points in the spectrum, and

"ideriv" to specify either 0th or 1st derivative. These are normally

specified in the TDLL program for non-MOMD spectra.

For "nort">1, MOMDL will also produce an output spectrum for each

individual tilt angle if the +s switch is activated. The files will

be named <fileID>.001, <fileID>.002, ... up to "nort".

NOTE: The most frequent problem encountered in the MOMD spectrum

is including an insufficient number of director tilt angles (i.e.

specifying "nort" too small). This can produce "oscillations" in

the spectrum that have the same qualitative appearance as those

produced by an insufficient basis set. The problem can be overcome

simply by including more angles. In our experience, as few as ten

or as many as fifty angles may be needed. In general, more angles

may be needed for (1) smaller inhomogeneous linewidths, (2) higher

local ordering, or (3) slower motions. Unfortunately, cases which

require more angles also tend to require larger basis sets, so it

is often very profitable to use pruned basis sets.

4)   LBLL program

Two parameter have been added to the LBLL program that are related to

the new capabilities for basis set pruning and MOMD calculations:

nort   - Number of orientations (different values of the director

tilt angle psi) over which to integrate in a MOMD calculation.

nort=1 for non-MOMD calculations. For intermediate slow motion

with weak ordering, a value of 10 is usually suitable. Higher

ordering in the presence of either fast or very slow motion

(where the linewidths are relatively narrow) require more;

in our experience, no more than 30 are required at X-band.

btol   - Pruning tolerance for EPRBL and PRUNEL programs (see below).

Typical values suitable for calculation of cw-EPR spectra

are 0.01 to 0.03. A btol of zero specifies the entire

truncation set. Surprisingly, similar values for btol have

proved sufficient for calculating 2D-FT-EPR spectra, in contrast

with the conclusions of Vasavada et al.

For the case of a MOMD calculation, LBLL assumes itype=1 (that is,

MOMD calculations are always carried out using conjugate gradients

tridiagonalization). In addition, for MOMD calculations, LBLL prompts

the user for the fieldi,fieldf,nfield, and ideriv parameters. These are

normally specified for field-swept CG calculations (EPRBL) or in the

spectral calculation carried out by TDLL. They are needed for MOMDL

since this program produces a <name>.spc file directly instead of

the <name>.tdl file.

5)   Calculation switches

The manner in which switches are specified to and interpreted by

the programs has been changed slightly, and some new switches have

been added.

Currently active switches are:

d : reset to the default flags: -r -m -v -s -a

+r : read the matrix and vector elements from old files

rather than calculate again.  If the file doesn't

exist, it calculates those elements.

+m : store the matrix elements into file with extension

<.mtx>.

+v : store the vector elements into file with extension

<.stv>.

+s : save the spectra calculated for individual orientations

in a MOMD calculation, in files <name>.001, <name.002> ...

+a : distribute the MOMD orientations using constant angle step

( default is constant cos(angle) )

4)   File names

The file naming convention follows that of Version 1.2 (see below).

One new files type has been added since Version 1.3: <name>.ind,

which is a list of the indices associated with a pruned basis set.

The following list summarizes the various possible file types and

indicates the program units for which they serve as input files (I),

output files (O), or both (B):

L  E  E  E  M  P  T

B  P  P  P  O  R  D

Program Name  -->         L  R  R  R  M  U  L

L  L  C  B  D  N  L

L  G  L  L  E

L        L

<fileid>.PAR   Parameter file (unformatted)          O  I  I  I  I  I  I

<fileid>.FMT   Parameter listing (formatted)         O  O  O  O  -  -  -

<fileid>.SPC   Calculated spectrum                   -  -  -  -  O  -  O

<fileid>.LOG   Log file for calculation              -  O  O  O  O  -  -

<fileid>.TDL   Tridiagonal SLE matrix (unformatted)  -  O  O  O  -  -  I

<fileid>.TDF   Tridiagonal SLE matrix (formatted)    -  -  -  -  -  -  O

<fileid>.MTS   MTS database file                     -  -  -  O  -  -  -

<fileid>.MTX   SLE matrix (unformatted)              -  O  O  O  -  -  -

<fileid>.MTF   SLE matrix (formatted LMATRX output)  -  -  -  -  -  -  -

<fileid>.EGV   SLE eigenvalues/weights               -  -  -  -  -  -  O

<fileid>.STV   Starting vector (unformatted)         -  O  O  O  -  -  -

<fileid>.BSS   Maximum basis vector projections      -  -  -  O  -  I  -

<fileid>.IND   Basis set index list                  -  I  I  B  I  O  -

Version 1.3

1)   LBLL program

The LBLL program now expects three values for the diffusion tensor,

DX, DY, and DZ, instead of DXY and DZZ. This permits the diffusion

tensor to be fully anisotropic.

Version 1.2:

1)   LBLL program

The tedious "Do you want new values?" query has been

eliminated from the user interface. In Version 1.2, if the

user wishes new values, they should simply be typed in at the

"Enter new value(s)" prompt. If new values are *not* desired,

the user may simply type <ENTER> and the displayed

parameter(s) will not be altered. If fewer than the expected

number of values are entered, the remainder of the values are

assumed to be zero.

2)   TDLL program

Three new prompts appear in the TDLL program. The first two allow

new flexibility in calculating experimental spectra from the

continued-fraction expression. The program now includes

options to specify direct calculation of the first-derivative

spectrum, and a specification for the number of points in the

calculated spectrum. The third prompt is for an option to sort

the eigenvalues of the matrix by weighting coefficient rather

than by field.

3)   LMATRX program

The MATLST program has been replaced by LMATRX, which performs

essentially the same function, but includes a list of the bra

and ket basis set indices with each matrix element. The program

takes file <fileID>.mtx as input and produces formatted output

in <fileID>.mtf.

4)   LVECTR program

The STVT program has been replaced by LVECTR, which performs

the same function. The program does not produce an output file,

but prints the listing of the starting vector matrix elements

directly to the screen (standard output). This may be caputured

using output redirection in either DOS or a UNIX environment.

5)   ORDER program

The D200 program has been replaced by ORDER. Program ORDER

calculates an order parameter corresponding to each of the

coefficients in the potential, i.e. it calculates the

expectation value of the L,K spherical harmonic <Y^L_K> for

each C(L,K) in the potential. The program D200 only performed

this function for L=2,K=0.

Upon entry to program ORDER, the user is prompted for the

potential coefficients. The coefficients should be specified

in a list separated by spaces (or commas) in the order c20,

c22, c40, c42, c44. The user may specify only as many

coefficients as needed. For example, for a potential

consisting of the c20 and c22 terms, only two numbers need be

entered on the input line, and the program will only calculate

two order parameters. However, all coefficients must be

specified up to the highest-order coefficient desired. For

example, in order to specify a potential with zero c20 or c22

but a nonzero c40, one has to enter zeroes explicitly for both

c20 and c22 on the input line. The program will calculate three

order parameters in this case.

6)   Filename convention

The convention for naming EPRLL calculation files has been changed.

Whereas Version 1.0 used a two-character file identifier, the

file I.D. in Version 1.2 may be specified by any string of

legal characters up to an arbitrary limit of 30 characters or

the limit permitted by the system, whichever is smaller.

Filenames are all constructed in the form <fileid>.<ext>,

where "ext" indicates the specific type of EPRLL file. See list

above for the various possible file types and their meanings.

7)   Specification of fileIDs on UNIX command line

Like the Version 1.0 programs, the current set of programs all accept

a list of file ID's specifying calculations to be carried out in a

single program run. This feature makes it convenient to set up several

long calculations for an overnight run on a PC or a single batch job,

for example. For running the programs in a UNIX environment, the

commented-out code between the dotted lines in GETIDS.F may be restored

so that the file IDs may be specified directly on the command line.

Under DOS, the user is prompted for filenames in the sequence until a

blank line is entered.

8)   Option switches for UNIX command line

Certain options may be specified either as elements of the input

filename list or on the command line under UNIX. Four switches may be

specified in EPRLL and EPRCGL that toggle flags for storage of the

matrix and starting vector into binary files that may subsequently

be printed out by the LMATRX and LVECTR programs (see below). To turn

on matrix or vector storage, specify "+m" or "+v" in place of a file ID.

The matrix or vector calculated for all file ID's appearing later in

the list will be stored. Storage may be turned off by the "-m" and "-v"

switches. In the present implementation, each switch must be given as a

separate list element. The following give examples of how the switches

are used. The command and dialog below specify that matrix files named

calc1.mtx and calc2.mtx are to be output for each of the first two

calculations, but not for calc3.

UNIX command line:

> eprll +m calc1 calc2 -m calc3

Under DOS (or under UNIX when no command line arguments are given):

> eprll

######################################################################

EPRLL Version 1.5

==================

Please enter file identifier (<CR> to end): +m

Please enter file identifier (<CR> to end): calc1

Please enter file identifier (<CR> to end): calc2

Please enter file identifier (<CR> to end): -m

Please enter file identifier (<CR> to end): calc3

Please enter file identifier (<CR> to end):

(...program starts calc1...)

Other aspects of running the programs should work the same way as

documented in the Schneider and Freed book chapter cited above.

CHANGES IN COMPUTATION ALGORITHM

In addition to these changes in the user interface, there are some

program improvements that are mainly transparent to the user.

Version 1.6

This section is intended for users who need to know the

specific details of the basis set differences between EPRLF and

EPRLL. We consider first the basis set used in EPRLF. This can be

represented as |L K M pS qS pI qI>, where L,K, and M are the

quantum numbers corresponding to a set of generalized spherical

harmonics (or Wigner rotation matrix elements) and the p's and q's

are the transition indices for the electronic (S) and nuclear (I)

spins. In the general case, for electron spin 1/2 and a nuclear

spin I, the quantum numbers can take on the following values:

L    0 to Lmax

K    -L to +L

M    -L to +L

pS   -1 to +1

qS   -1+|pS|, 1+|pS|, ..., 1-|pS|

pI   -2I to +2I

qI   -2I+|pI|, -2I+|pI|+2, ..., 2I-|pI|

EPRLL takes advantage of two "symmetrizations" of this basis

set with respect to the K and M quantum numbers. By

"symmetrization", we mean a transformation that defines two new

basis functions consisting of symmetric and antisymmetric

combinations of each pair of original basis functions having

positive and negative values of the specified quantum number. That

is, for K-symmetrization,

|L �1 K' M ...> = sqrt(+/-1) [ |L K' M ...>

+/- (-1)^(L+K') |L -K' M ...> ]

where the index K' now only assumes values from 0 to L, and we have

introduced a new index, jK, which can assume the values +1 or -1

corresponding to the symmetric or antisymmetric combinations. (Note

that the above equation omits a normalization factor for purposes

of clarity.)

A similar transformation can be written for the M quantum number.

Because of the relationship between this quantum number and the

spin transition indices (see Meirovitch et al, 1982), the pS and pI

quantum numbers must also change sign with the M index in this

transformation:

|...�1 M' pS' qS pI' qI> =

[ |...L K M' pS' qS pI' qI>

+/- (-1)^(L+M') |...-M -pS' qS -pI' qI> ]

Here the new index jM=+/-1 indicates the symmetric/antisymmetric

combinations, and the new indices M', pS', and pI' can assume only

nonnegative values (Note that this transformation is also not

normalized for clarity).

The K-transformation is used by both EPRLF and EPRLL since it

produces a complex-symmetric stochastic Liouville matrix. This

feature is required in order to take advantage of the efficient

Lanczos algorithms for solution of the SLE. However, the jK=-1

elements vanish in the case that (i) the magnetic tensors have the

same principal axes and (ii) the rotation between the diffusion

axes and the magnetic axes consists at most of the polar Euler

angle (i.e. the Euler angles specifying the diffusion axes in the

magnetic frame are (0,beta,0) where beta may also be zero). In

versions previous to 1.6, EPRLL restricted the calculation to these

specific cases, so that the jK=-1 manifold could be dropped from

the calculation.

In version 1.6, the jK quantum number has been reintroduced. Since

K could previously only assume non-negative values, and jK=+1 was

assumed, it is possible to encode jK using the sign of the K

quantum number. (For K=0, jK is given by the parity of L). Thus,

negative K-values actually correspond to basis vectors with jK=-1,

and K>0 in the above definition of K-symmetrization.

The M-transformation is used only by EPRLL. In the absence of

a nuclear Zeeman interaction, the spin Hamiltonian is symmetric

with respect to a change in the signs of pS and pI, and the jM=-1

terms vanish. EPRLL takes advantage of this fact and neglects the

nuclear Zeeman interaction in order to drop the jM=-1 basis

functions. However, it is also possible to re-include the anti-

symmetric M-space and encode jM using the signs of M and pI.

(jM takes the sign of M. If M=0, jM takes the sign of pI.

If M=pI=0, jM is determined by the parity of L).

Version 1.5

Programs MOMDL and TDLL now carry out the Gaussian line broadening by

convolution with a Gaussian function using a fast Fourier transform.

Subroutine MATRLL incorporates the molecular frame-dependent width

by adding in real elements to the stochastic Liouville matrix.

Version 1.4

1)   The method of matrix and starting vector calculation has been

adapted to accommodate the new basis pruning scheme. Specifically,

everywhere a series of five nested loops over each of the basis

indices appeared in the old programs a single loop over the basis

set dimension has been substituted. This permits unnecessary basis

vectors to be removed from the calculation without the need for

any selection rules.

Since a "pruned" set cannot be determined by selection rules,

this change requires that the basis set indices be kept in five

lists (common /indexl/). A new routine, LBASIX, has been added

that either reads a list in from a pre-existing <name>.ind file,

or builds the lists for a given set of truncation indices.

Version 1.3

1)   The MATRLL program has been updated to calculate matrix elements

for a fully anisotropic rotational diffusion tensor. This modification

is fairly trivial in the case of isotropic media, but somewhat more

complicated in the presence of an orienting potential in anisotropic

fluids.

2)   The TDLL program now utilizes a more efficient continued-fraction

algorithm provided by D.J. Schneider for both the zeroth and first

derivative EPR spectrum, thus avoiding possible artifacts that can

arise from numerical differentiation of the spectrum.

Version 1.2

1)   The stochastic Liouville matrix storage convention has been

changed to take advantage of its complex symmetric property.

In version 1.2, only half of the matrix is stored, thus

permitting calculations with larger basis sets on systems with

limited memory resources.

2)   The TDLL program now utilizes a continued-fraction expression

for directly calculating the first derivative EPR spectrum, thus

avoiding possible artifacts that can arise from numerical

differentiation of the spectrum.

3)   The EPRBL program has been updated to avoid problems observed

in version 1.1 for calculations in the case of a strong

orienting potential. In versions 1.0 and 1.1, the

preconditioned-CG method often fails to converge in this case,

presumably because the real part of the SLE matrix has

significant off-diagonal elements in the presence of a

potential.

4)   An alternative package to the subroutines in W3J.F is

available in file W3JBIG.F. The alternative package uses a

different calculation algorithm based on a method of Roothan

for calculating 3J symbols in the case that the J2 value is

greater than 2. The new algorithm has the advantage that it is

more efficient and permits larger L values in basis sets for

calculations involving an orienting potential. However, it has

the drawback that it requires the calculation and storage of

a large array of binomial coefficients. This feature makes it

impracticable for the PC implementation of EPRLL, but if the

programs are to be ported to a mainframe, we recommend

substitution of W3JBIG.F for W3J.F. The calling sequence for

function w3j from the main programs is exactly the same for

W3J.F and W3JBIG.F. The routines in W3JBIG.F also require file

BINCOM.INC.

LIST OF PROGRAM SUBROUTINE CHANGES

For advanced programmers and other EPRLL connaisseurs, the following list

briefly indicates the major program modifications from Version 1.0,

including those changes that first appeared in Versions 1.1 through 1.4:

Many trivial changes have not been noted below.

Version 1.5

GCONVL.FOR     has been added to carry out the convolution of a (complex)

spectrum with a Gaussian lineshape.

FFT.FOR        is a discrete fast Fourier transform routine used in the

convolution.

Version 1.4

EPRBL.FOR     has been substantially modifed from Version 1.3. In fact, this

is now mainly a driver routine that calls the subroutines

EPRFSL.F and MTSL.F, which now carry out the functions

previously coded in EPRBL.F and TNLL.F. In addition, in the

case of a MOMD calculation, it will carry out swept-field

calculations for the series of orientation angles and combine

the results in a single pruned basis file.

EPRCGL.FOR    A call to the subroutine LBASIX is now required before the

call to MATRLL in each case.

EPRFSL.FOR    is a new routine the carries out the swept-field CG calculation

previously coded in program EPRBL.F. It is designed to be called

repetitively by EPRBL.F in the case of MOMD calculations.

GETIDS.FOR    has been slightly modified so that it doesn't count

switches as filenames, an irritating feature of Version 1.3.

MATRLL.FOR    has been modified to use the (potentially pruned) basis

set stored in common block INDEXL rather than looping

through the indices of the truncation basis set. The

structure of the subroutine now consists of only two loops,

one over column index and one over row index, in place of

the multiple loop structure in Version 1.3 MATRLL.

To avoid repetitive calculations, the indices are checked

within each loop to determine which have changed. Oddly,

this routine is actually somewhat slower than the non-pruning

MATRLL for the full truncation set, but dramatically faster

once pruning has been implemented.

LBASIX.FOR    is used to load a list of basis set indices into common

block INDEXL. It contains the multiple loop structure

previously found in MATRLL, STVECT, LMATRX, LVECTR, and

WRLBAS. It requires the fileID string to be passed as an

argument so that it may search for a pruned basis set

in the <fileID.ind> file. If this file is found, the index

arrays are read from the file; otherwise, LBASIX loops

through and builds index arrays for the full truncation set.

LMATRX.F and  have been modified to use the basis set stored in common

LVECTR.FOR   INDEXL rather than looping through the basis set indices.

PRUNEL.FOR    is a new program that can be used to modify the pruned

basis set according to a different tolerance level once

a <fileID>.bss file is available from EPRBL.

MOMDL.FOR     is a new program that utilizes the EPRCGL calculation

to calculate a series of spectra at different tilt angles

and integrates over tilt angle orientation.

MTSL.FOR      replaces the basic function of the old TNLL program.

It is now coded as a subroutine to be called by EPRBL.

SETFLG.FOR    is a new routine used to set the switches specifying

options for the EPRLL, EPRCGL, EPRBL, and MOMDL programs

from tokens on the command line (or elements of the

input list, for non-UNIX implementations)

STVECT.FOR    has been modified to use the basis set stored in common

INDEXL rather than looping through the basis set indices.

TNLL.FOR      has been eliminated. Its function is replaced by the new

EPRBL program.

WRLBAS.FOR    has been modified to use the basis set stored in common

INDEXL rather than looping through the basis set indices.

This is the new name for the original WRBAS.F routine.

INDEXL.INC    a new common block that contains a list of each

basis vector index for every element of the basis set.

The lists must be set up by calling the LBASIX routine

before performing any operation on the SLE matrix or

starting vector.

EPRDAT.INC    The variables neltot,nelv,nelim,nelre have been

added to this common block. The numbers of real and

imaginary matrix elements are saved with the

parameter file since the LMATRX routine for the new

matrix format needs them.

Version 1.3

ANXLK.FOR     replaces CALXLK.F. To handle the case of a fully anisotropic

diffusion tensor in anisotropic media (i.e. with an orienting

potential), this routine includes two new terms in the summation

over L1,L2,K1 (and K2) indices. See internal Cornell report by

D.E. Budil (Oct.,1992) or Earle et. al. (in preparation for

J. Phys. Chem., 1993) for details.

CALXLK.FOR    has been replaced by ANXLK.F.

CFVD.FOR      is a further upgraded algorithm for evaluation of a complex-

valued continued fraction (supplied by D.J. Schneider).

EPRLL.F,

EPRCGL.F,

and EPRBL.FOR all check the version number identification in the parameter

file before proceeding with the calculation.

LBLL.FOR      now permits input of Dxx, Dyy, and Dzz diffusion tensor

components, and checks for conditions under which the

K-bandwidth of the matrix might need to be incremented by

two because of diffusion tensor anisotropy.

MATRLL.FOR    calculates matrix elements for a fully asymmetric diffusion

tensor.

RDDAT.F and   incorporate the dxx,dyy,and dzz parameters added to the

WRDAT.FOR    common block /EPRDAT/. In addition, a version identification

string is now written by WRDAT and verified by RDDAT

W3J.FOR       An error in the closed-form calculation for the case where

J2=1 has been fixed. This error is inconsequential for the

great majority of calculations, but may prove a problem

in the case of high ordering with director tilt angles

between 0 and pi/2.

EPRDAT.INC    The variables dxx, dyy, and dzz have been substituted

for dxy and dzz in this common block.

Versions 1.1 and 1.2

BESSI0.F AND

BESSI1.FOR    A usually harmless bug in the Taylor-series expansion for

intermediate-sized arguments has been corrected in these

two routines.

CSDPC.FOR     has been eliminated. It originally calculated the

preconditioning matrix from the diagonal real part

of the SLE matrix. This function has been re-coded

in the EPRBL.F and CSPCCG.F routines.

CSPCCG.FOR    has been modified to allow preconditioning to be

bypassed in the CG algorithm by including a flag in

the calling sequence.

D200.FOR      has been replaced by program ORDER.

DAWSON.FOR    is no longer used (it was called by program D200).

EPRBAS.FOR    has been replaced by a combination of the LBASIX and

WRBAS routines.

EPRBL.FOR     now simply uses the un-preconditioned CG algorithm

in the presence of an orienting potential.

EPRDAT.INC    The variables neltot,nelv,nelim,nelre have been

added to this common block. The numbers of real and

imaginary matrix elements are saved with the

parameter file since the LMATRX routine for the new

matrix format needs them.

EPRMAT.INC    incorporates two new index arrays, jzmat, and

kzmat, which have been added to accommodate the new

matrix storage convention.

INDEXL.INC    is a new common block containing the indices of the

basis set functions for use by programs LBLL, TNLL,

LMATRX, and LVECTR.

LBASIX.FOR    is a new subroutine. Its function is to construct a

list of basis set indices in common block

INDEXL.INC for use by the LBLL, TNLL, LMATRX, and

LVECTR programs.

LMATRX.FOR    replaces the old MATLST program. It has been

modified to read the updated matrix storage format

and list the basis indices for all matrix elements.

LBLL.FOR      has been updated to streamline the user dialog and

contains additional diagnostic messages mainly to

clarify the allowable types of diffusion models.

LVECTR.FOR    replaces the old STVT program. It calculates and

prints out the elements of the starting vector.

MATRLL.FOR    now adheres to the new matrix storage convention,

and its code has been rearranged to improve the

program's efficiency for non-optimizing compilers.

RDDAT.FOR     has been updated to write the new parameters added

to common /eprdat/.

SCMVM.FOR     the matrix-vector multiply routine used in both the

Lanczos and conjugate gradients tridiagonalization

methods has been modified to utilize the new matrix

storage convention.

STDDIM.INC    now includes a parameter mxdim1 which should one

more than the maximum possible matrix dimension

(mxdim). This is used to dimension the jzmat and

kzmat arrays in EPRMAT.INC.

STVECT.FOR    returns the number of vector elements in common

/eprdat/ instead of by argument.

TDLL.FOR      has been modified to inquire whether the user

wishes a first derivative spectrum, and also to

request the number of points in the calculated

spectrum.

TNLL.FOR      has been updated to provide more informative MTS

information to the output screen as well as the

.MTS file.

WRBAS.FOR     replaces EPRBAS. writes a list of the basis set

indices to a specified logical unit.

WRDAT.FOR     has been updated to write the new parameters added

to common /eprdat/.

Specific changes in these files are documented in the program

comments where possible; and the modified files are denoted with

the version number of the most recent update in the first line of

the source code. Files other than those listed above may still say

"Version 1.0" and have not been altered from the published version.

David E. Budil

Department of Chemistry

Cornell University

Questions and notification of bugs may be addressed to:

crepeau@msc.cornell.edu      (Dr. R.H. Crepeau)

dbudil@neu.edu               (Prof. D.E. Budil)