Description and Help for tr2d.xls

Open the file

Open ‘tr2d.xls’ and select ‘Enable Macros’ if asked.  First look at the file layout.  There are 3 sheets:

- nodes	nodal geometry (ft), loads (kips) and fixity. A fixed dof is declared by placing a ‘1’ in the appropriate column (Rx or Ry).  Any value less than ‘1’ (e.g. blank or zero) indicates a free dof.

- mems	member connectivity and cross section area (in2).

- system	initially blank, this sheet is used in forming Ke and solving for displacements.

Note that the ‘nodes’ sheet does not show displacements and that the ‘mems’ sheet does not show lengths, displacements, forces, or stress.  All of this could be added.  The list of node- and member-numbers must both be a set of consecutive numbers beginning at 1.

Check for libraries

You might need to include the ‘solver.xla’ (or possibly ‘solver.xls’) library (my system does not include it by default).  Check this by going to Tools -> Macro -> Macros and clicking on Edit.  This will open the VB editor (where you can see the code).  In the VB editor, select Tools -> References and look for ‘solver.xla’ or ‘solver.xls’ and make sure it’s checked.  Select OK and then go back to Excel.  If you don’t have these, you’ll need the MS Office installation disk.

Run the macro

It’s recommended to view the ‘mems’ sheet before running because then you can see what’s happening but this is not necessary; you can run it while viewing any sheet.  Select menu item Tools -> Macro -> Macros to open the macro dialog.  Select the macro named ‘makeAndOpt’ and click Run.  This will form the system matrix in a formula-based manner and then run the Excel solver to optimize the system.  For your first run, you may want to select ‘Yes’ when asked if you want to scale the initial areas but you can play with this more later.  Once you’ve run ‘makeAndOpt’, subsequent optimization runs can be done “as usual” with the solver dialog or by running the ‘runOptOnly’ macro as long as there are no changes to the connectivity or fixity.

Interestingly, for this example the following sequence of steps causes Excel to converge on a local min.

- Initialize all bar areas to 1.0

- Run without scaling.  Excel will report that it did not find a solution.  Select ‘Keep Solver Solution’ anyway.

- Run again, but this time scale the initial areas.  This time Excel will find a solution but the objective of 570 is far from the value of 334, which it will find if you run the first time with scaling.  Note too that the value of 334 can be improved to 327 with subsequent runs.

# The code

It’s all in VB, and you can see it by selecting menu path Tools -> Macro -> Macros and then clicking on ‘Edit’.  The trick here is that everything is set up in terms of cell references rather than values.  There is no storage of the typical arrays (i.e., NM, NP, R, …) because they are already there on the sheets.  The major drawback is the use of the MINVERSE and MMULT functions.  This will be slow for larger systems.  Note that Excel provides a way to call a macro at each trial solution within the optimization process, which means that there are many possibilities to expand this approach.