Lab 10 - Internal Combustion Engine

(Recommended material: Chapter 10 – Premixed IC Engines)

- Video demonstrations of this lab can be found on YouTube at http://www.youtube.com/user/FndmtlsofCombustion

In this lab, a spark-ignited internal combustion engine will be examined. This engine, as with the one in your automobile, injects a premixed fuel and air mixture into a cylinder at approximately atmospheric pressure (unless there is a turbocharger). The mixture is then compressed by the piston to top dead center (TDC) where the mixture is ignited by a spark plug. After ignition and combustion of the mixture, the resulting pressure delivers mechanical power while the piston goes back down to bottom dead center (BDC). At this point the exhaust/intake cycle begins and then the cycle is repeated.

Intake		             Compression

Ignition and expansion	      	      Exhaust

In the laboratory, the spark timing will be changed between three cases: regular, advanced, and retarded. In the regular case, the spark is timed to coincide more or less with the top of the cylinder’s stroke. However, when the spark is activated too soon or too late, the volume of the combustion chamber is changing. This change in the volume of the chamber will have effects on the pressure trace of the cycle, on the pollutants created, and ultimately the performance of the engine. Emissions will be measured using a Horiba analyzer for four cases: baseline timing sampled before the catalytic converter, baseline timing sampled after the catalytic converter, advanced timing sampled before the catalytic converter, and retarded timing sampled before the catalytic converter.

Exercises

- Calculate the emissions data (in mole fraction) from the engine for all the cases tested. Compare and comment.