Pool Fires and Fire Storms

(Recommended material: Chapter 7 – Non-premixed combustion)

- Video demonstrations of this lab can be found on YouTube at http://www.youtube.com/user/FndmtlsofCombustion

A pool fire is usually characterized by a burning “puddle” of liquid fuel. Liquid fuel pools represent a potential hazard in many applications ranging from accidents at industrial plants to arson fires (i.e. the use of accelerants). A pool fire’s rate of burning is usually expressed by the regression rate R, equivalent to the volumetric loss of liquid per unit surface and unit time [units of length per unit time]. Three regimes can be distinguished as a function of the diameter D of the pool. If D < 3 cm, the flame is laminar. For D > 100 cm the flames are fully turbulent and the burning rate R is independent of D. The range 3 cm< D < 100 cm is the transition regime, where the minimum regression rate is found. The shape of the curve can be explained by the role of the three heat transfer mechanism:  conduction to the rim, convention, and radiation. In this lab, the time for a fixed amount of fuel to burn will be measured for a range of pool diameters D.

R

D (cm)

3

100

Laminar

Transition

Turbulent

The second part of the lab will show a ‘fire whirl’ or ‘fire storm’. Fire whirls are a rare but potentially catastrophic form of fire. In order for one to exist, there must be a source of circular flow that produces large swirl velocities as air is entrained into the fire plume. The vertical acceleration induced by the buoyancy stretches out the flame, substantially increase its height and burning rate. Fire whirls are known to increase substantially the danger of natural and accidental fires. The fire whirl phenomenon will be explored by placing and moving various cylindrical blockages (such as a cut PVC or metal pipe) around a pool fire and discussing the resulting behavior.

Exercises

- Plot the time it takes the fuel to burn as a function of dish (pool) diameter D. Use theory to explain the trend.