Lab 11 - Spray Combustion

(Recommended material: Chapter 11 – Diesel engines)

- Video demonstrations of this lab can be found on YouTube at http://www.youtube.com/user/FndmtlsofCombustion

In many combustors that burn liquid fuels, the fuel is sprayed into the combustor and burns with the air as in a diffusion flame. A fuel nozzle is used to break the fuel stream up into a fine-droplet spray. In some cases the fuel is simply injected at a high velocity into the surrounding air; in other cases a high velocity air stream is used to aid in breaking up the fuel into fine droplets (air blast atomizer). In order for the fuel to burn in a short time, the fuel must be broken up into small droplets. Spray combustion usually involves a complex flow in which droplets are formed and evaporate. Oxygen is depleted in the central part of the spray and burning occurs on the boundary, much the same way as occurs with jet diffusion flames. Some liquid droplets will find their way to an oxygen environment and burn as single droplets with a surrounding flame.

A simple conical spray ‘atomizer’ nozzle will be examined. This type of fuel nozzle is common to home and small industrial oil furnaces. Alcohol will be used as the fuel. The fuel spray and its combustion in the ambient laboratory air will be studied and the general characteristics of the spray flame observed.

Graphic courtesy of Dr. John Dec