Lab 6 - Premixed Flame Propagation and Flammability Limits

(Recommended material: Chapter 6 – Premixed Combustion)

- Video demonstrations of this lab can be found on YouTube at http://www.youtube.com/user/FndmtlsofCombustion

A flame will propagate through a fuel and oxidizer mixture only for a certain range of equivalence ratios. The limiting values of equivalence ratio that will allow the propagation of the flame are at the lean and rich end of the flammable range, and are called the flammability limits. Within the flammability limits, the flame will propagate at a certain speed. In this experiment a mixture of methane and air at a predetermined equivalence ratio and mass flow rate is injected at one end (the closed end) of a Pyrex glass tube 89 mm in diameter and 1216 mm length, and allowed to flow freely through the tube, exiting at the other end (open end). At the open end, the mixture is ignited with a hot ceramic surface, and if the mixture is within the flammability limits a premixed flame propagates upstream through the tube toward the closed end. Temperatures are measured by five Type R thermocouples spaced 203 mm apart. Flame speed is measured by five photo sensors also placed every 203 mm along the tube. The objective of this experiment is to investigate the flammability limits of a mixture of methane and air at atmospheric pressure, and to estimate the laminar and turbulent flame speeds for a range of equivalence ratios.

General procedure:

- Turn on igniter and allow it to heat up.

- Begin flowing premixed fuel-air mixture through tube starting with lean mixtures and progressing through rich mixtures. It is recommended to fix the fuel flow rate at 1,170 cc/min and vary the air flow rate from about 6,800-20,000 cc/min. Log temperature and photo sensor voltage for a range of equivalence ratios.

- Note that it is possible to select fuel and air flow rates such that the flame speed balances the flow of the mixture and the flame becomes stationary.

- Explore the flame behavior near the flammability limits.

Exercises

- Determine the equivalence ratios for the experiments run. Check that they are inside the flammability limits.

- Report the measured flame speeds for the different mixture ratios tested. Compare with the values in the literature and those calculated with Cantera in lab 5.

- Comment on the possible reasons for the differences between our experimental flame speed measurements and the literature/Cantera values.