Lab 1 - Introduction to Cantera: Thermodynamics and Chemical Kinetics

(Recommended material: Chapter 2 – Thermodynamics)

- Video demonstrations of this lab can be found on YouTube at http://www.youtube.com/user/FndmtlsofCombustion

Introduction

The purpose of this lab is to learn to use the computer software Cantera (http://code.google.com/p/cantera/), developed by Prof. David Goodwin of the California Institute of Technology. You will use Cantera throughout the course for homework assignments. In lab #4, we will use Cantera to calculate premixed laminar flame speeds and examine flame structure. Cantera is an open-source combustion code available to use and modify for free.  It can be used to examine multiphase chemical equilibrium, one-dimensional flames, reaction path diagrams and much more.

Cantera Installation

Though Cantera can be used with Linux and Mac OSX and/or run through C++ and Fortran, we will briefly describe how to install Cantera on a PC running Matlab and the free scripting language Python. Take care to use compatible versions of Cantera, Python, Numpy, and, if needed, Numarray. When installing Cantera and Python, make the file paths as short as possible. In other words, don’t bury them inside other folders that are inside another, etc.

- Download Cantera from http://code.google.com/p/cantera/ Older versions of Cantera can be found at http://sourceforge.net/projects/cantera

- Download Python from http://www.python.org

  - Download Numpy and Numarray from http://prdownloads.sourceforge.net/numpy/

- Install Python

- Install Numpy and Numarray if needed

- Set the environmental variables in the Control Panel. Make sure the PATH variable includes “c:\pythonXX\”

- Install Cantera

- Install the Cantera-Python interface (.exe inside Cantera folder)

- Start Matlab and add the folder c:\program files\Cantera\MATLAB\toolbox\cantera to its path

- Test the Cantera installation by executing one of the tutorials

- Test the Python/numeric/numarray installation. Start a command prompt and navigate to C:\Program Files\Cantera\demos\Python\flames. Run the “adiabatic_flame.py” script by typing “adiabatic_flame.py” at the command prompt.

Additional help and information on Cantera can be found at:

- http://sourceforge.net/projects/cantera/

- http://tech.groups.yahoo.com/group/cantera/

- http://code.google.com/p/cantera/

Getting Started

A good place to start learning how to use Cantera is the MATLAB tutorials that are distributed with Cantera.  These are .m files that should now be located in c:\Cantera\tutorials\MATLAB. From MATLAB, open the file in this folder called “tut1.m” (i.e. tutorial 1). This can be run by hitting F5. Better yet, read through the .m file and paste the uncommented MATLAB commands into the command window and execute them one at a time. The tutorials are short, and it is recommended you complete at least tutorials 1, 3, 4, 5, and 7. These should give you an idea of how Cantera works and the types of calculations that can be performed with Cantera.

Examples of Equilibrium Calculations with Cantera and MATLAB

a) Use Cantera to calculate the adiabatic equilibrium flame temperature and mixture composition for a stoichiometric methane-air mixture initially at 300K and 101.3 kPa (use the GRI 3.0 mechanism).

Enter the following into the MATLAB command window:

g=IdealGasMix('gri30.cti');

set(g,'T',300,'P',1.013e5,'X','CH4:1.0001,N2:7.52,O2:2');

equilibrate(g,'HP')

The first line creates an object ‘g’ which is an ideal gas mixture. The part inside the single quotes specifies that the species, kinetics, and thermodynamics are calculated according to the GRI 3.0 mechanism for methane. The initial conditions (temperature, pressure, composition) are set by the second line. Note that 1.0001 moles are used instead of 1 because the Cantera equilibrium solver sometimes cannot converge for exactly stoichiometric conditions. The last line finds the equilibrium composition at constant enthalpy (H) and pressure (P). The thermodynamic properties of the equilibrated mixture and its composition should be printed to the screen.

b) Use the Cantera equilibrium solver to calculate the heat of reaction for the combustion of a stoichiometric mixture of methane and air, initially at 300 K and 101.3 kPa (use the GRI 3.0 mechanism).

Enter the following into the MATLAB command window:

g=IdealGasMix('gri30.cti');

set(g,'T',300,'P',1.013e5,'X','CH4:1.0001,N2:7.52,O2:2');

Qr = enthalpy_mass(g);

equilibrate(g,'TP')

Qp = enthalpy_mass(g);

The initial two lines are similar to case a). The third line stores the enthalpy of the mixture in the variable Qr, and the fourth line equilibrates the mixture under constant pressure and temperature (note the 'TP' instead of 'HP'). The fifth line stores the enthalpy of the mixture in the variable Qp. The heat of reaction (in J/kg of mixture) is then Qp-Qr. This can be converted to an enthalpy of reaction in J/kg of fuel, which is equal in magnitude to the heat of combustion.

Explosion in a “Zero-Dimensional” Homogeneous Mixture

Here we will simulate a kinetically-controlled explosion of a methane/air mixture in a constant volume adiabatic homogeneous reactor.  The reactor is essentially a perfectly-insulated volume of gas, and this type of problem is known as “zero-dimensional” kinetics because the system has no explicit spatial dimensions but it evolves in time.  We will use the GRI 3.0 methane mechanism.

1.) Open a new Matlab script and type the following lines of code:

% Zero-dimensional kinetics.

% Constant volume and internal energy

% Create gas object:

gas = IdealGasMix('gri30.xml');

% Get molecular weight and number of species:

mw = molecularWeights(gas);

nsp = nSpecies(gas);

% Set gas temperature, pressure, and initial mole fractions:

set(gas,'T',350.0,'P',oneatm,'X','CH4:1,O2:2,N2:7.52');

% Set an array used by the ODE solver

y0 = [temperature(gas)

massFractions(gas)];

% The following line sets the time interval (in seconds) that will be simulated.

% You will have to change the second number (the end time).

tel = [0 2e25];

options = odeset('RelTol',1.e-5,'AbsTol',1.e-12,'Stats','on');

t0 = cputime;

out = ode15s(@conuv,tel,y0,options,gas,mw);

disp(['CPU time = ' num2str(cputime - t0)]);

%Make plots:

figure(1);

plot(out.x,out.y(1,:));

xlabel('time');

ylabel('Temperature');

title(['Final T = ' num2str(out.y(1,end)) ' K']);

figure(2);

ioh = speciesIndex(gas,'OH');

plot(out.x,out.y(1+ioh,:));

xlabel('time');

ylabel('Mass Fraction');

title('OH Mass Fraction');

figure(3);

ino = speciesIndex(gas,'NO');

plot(out.x,out.y(1+ino,:));

xlabel('time');

ylabel('Mass Fraction');

title('NO Mass Fraction');

2.) Save and execute the above commands. After crunching numbers for a few seconds, Cantera should generate plots of temperature, OH concentration, and NO concentration as a function of time.

3.) The “ignition delay time” can be considered the time at which the reaction “takes off”.  For our purposes, this can be considered the time at which the temperature rise corresponds to approximately 50% of the final value.  Try experimenting with different initial temperatures, pressures, etc. to see how this affects the ignition delay time.