Lab 2 - Combustion Bomb Calorimeter

(Recommended material: Chapter 2 – Thermodynamics)

- Video demonstrations of this lab can be found on YouTube at http://www.youtube.com/user/FndmtlsofCombustion

The combustion bomb calorimeter is used to determine the heat of combustion of fuels. The equipment typically consists of a 560 ml volume, stainless steel, pressurized vessel (reaction chamber) containing a steel dish for liquid fuel and fitted with connections for data acquisition, ignition wires and ports for gas charging and venting. The vessel is immersed in 2 liters of water bath consisting of an insulated steel bucket with lid, a stirring rod and a thermocouple.

A small amount of fuel (3 ml methanol (CH3OH)) is used in this experiment, and the vessel is filled with oxygen to a gauge pressure of 8.27 atm (120 psig). A nichrome electrical wire connected to a battery is used to ignite the fuel. The heat released by the combustion of the fuel causes the temperature of the vessel and water to increase to a new equilibrium value. Once the vessel and water temperatures are stable, the heat of combustion of the fuel can be calculated from the temperature increase of the system, the masses of the water and the vessel, and the heat capacities of these elements.

Given:

Steel vessel mass (in video = 4.704 kg)

Steel bucket with lid mass (in video = 2.447 kg)

For water	cp = 4.18 kJ/kg K

For steel	cp = 0.48 kJ/kg K

For methanol	hf° = -200.66 MJ/kmol

ρ = 790 kg/m3

Measure:

Temperature rise of the water bath.

Exercises

Find:

- The heat of combustion of CH3OH using the experimental measurements.

- The heat of combustion of CH3OH using thermodynamic theory.

- The heat of combustion of CH3OH using Cantera.

- Compare and comment.