Appendix A: Multi-criteria decision making analysis techniques

A.1 Introduction

In quality and reliability fields, there are many multi-criteria decision making (MCDM) problems such as product design evaluation and supplier selection. In this appendix we present a brief overview of typical MCDM methods. Section A.2 presents the basic concepts of MCDM problems. Four typical MCDM methods are presented in Sections A.3 through A.6, respectively.

A.2 Basic concepts of multi-criteria decision making problems

MCDM problems typically deal with multiple conflicting criteria, attributes or goals. Suppose that one wants to purchase a car. The main criteria to be considered can be cost, comfort, safety, fuel economy and so on. Some of the criteria (e.g., safety) is typically in conflict with the other criteria (e.g., cost). As such, MCDM deals with structuring and solving decision problems involving multiple criteria with different importances or preferences.

MCDM problems roughly fall into the following two categories: multiple-criteria evaluation and multiple-criteria design. A multiple-criteria evaluation problem begins with several known alternatives. Each alternative is represented by its performances against multiple criteria. The problem is to choose the best alternative or to find a set of good alternatives. The multiple-criteria design problems aim to find the preferred values of one or more decision variables by solving a series of mathematical programming models. In this appendix, we focus on the multiple-criteria evaluation.

As an example, we consider the selection problem of a manufacturing facility. Suppose that there are several different configurations available for selection. These configurations are called the alternatives. The selection decision needs to consider a set of issues such as cost, performance characteristics, maintenance, and so on. These are called the decision criteria. The performance of an alternative against a given criterion can be evaluated using a specific measure (either subjective or objective). An alterative may outperform over the others in a certain criterion but may be poorer than the others in another criterion. This necessitates considering the relative importance of a criterion. When all the performances under all the criteria for each configuration are known, the problem is to determine the best alternative. This problem is usually called the multi-criteria selection problem.

There are a number of MCDM methods or models to solve MCDM problems, including the weighted sum model (WSM), weighted product model (WPM), analytic hierarchy process (AHP), technique for order preference by similarity to ideal solution (TOPSIS), data envelopment analysis (DEA), outranking approach (ELECTRE), multi-criteria optimization and compromise solution (VIKOR), and so on. In this appendix, we focus on the first four methods that are relatively simple and have been widely used.

A.3 Weighted sum model

Suppose that {, } is a set of decision alternatives and  is a set of criteria according to which the performances of an alternative are evaluated. The problem is to determine the optimal alternative  with the best overall performance with respect to all the criteria.

The performances of alternatives are expressed in matrix form. A decision matrix  is an  matrix in which element  indicates the performance of alternative  when it is evaluated against criterion . The relative importance of criterion  is represented by weight , which meets

, . 							(1)

Assume that the performance against any criterion is larger-the-better (a smaller-the-better performance can be easily transformed into a larger-the-better performance through a simple transformation). The performance of alternative  is evaluated using the overall performance score , given by

. 							(2)

The best alternative is the one that has the largest overall performance score.

Applying the WSM to a specific MCDM problem needs to address the following three issues:

- Specification of ,

- Transformation of  considering the features of criteria, and

- Normalization of .

We first look at the first issue. In some situations,  can be objectively measured, e.g., fuel consumption per 100 kilometers of a car. In this case, we can directly take the measured value as . In the other situations,  cannot be objectively measured (e.g., one’s skill in a certain aspect) so that the value of  has to be specified based on the subjective judgment from one or more experts. In this case, an appropriate scale for measuring the performance must be defined. Typical scales are 5-point scale from 1 to 5, 7-point scale from 1 to 7 or 9-point scale from 1 to 9.

The second issue is the desirability of magnitude of  under Criterion . Generally, there are three different cases for the desirability:

- A large value of  is desirable. This case is termed as the “larger-the-better” or the maximization case.

- A small value of  is desirable. This case is termed as the “smaller-the-better” or the minimization case.

- There is a desired target value  for . This case is termed as the “nominal-the-best” or “on-target-better”.

To make Eq. (2) meaningful, all the values of  under various criteria must be transformed into “smaller-the-better” or “larger-the-better” values. Usually, we transform the value of  under a smaller-the-better or on-target-better criterion to the maximization case so that the performance values under all the criteria are “larger-the-better”. Two simple transforms for the smaller-the-better case are as follows:

(3)

where  is an appropriately specified value with . For the on-target-better case,  transforms  to the smaller-the-better case. Using  to replace  in Eq. (3), the on-target-better case is transformed to the maximization case.

The third issue is the normalization of . There are two purposes to normalize . The first purpose is to make the magnitude of  under different criteria be in the same interval so that the criteria weights have meaningful; and the second purpose is to make  dimensionless so as to avoid the case where the performances with different units are added. Consider the maximization case. Let

, .			(4)

A special case of Eq. (4) is  and . We will use this special case for all the examples in this appendix. Eq. (5) normalizes  to :

.									(5)

Example A.1: An MCDM problem involves three alternatives and four criteria, which all are larger-the-better. The criteria weights  are shown in the second row of Table A.1; and the values of  are shown in the third to fifth rows of Table A.1. The problem is to select the best alternative.

Using Eq. (5), we obtained the normalized values of , which are shown in the seventh to ninth rows of Table A.1. The overall performance scores of alternatives evaluated from Eq. (2) are shown in the last row of the table. As seen, Alternative 2 has the largest overall performance score and hence is the best alternative.

Table A.1 Computational process for Example A.1

|  |  |  |  |  | Matrix |
| --- | --- | --- | --- | --- | --- |
|  | 0.05 | 0.25 | 0.38 | 0.32 |  |
|  | 24 | 23 | 15 | 40 |  |
|  | 13 | 41 | 18 | 36 |  |
|  | 45 | 14 | 39 | 13 |  |
|  | 45 | 41 | 39 | 40 |  |
|  | 0.5333 | 0.5610 | 0.3846 | 1 |  |
|  | 0.2889 | 1 | 0.4615 | 0.9000 |  |
|  | 1 | 0.3415 | 1 | 0.3250 |  |
|  | 0.6331 | 0.7278 | 0.6194 |  |  |

A.4 Weighted product model

The WPM evaluates the overall performance of alternatives  by

. 								(6)

The alternatives  and  can be compared by the following ratio:

.						(7)

If , then alternative  is more desirable than alternative  for the maximization case. The best alternative (denoted as ) is the one that has the largest overall performance score  or meets . Since  is dimensionless, the WPM allows using the relative values instead of the actual values of .

Example A.2: Use the WPM to solve the problem in Example A.1.

We fix  and examine the cases of  and 3. The upper part of Table A.2 shows the values of ; and the bottom part shows the values of . The last column of the bottom part shows the values of  and . Since 1.2696 and , the best alternative is Alternative 2; since , the worst alternative is Alternative 3. These are consistent with the results obtained from the WSM.

Table A.2 Computational process for Example A.2

|  |  |  |  |  | Matrix |
| --- | --- | --- | --- | --- | --- |
|  | 1.8462 | 0.561 | 0.8333 | 1.1111 |  |
|  | 0.5333 | 1.6429 | 0.3846 | 3.0769 |  |
|  | 0.05 | 0.25 | 0.38 | 0.32 |  |
|  | 0.0307 | -0.1445 | -0.0693 | 0.0337 | 0.8612 |
|  | -0.0314 | 0.1241 | -0.3631 | 0.3597 | 1.0933 |

A.5 Analytic Hierarchy Process

For an MCDM problem the criteria weights represent the preferences of the decision maker and have to be determined based on subjective judgments. Similarly, the performance scores of alternatives against some or all of criteria sometimes need to be specified by experts. In these cases, an effective approach is needed to specify the weights and scores, and the AHP can be used for this purpose.

The AHP is a technique for structuring and analyzing complex decision problems. It involves the following multi-step procedure:

- Step 1: Structuring the problem into a hierarchy

- Step 2: Comparative judgments

- Step 3: Deriving the priority vector, and

- Step 4: Calculating the global score of each alterative.

Specific details are presented as follows.

A.5.1 Structuring the problem into a hierarchy

The AHP models a decision problem as a hierarchy. An AHP hierarchy consists of an overall goal, a group of alternatives for reaching the goal, and a group of criteria that relate the alternatives to the goal. Depending on the complexity of the problem, the criteria can be further broken down into sub-criteria, and a sub-criterion can be further broken down. The goal is placed at the top, the criteria and sub-criteria are sequentially placed in the intermediate levels, and the alternatives are placed at the bottom. The goal, criteria (or sub-criteria) and alternatives are called the nodes. The relative importance or preference of a node is called priority. The priority of the goal is always 1; the priorities of the criteria are called the criteria weights and the priorities of the alternatives are called the performance scores of the alternatives against a certain criterion. For example, the problem discussed in Example A.1 can be represented by a three-hierarchy AHP structure shown in Fig. A.1.

Fig. A.1 AHP hierarchy of Example A.1

Assume that Criteria 3 in Example A.1 can be further broken down into three sub-criteria . In this case, the problem will have a four-hierarchy structure as shown in Fig. A.2, where only the Criteria 3 and its sub-criteria are shown. The figures in brackets indicate the relative weights (local weights or local priorities) of the sub-criteria with respect to the criterion, and the figures outside the brackets are the global weights (or global priorities). Let  () denote the local weights, which meet  and . As such, the global weights are given by . Clearly, we have .

Fig. A.2 Decomposition of Criterion 3

A.5.2 Comparative judgments

A.5.2.1 Comparison matrix

The AHP uses pairwise comparisons and a 9-point scale to quantify the subjective judgments of experts about the criteria (or sub-criteria) weights or performance scores of alternatives against criteria (or sub-criteria). For the case of specifying the criteria weights, the criteria are pairwise compared against the goal for importance, and the results are expressed in a  comparison matrix (also termed as judgment matrix). For the case of specifying the performance scores, the alternatives are pairwise compared against each of the criteria for preference, and the results are expressed in a  comparison matrix.

Generally, the element of comparison matrix  represents the relative importance between two compared objects in terms of ratio. Let  denote the “true value” of the weight or priority of the -th object. Theoretically,  meets the relation

.						 			(8)

From Eq. (8), we have

(9)

Due to Eq. (9), the number of pairwise comparisons required is  for the criteria comparison or  for the alternative comparison.

The 9-point scale for quantifying pairwise comparisons is shown in Table A.3. As such, possible values of  are the integers from 1 to 9 and their reciprocals. Generally, one chooses a value from 1, 3, 5, 7 and 9. If one hesitates between two adjacent values in these five values, 2, 4, 6 or 8 can be used.

Table A.3 Semantics of the 9-point scale

| Grade | Semantics |
| --- | --- |
| 1 | Equal (equally important) |
| 3 | Moderate (moderately/weakly/slightly more important) |
| 5 | Strong (strongly more important) |
| 7 | Very strong (very strongly/demonstrably more important) |
| 9 | Absolute (extremely/absolutely more important) |
| 2, 4, 6, 8 | Compromises/between |

Example A.3: Consider the criteria weights in Example A.1. The problem is to generate a criteria weight comparison matrix expressed in the 9-point scale, and the elements of the matrix approximately meet Eq. (8).

The problem needs to round down or up  or  to the nearest integer within 1 to 9. Specifically, if , then . In this case, we take . If , then . In this case, we take . As such, we obtained the criteria weight judgment matrix shown in Table A.4.

Similarly, using the data in Table A.1 we can obtain the judgment matrix for alternatives with respect to Criterion 1. The results are shown in Table A.5.

Table A.4 Judgment matrix of criteria weights for Example A.1

|  | 1 | 1/5 | 1/8 | 1/6 |
| --- | --- | --- | --- | --- |
|  | 5 | 1 | 1/2 | 1 |
|  | 8 | 2 | 1 | 1 |
|  | 6 | 1 | 1 | 1 |

Table A.5 Judgment matrix of performances of alternatives against Criterion 1

|  | 1 | 2 | 1/2 |
| --- | --- | --- | --- |
|  | 1/2 | 1 | 1/3 |
|  | 2 | 3 | 1 |

A.5.2.2 Consistency index

The pairwise comparisons can be inconsistent. For an  judgment matrix, the AHP checks the consistency of judgments using a consistency index given by

(10)

where  is the largest eigenvalue of the judgment matrix (for more details about the eigenvalues and eigenvectors, see Appendix C). Let

(11)

denote the consistency ratio, where  is called the random consistency index, whose values are shown in Table A.6. The inconsistency is acceptable if ; otherwise, the judgments need to be revised.

Table A.6 Random consistency index

|  | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.58 | 0.90 | 1.12 | 1.24 | 1.32 | 1.41 | 1.45 | 1.49 |

Example A.3 (continued): Check the inconsistencies of the judgment matrices given by Tables A.4 and A.5.

The largest eigenvalue of these two judgment matrices are = 4.0407 and 3.0092, respectively. The consistency indices obtained from Eq. (10) are 0.0136 and 0.0046, and the consistency ratios obtained from Eq. (11) are 1.5% and 0.8%, respectively. Since they are much smaller than 0.1, the inconsistencies of the matrices are acceptable.

A.5.2.3 Comparison matrix under group decision making

If there are multiple comparison matrices from several experts for the same problem, these matrices can be aggregated into a single comparison matrix using the geometric average. This is because the geometric average can maintain the relation given by Eq. (9).

To illustrate, we consider the problem associated with Table A.5 and assume that two experts give different comparison matrices, which are shown in Table A.7. The aggregated matrix using the geometric average is shown in the right part of Table A.7. As seen, the aggregated judgment matrix meets Eq. (9) (i.e., ).

Table A.7 Aggregation of comparison matrices

|  | Expert 1 |  |  | Expert 2 |  |  | Aggregated |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 1/2 | 1 | 1 | 1/2 | 1 | 1.4142 | 0.5 |
|  | 1/2 | 1 | 1/3 | 1 | 1 | 1/4 | 0.7071 | 1 | 0.2887 |
|  | 2 | 3 | 1 | 2 | 4 | 1 | 2 | 3.4641 | 1 |

A.5.2.4 Intransitivity of the 9-point scale

The semantics and numerical scales defined in Table A.3 can lead to inconsistency. To illustrate, let us suppose that objects ,  and  are compared. If object  is slightly more important than object  (this implies that 3); and object  is slightly more important than object  (this implies that 3). According to the semantics of the scale, object  should be strongly more important than object  (this implies that ). On the other hand, if the transitivity holds, we would have , which is different from the result obtained from semantics of the scale. This paradox results from the intransitivity of the 9-point arithmetic scale.

To solve this problem, one can use the geometric scale:

(12)

where  is a parameter to be specified. There are two ways to specify the value of . The first way is to make the geometric scale and the 9-point arithmetic scale have the same maximum value, i.e., . This yields =1.3161. The second way is to make the two scales have the least sum of squared errors, given by

. 		 				(13)

Minimizing  yields . In this case, the maximum value of the geometric scale is . It is noted that  is vey close to the two values of  obtained above and it corresponds to . We recommend the geometric scale with .

To apply this scale, we first construct an initial judgment matrix , which meets

, ,  for 				(14)

where  is the grade of the 9-point scale. The initial judgment matrix are then transformed to the final judgment matrix using the geometric scale given by Eq. (12), i.e.,

(15)

Example A.4: Consider the performance scores of the three alternatives against Criterion 1 in Example A.1, which are known to be 24, 13 and 45, respectively. The problem is to transform them into a judgment matrix under the geometric scale.

It is noted that the grade difference in relative importance can be defined as 45/9 = 5. In other words, if there is a difference of 5 between two performance scores, then the corresponding two alternatives have a grade difference 1. In this way, we have the initial comparison matrix given in the left-hand side of Table A.8. The comparison matrix under the geometric scale can be easily calculated using Eq. (15) with . The results are shown in the right-hand side of Table A.8. It is noted that the derived comparison matrix has , and the corresponding consistency ratio is 0.8%, implying that the inconsistency is small.

Table A.8 Comparison matrix derived from the initial judgment matrix

|  | Initial judgment matrix |  |  | Comparison matrix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | -3 | 1 | 1.3333 | 0.4219 |
|  | -1 | 0 | -5 | 0.75 | 1 | 0.2373 |
|  | 3 | 5 | 0 | 2.3704 | 4.2140 | 1 |

A.5.3 Deriving the priority vector

For an  comparison matrix, there are several methods to obtain the priority vector (). In the original AHP, the priority vector is given by the unit principal eigenvector of the comparison matrix. Since the eigenvector method is somehow mathematically intractable, some simple methods have been developed to find the priority vector. Referring to Table A.9, it is noted that the row mean (geometric or arithmetic)  is proportional to . Therefore, the priority vector can be derived from the row means, and is given by

, . 				 			(16)

Usually, the row geometric mean is used.

Table A.9 Computation of criteria priority vector

| \ | or | or | … | or | Row mean |
| --- | --- | --- | --- | --- | --- |
| or | 1 |  | … |  |  |
| or |  | 1 | … |  |  |
| … | … | … | … | … | … |
| or |  |  | … | 1 |  |

Example A.5: Consider the matrices given in Tables A.4 and A.5. The true values of priorities are known and shown in the 3rd column of Table A.10. The priority vectors derived from the three methods are shown in the 4th to 6th columns of Table A.10. The last row shows the sum of the absolute errors relative to the true values. As seen, the error associated with the geometric mean method is smallest and the error associated with the arithmetic mean method is largest.

Table A.10 Priorities from different methods

| Table | Priority | True value | Eigenvalue vector | Geometric mean | Arithmetic mean |
| --- | --- | --- | --- | --- | --- |
| A.4 |  | 0.05 | 0.0494 | 0.0500 | 0.0497 |
|  |  | 0.25 | 0.2476 | 0.2477 | 0.2501 |
|  |  | 0.38 | 0.3944 | 0.3940 | 0.4001 |
|  |  | 0.32 | 0.3086 | 0.3083 | 0.3001 |
| A.5 |  | 0.2927 | 0.2970 | 0.2970 | 0.3088 |
|  |  | 0.1585 | 0.1634 | 0.1634 | 0.1618 |
|  |  | 0.5488 | 0.5396 | 0.5396 | 0.5294 |
| Error |  |  | 0.0472 | 0.0464 | 0.0792 |

A.5.4 Calculation of global score

Once criteria weights and performance scores are obtained, the global scores of alternatives are calculated using the WSM given by Eq. (2). The alternatives can be ranked based on their global scores, and hence the best alternative can be easily identified.

A.6 TOPSIS

According to the nature of a criterion (i.e., larger-the-better or smaller-the-better) and the criterion scores of alternatives, TOPSIS defines ideal and negative-ideal solutions of an MCDM problem. The distances of an alternative relative to the ideal and negative-ideal solutions are used to evaluate the preference of the alternative. The best alternative should have the shortest distance from the ideal solution and the farthest distance from the negative-ideal solution. Specific procedure to find the best alternative is outlined as follows.

Let  denote the performance measure (or rating) of the -th alternative with respect to the -th criterion. The normalized rating is given by

. 								(17)

It is noted that  is dimensionless. The matrix  is called the normalized decision matrix. Let  denote the weight of the -th criterion. The weighted normalized matrix is defined as . For the -th criterion, let

, . 					(18)

Let  denote the ideal solution and  denote the negative-ideal solution. If the -th criterion is larger-the-better, we have  and  ; otherwise,  and  . The ideal and negative ideal solutions may actually be nonexistent, and hence only serve as two reference points.

Let  and  denote the distances of the -th alternative to the ideal and negative-ideal solutions, respectively. They are calculated as

, . 				(19)

The relative closeness of the -th alternative with respect to the ideal solution  is defined as below:

. 								(20)

The best alternative should have the largest relative closeness.

Example A.6: The known conditions are the same as those in Example A.1 and shown in the 2nd to 5th rows of Table A.11. The problem is to evaluate the three alternatives using the TOPSIS method.

Matrix  is shown in the 6th to 8th rows of Table A.11; and Matrix  is shown in the 9th to 11th rows of Table A.11. Assume that all the criteria are larger-the-better. The ideal and negative ideal solutions are given respectively by

(0.0428, 0.209, 0.3257, 0.2312), (0.0124, 0.0714, 0.1253, 0.0751).

The closeness values and rank numbers of the alternatives are shown in Table A.12. As seen, the best alternative is . This is consistent with the results of Examples A.1 and A.2. However, different from the results obtained in Examples 1 and 2, Alternative 3 is thought to be superior to Alternative 1 in this example. This implies that different method may give different selection, and hence it is a good practice to try several methods for a specific problem.

Table A.11 Relevant matrices for Example A.5

|  |  |  |  |  | Matrix |
| --- | --- | --- | --- | --- | --- |
|  | 0.05 | 0.25 | 0.38 | 0.32 |  |
|  | 24 | 23 | 15 | 40 |  |
|  | 13 | 41 | 18 | 36 |  |
|  | 45 | 14 | 39 | 13 |  |
|  | 0.456 | 0.4689 | 0.3297 | 0.7225 |  |
|  | 0.247 | 0.8359 | 0.3956 | 0.6503 |  |
|  | 0.855 | 0.2854 | 0.8572 | 0.2348 |  |
|  | 0.0228 | 0.1172 | 0.1253 | 0.2312 |  |
|  | 0.0124 | 0.209 | 0.1503 | 0.2081 |  |
|  | 0.0428 | 0.0714 | 0.3257 | 0.0751 |  |

Table A.12 Closeness and ranking of alternatives for Example A.5

|  |  |  |  | Rank |
| --- | --- | --- | --- | --- |
|  | 0.21246 | 0.2556 | 0.5462 | 3 |
|  | 0.1683 | 0.2824 | 0.6266 | 1 |
|  | 0.1856 | 0.2595 | 0.5830 | 2 |