iM-3451

CATALOG No.:

2009.02

- iM - 3451

- Motion Sensor Data Sheet

- Ver. 1.0

- Advanced Material on Technology

| Specification Revision Page |  |  |  |
| --- | --- | --- | --- |
| Revisions | Description of Change | Date | Approval |
| 0.5 | - IIC Serial Peripheral Interface - p.14 Multiple-Byte Read Sequence - Register Descriptions - p.19 changes register table | 18 Feb 2009 |  |
| 1.0 | Release Version | 12 Mar 2009 |  |

- Overview

Motion sensor is a 6-axis module consisting of 3-axis magnetic sensor and 3-axis accelerometer. It allows highly accurate motion detection with geomagnetic direction and tilting data. iM-3451 is a world’s smallest class of motion sensor module with embedded uCom. Moreover, our exclusive embedded calibration algorithm, iRAC, eliminates the need for initial manual calibrations. It also enables users to access to reliable motion data virtually anywhere. Therefore, motion sensor is suitable for deployment in hand-held devices, where diverse movements are constantly expected, in order to acquire accurate positioning and direction data.

- Features

● 6-Axis Motion sensor: 3-axis accelerometer + 3-axis magnetic sensor + u-Processor

● Package: 4.5X4.5X1.2mm, LGA, Lead Free, Halogen Free

● Embedded micro-processor

- 16KB Flash, 10MIPS, 2KB SRAM, Watchdog

● Built-in software: for Calibration free

1) iRAC (Intelligent Real time Automatic Calibration)

- Automatic offset compensation

- Automatic temperature compensation

- Automatic sensitivity calibration

- Automatic calibration for magnetic field disturbance

2) Automatic Roll/Pitch Compensation

3) User application software (if required)

● Serial interface: IIC, SPI

● Wide Magnetic Measurable Range: +/- 10Gauss, appropriate for mobile phone

● Minimum Measurable Magnetic field: 10nT (0.1mGuass)

● Maximum sampling rate : 30Hz

● Azimuth & Roll/Pitch Resolution: 1 degree

● Low power: Adjustable by changing clock frequency & sampling rate

less than 6.5mA in active mode for 6axis, at internal 10MHz clock

less than 25 uA in all power down mode

- Applications

● Personal Navigation in mobile phone

● Calorie counter

● Constellation seeking device

● 3-D presenter

● Game controller

● Robot motion control

- Functional Block Diagram

- Absolute Maximum Ratings

| Item | Symbol | Min. | Max. | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| DC Supply Voltage (AVDD) | - | - 0.3 | + 4.0 | V |  |
| DC Supply Voltage (DVDDIO) | - | -0.3 | + 5.0 | V |  |
| Storage Temperature Range | - | - 55 | + 125 | ℃ |  |
| Mechanical Shock | - | - | + 5,000 | g |  |
| ESD | - | - | + 2,000 | V |  |

- Recommended Operating Conditions

| Item | Symbol | Min. | Max. | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| Operating AVDD Supply Voltage Range | - | + 2.3 | + 3.6 | V | Input |
| Operating DVDDIO Supply Voltage Range | - | + 1.6 | + 5 | V | Input |
| Operating VDD Supply Voltage Range | - | +1.8 | + 1.95 | V | Internal Power Supply Output |
| Vrefp |  | 0.4 |  | V | Upper reference voltage (Output) |
| Vrefn |  | 1.4 |  | V | Lower reference voltage (Output) |
| Operating Temperature Range | - | - 40 | + 85 | ℃ |  |

- DC characteristics

| Item | Symbol | Min. | Max. | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| Input high voltage 1 | VIH1 | 0.7 x DVDDIO | DVDDIO + 0.6 | V | All Digital I/O Port except RESET |
| Input high voltage 2 | VIH2 | 0.9 x DVDDIO | DVDDIO + 0.6 | V | /RESET(Schmitt input) |
| Input low voltage 1 | VIL1 | -0.5 | 0.3 x DVDDIO | V | All Digital I/O Port except RESET |
| Input low voltage 2 | VIL2 | -0.5 | 0.3 x DVDDIO | V | /RESET(Schmitt input) |

- Compass Characteristics

| Item |  | Min. | Typ. | Max. | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Azimuth accuracy |  | - 5 | - | + 5 | degree |  |
| Azimuth range |  | 0 | - | 359 | degree |  |
| Azimuth resolution |  | - | 1 | - | degree |  |
| Flux density measurement range |  | - 1,000 | - | + 1,000 | μT | +/- 10 Gauss |
| Maximum sampling rate(Azimuth) |  | - | 30 |  | Hz | (Raw Data sampling -100Hz) |
| Power Consumption | Active | - | - | 22.5 | mW | @DVDDIO=3.0V @AVDD=3.0V |
|  | Power Down |  |  | 30 | uW |  |
| Roll / Pitch compensation range |  | - 89 | - | + 89 | degree |  |

Accelerometer Characteristics

| Item | Min. | Typ. | Max. | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| Operating Temperature Range | -40 |  | 85 | ℃ |  |
| Zero g Offset voltage (0g) | -10 |  | 10 | count |  |
| Sensitivity (±2g,±4g,±8g) | 100 | 200 | 800 | Count/g | adjustable |
| Non-linearity |  | 2 | +/- 2 | % of FS |  |
| Cross Axis Sensitivity |  | 2 |  | % |  |
| Maximum Sampling rate |  | 100 |  | Hz |  |
| Roll / Pitch resolution | - | 1 | - | degree |  |
| Roll / Pitch accuracy | -5 | - | +5 | degree |  |

- IIC characteristics

| Parameter | Max. | Units | Description |
| --- | --- | --- | --- |
| Clock Frequency | 400 | kHz | SCL |
| Sink Current | 2 | mA | SDA, SCL |

- SPI characteristics

| Parameter | Typ. | Units | Description |
| --- | --- | --- | --- |
| Clock Frequency | 1 | MHz | SCK |

- Pin Descriptions

● Serial Interface

| Pin name | Pin No. | Pin type | Initial | Function |
| --- | --- | --- | --- | --- |
| INT | 17 | DO |  | PB6 INT (High when the data are updated) |
| /WAKEUP | 19 | DI |  | PB5 Wake Up signal : active “Low” |
| IIC / SPI select | 20 | DI |  | PB4 High (1) : IIC / Low (0) : SPI |
| SCK / SCL | 21 | DI |  | PB3 Clock Input in SPI mode. Clock Input in IIC mode |
| MISO / SDA | 22 | DIO |  | PB2 Data Output in SPI mode Data Input/Output in IIC mode |
| MOSI | 23 | DI |  | PB1 Data Input in SPI mode, |
| /SS | 24 | DI |  | PB0 Chip Selection in SPI mode. |

● Power

< External Power >

| Pin name | Pin No. | Pin type | Function |
| --- | --- | --- | --- |
| DVDDIO | 16 | PWR (Input) | Digital I/O power supply |
| AVDD | 15 | PWR (Input) | Analog power supply. |
| AVSS | 7,8,12 | PWR (Input) | GND for Analog |
| DVSS | 11 | PWR (Input) | GND for Digital |

< Internal Power >

| Pin name | Pin No. | Pin type | Function |
| --- | --- | --- | --- |
| VDD | 1, 14 | PWR (Output) | Digital power output pin. External capacitor must be connected for stable power. (2.2nF) |
| Vrefp | 2 | PWR (Output) | Analog reference power output pin External capacitor must be connected for stable power. (2.2uF) |
| Vrefn | 3 | PWR (Output) | Analog reference power output pin External capacitor must be connected for stable power. (2.2uF) |

● Etc

| Pin name | Pin No. | Pin type | Initial | Function |
| --- | --- | --- | --- | --- |
| /RESET | 10 | DSI | H | Reset pin, Active “Low”, Schmitt input. |
| TEST MODE | 13 | DI |  | Only for internal test. It must be connected with GND signal. |
| CLKIN | 18 | DI |  | PB7 External clock in |

● NC ( Non Connected)

| Pin name | Pin No. | Pin type | Initial | Function |
| --- | --- | --- | --- | --- |
| NC | 4,5,6,9 | - | - | Only for internal test |

Note

DI:  Digital input				DO:  Digital output

DSI: Digital Schmitt input			DIO: Digital bi-directional

PWR: Power

- Package Information (unit: mm)

● Dimension

● Marking

Identification mark

Model number

Lot number

Component code

Weekly code

- A P P E N D I X

- Coordinate system

Basically, the right-hand rule is used for determining the sign of a rotation: point your right thumb into the positive rotation axis and curl your fingers into the forward rotational direction.

- Typical Connection ( IIC default )

- IIC Serial Peripheral Interface

IIC is suited for typical micro-controller. It is designed to easily connect any µcom to IIC compliant slave. The 2 required lines on the slave side are the clock (SCL) signal and the data (SDA) signal.

- Typical Operating Circuit

● Typical Connection

- Protocol

● Transaction Rules

Each data transferred on the SDA is accompanied by a pulse on the clock signal. The data on the SDA must be stable when the clock is high. The only exception is for generating start and stop. The master initiates and terminates a data transmission.

Start condition: To start reading/writing operation, it is necessary to generate a start condition by switching the SDA input from high level to low level when the SCL input is high level.

Stop condition: To stop reading/writing operation, it is necessary to generate stop condition by switching the SDA input from low level to high level when the SCL input is high level. When the stop condition is generated, the operation is stopped, the data is processed and the system enters in standby mode.

● Single-Byte Write Sequence

The Single Byte Write Sequence is composed of 3 bytes of 9 bits, the 9th bit being the acknowledgement. It starts by a start condition and ends by a stop condition. The first byte is the Slave Address, the second is the Register Address and the third is the Data Byte. The LSB of the Slave Address is the R/W indicator.

When the master(Host) writes the data on the sensor :

1. Host issues a START condition on the IIC.

2. Transmit the slave address (0x50) to the sensor with generating CLK.

3. After receiving ACK from the sensor, transmit the command (register address) to the sensor.

4. After receiving ACK from the sensor, transmit the data(8bits) to the sensor.

5. After receiving the data, host issues a STOP condition.

● Two Byte Read & Muliple-Byte Read Sequence

There is no Single-Byte Read Sequence, so that "DEV_ID" and "REVNUM" should be read at word length if it would be read. In this case, upper byte of the word is "zero" and the valid data is in lower byte. Only Word read sequence is provided as following:

Two Byte Read Sequence

When the master(Host) reads the data on the sensor :

1. Host issues a START condition on the IIC.

2. Transmit the slave address (0x50) to the sensor with generating CLK.

3. After receiving ACK from the sensor, transmit the command (register address) to the sensor.

4. After receiving ACK from the sensor, host issues a STOP or a REPEATED START condition.

5. transmit the slave address(0x51) again after making a START condition.

6. After receiving ACK from the sensor, receive the data(16bits – MSB first) from the sensor.

7. After receiving the data, host issues a STOP condition.

Multiple-Byte Read Sequence

The Multiple Byte Read Sequence enables reading of several bytes continuously. And read address is automatically increased. The continuous reading sequence ends with stop condition.

● Slave address field

| Bit | NAME | Value | Description |
| --- | --- | --- | --- |
| Bit[7...0] | Slave address | 01010000(=0x50) 01010001(=0x51) | Write Sequence Read Sequence |

- Typical Connection ( SPI )

- SPI Serial Peripheral Interface (Original SPI)

The Serial Peripheral Interface (SPI) is a 4-wire Clock Synchronous Serial Interface. It is designed to easily connect any CPU to SPI compliant slaves using only 3 lines plus one chip select line per slave. Few slaves can be connected on the CPU SPI. The 4 required lines on the slave side for the SPI are MISO, MOSI, SCK signal and /SS signal.

● Typical Operating Circuit

- Typical Connection with one Slave

- Typical Connection with two Slaves

- Protocol

- The data signal is written on the falling edge of the SCK and it is read on the rising edge of the SCK. The data format is MSB in first and LSB in last. When /SS is High, then this device stops operating.

● Single Byte Write Sequence

One Word or more Read/Write Cycle is enabled when the SIOST is kept low for the entire transmission of the Word Mode and brought back to high at the end. Word read/write Sequence is as followings.

● Single Byte Read Sequence

● Multi Byte Read Sequence

- Register Table

| Register | Symbol | R/W | Explanation |
| --- | --- | --- | --- |
| 01(h) | DEV_ID | R | Device ID |
| 02(h) | REVNUM | R | Revision Number |
| 03(h) | MAG_STATUS | R | Big Magnetic Field Status Flag |
| ∼ | - | - | reserved |
| 14(h) | POWER | W | Power Down Mode |
| ∼ | - | - | reserved |
| 20(h) | AZIMUTH_H | R | MSB of AZIMUTH |
| 21(h) | AZIMUTH_L | R | LSB of AZIMUTH |
| 22(h) | PITCH_H | R | MSB of PITCH angle |
| 23(h) | PITCH_L | R | LSB of PITCH angle |
| 24(h) | ROLL_H | R | MSB of ROLL angle |
| 25(h) | ROLL_L | R | LSB of ROLL angle |
| 26(h) | ACC_XH | R | MSB of Accelerometer X axis |
| 27(h) | ACC_XL | R | LSB of Accelerometer X axis |
| 28(h) | ACC_YH | R | MSB of Accelerometer Y axis |
| 29(h) | ACC_YL | R | LSB of Accelerometer Y axis |
| 2A(h) | ACC_ZH | R | MSB of Accelerometer Z axis |
| 2B(h) | ACC_ZL | R | LSB of Accelerometer Z axis |
| 2C(h) | MAG_XH | R | MSB of Magnetic Sensor X axis |
| 2D(h) | MAG_XL | R | LSB of Magnetic Sensor X axis |
| 2E(h) | MAG_YH | R | MSB of Magnetic Sensor Y axis |
| 2F(h) | MAG_YL | R | LSB of Magnetic Sensor Y axis |
| 30(h) | MAG_ZH | R | MSB of Magnetic Sensor Z axis |
| 31(h) | MAG_ZL | R | LSB of Magnetic Sensor Z axis |
| ∼ | - | - | reserved |
| AA(h) | CAL_ACC | W | Accelerometer Offset Calibration Mode ( See page 22~26 ) |

- Register Descriptions

01(h)     "DEV_ID"                Device ID Register

Device ID                              10H

02(h)     "REVNUM"               Revision number Register

Revision number                     11H

03(h)     "MAG_ALARM"       External Big Magnetic Field Status Flag

-  No External Big Magnetic Field condition       00H

-  External Big Magnetic Field condition          01H

- This register is operated over than 0.4gauss external magnetic field distortion.

(In this condition, the Azimuth of iM-3451 may be directed wrong,

You need to calibration iM-3451)

14(h)     "AMS_POWER"      Power Management Register

| Registers | Descriptions |
| --- | --- |
| 00h(W) | ACTIVE : ALL POWER ON (Magnetic sensor ON, Accelerometer ON, MICOM ON) |
| 01h(W) | POWER DOWN: ALL POWER OFF (Magnetic sensor OFF, Accelerometer OFF, MICOM OFF) |
| 02(h) | POWER SAVE: Magnetic sensor OFF, Accelerometer ON, MICOM ON |

15(h) ∼19(h)  “Reserved”

20(h) ~ 21(h)     "AZIMUTH"

- Direction of iM-3451

- Data range: 0 ~ 360

- Resolution: 1 degree

22(h) ~ 23(h)     "PITCH"

- Pitch angle of iM-3451

- Data range: - 89 ~+ 89

- Resolution: 1 degree

24(h) ~ 25(h)     "ROLL"

- Roll angle of iM-3451

- Data range: - 89 ~+ 89

- Resolution: 1 degree

26(h) ~ 27(h)     "ACC_X"

- X axis Acceleration value

- Data range: - 2,048 ~ 2,047

- Resolution: 1 counts = 0.01225m/s2

28(h) ~ 29(h)     "ACC_Y"

- Y axis Acceleration value

- Data range: -2,048 ~ 2,047

- Resolution: 1 counts = 0.01225m/s2

2A(h) ~ 2B(h)     "ACC_Z"

- Z axis Acceleration value

- Data range: -2,048 ~ 2,047

- Resolution: 1 counts = 0.01225m/s2

2C(h) ~ 2D(h)     "MAG_X"

- X axis Magnetic flux density value

2E(h) ~ 2F(h)     "MAG_Y"

- Y axis Magnetic flux density value

30(h) ~ 31(h)     "MAG_Z"

- Z axis Magnetic flux density value

32(h) ∼A9(h)  “Reserved”

AA(h)          "CAL_ACC"

- Accelerometer Offset Calibration Mode

- See page 22 ~ 26

- Communication specification and software applications (IIC)

Communication mode : IIC serial interface.

Communication speed : Maximum 400 kHz(Fast mode)

Operation is not guaranteed for exceeding this speed.

- Communication specification and software applications (SPI)

Communication mode : SPI serial interface.

Communication speed : Typical 1 MHz

Operation is not guaranteed for exceeding this speed.

- Wake-up from power down mode

we serve two type of wake-up functions(using /WAKE_UP signal [ for SPI ] or using IIC communication at Address match [ for IIC ] )

● Using /WAKE_UP signal

● Using IIC communication

- iM-3451 Accelerometer Offset Calibration Mode Manual

The accelerometer’s offset can be changed by big shock, very high soldering temperature, etc. To sense the more precise azimuth, tilt and acceleration, it is essential to calibrate accelerometer’s offset. iM-3451 supports offset calibration mode as followings:

- Put iM-3451 (or product) on a table.

 Maintain stable condition before starting the CAL ACC offset mode. No vibration.

- Start the Accelerometer Offset Calibration Mode

 Receiving ‘0xAA’ and ‘0x55’ data from host CPU, iM-3451 is started the Accelerometer Offset Calibration Mode.

- And then, slowly rotate iM-3451 on the table, make one circle rotation.

 Don’t shake when IA-2351 is in the Accelerometer Offset Calibration Mode.

- Exit ACC offset mode

 Receiving again ‘0xAA’ and ‘0x55’ data, iM-3451 saves offset values and exits the Accelerometer Offset Calibration Mode.

- Even though the table has been tilted, acceleration sensor’s offset must be calibrated correctly.

- Note : Don’t shake or lean it when iM-3451 is in the ACC offset CAL mode.

- IMPORTANT NOTICE

*Specifications are subject to change without prior notification

AMOTECH CO., LTD.       ----                            ----------------------------------------AGENT-----------------------------------

17-2, Jamwon-Dong, Seocho-Gu,

Seoul, 137-902, Korea

Tel : +82-2-540-3850

Fax : +82-2-517-7183

All rights reserved © 2007 Advanced Material on Technology